3GPP TSG-RAN WG3 Meeting #113-e	R3-214084

- 16-26 Aug 2021

- Online

Agenda item:	10.2.4

Source: 	ZTE, China Telecom, China Unicom

Title: 	(TP for SON BL CR 38.413) Inter-system Load Balancing

Document for:	Agreement

Introduction

This contribution provides a TP for BL CR 38.413 [1] on the inter-system load balancing.

Text Proposal for 38.413

First Change

9.3.3.36	Inter-system SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >HO Report Information |  |  |  |  |
| >>Inter-system HO Report | M |  | 9.3.3.40 |  |
| >Failure Indication Information |  |  |  |  |
| >>Inter-system Failure Indication | M |  | 9.3.3.38 |  |
| >Energy Savings Indication |  |  |  |  |
| >>Cell State Indication | M |  | 9.3.3.aa |  |
| >Resource Status Reporting |  |  |  |  |
| >>Inter-system Resource Status Update | M |  | 9.3.3.cc |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

9.3.3.xx	Inter-system SON Information Request

This IE contains the request information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Request | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Request | M |  | 9.3.3.zz |  |
| >Inter-system Resource Status Reporting Initiation |  |  |  |  |
| >>Inter-system Resource Status Request | M |  | 9.3.3.dd |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

9.3.3.yy	Inter-system SON Information Reply

This IE contains the reply information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Response | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Reply | M |  | 9.3.3.bb |  |
| >Inter-system Resource Status Reporting Initiation |  |  |  |  |
| >>Inter-system Resource Status Response | M |  | 9.3.3.ee |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

- 9.3.3.cc	Inter-system Resource Status Update

This IE contains the results of requested measurements for Inter-system load balancing to be transferred.

Direction: NG-RAN node  E-UTRAN node, E-UTRAN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Reporting RAT |  |  |  |  | YES | ignore |
| >E-UTRAN |  |  |  |  |  |  |
| >>Composite Available Capacity Group | M |  |  | Contains the Composite Available Capacity Group IE as defined in TS 36.423[40]. |  |  |
| >>Radio Resource Status | O |  |  | Contains the Radio Resource Status IE as defined in TS 36.423[40]. |  |  |
| >NG-RAN |  |  |  |  |  |  |
| >>Composite Available Capacity Group | M |  |  | Contains the Composite Available Capacity Group IE as defined in TS 38.423[24]. |  |  |
| >>Radio Resource Status | O |  |  | Contains the Radio Resource Status IE as defined in TS 38.423[24]. |  |  |
| >>Number of Active UEs | O |  |  | Contains the Number of Active UEs IE as defined in TS 38.423[24]. |  |  |
| >> RRC Connections | O |  |  | Contains the RRC Connections IE as defined in TS 38.423[24]. |  |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

- 9.3.3.dd	Inter-system Resource Status Request

This IE contains the Inter-system resource status request information to be transferred.

Direction: E-UTRAN node  NG-RAN node, NG-RAN node  E-UTRAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Inter-system Resource Status Reporting type |  |  |  |  |  |  |
| >Event based Reporting |  |  |  |  |  |  |
| >>Event based Inter-system Resource Status Request | M |  |  |  |  |  |
| >>>Number Of Measurement Reporting Levels | M |  | ENUMERATED (2, 3, 4, 5, 10, ...) | The reporting node divides the cell load scale into the indicated number of reporting levels, evenly distributed on a linear scale below the reporting node's threshold for overload. The reporting node sends a report each time the cell load changes from one reporting level to another, and when the cell load enters and exits overload state. |  |  |
| >Periodic based Reporting |  |  |  |  |  |  |
| CHOICE Reporting RAT |  |  |  |  |  |  |
| >>E-UTRAN |  |  |  |  |  |  |
| >>> Cell Identity | M |  | OCTET STRING | Contains the E-UTRAN CGI IE, as defined in 9.1.3.9 |  |  |
| >>NG-RAN |  |  |  |  |  |  |
| >>> Cell Identity | M |  | OCTET STRING | Contains the NG-RAN CGI IE, as defined in 9.1.3.7 |  |  |
| >>Reporting Periodicity | M |  | ENUMERATED (1000ms, 2000ms, 5000ms,10000ms, …) |  |  |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

- 9.3.3.ee	Inter-system Resource Status Response

This IE contains the Inter-system load balancing response information to be transferred.

Direction: NG-RAN node  E-UTRAN node, E-UTRAN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Criticality Diagnostics | O |  | - |  | YES | ignore |

Editor's Note: The ASN.1 changes for the above changes are not yet included

End of Changes

References

- R3-213141 BLCR to 38.413_Addition of SON features enhancement  Ericsson