3GPP TSG-RAN WG3 #113e                                                                               R3-214120

Online, 16th – 26th August 2021

Agenda Item:	22.2.2

Source:	CMCC

Title:	(TP to TS 38.410) MBS session management over NG

Document for:	Approval

1	Introduction

This is a text proposal for TS 38.410 BLCR on the MBS session management over NG based on the discussion paper.

- Text proposal to TS 38.410 BLCR

--------------------------------------------------------------Start of the change---------------------------------------------------------

5.5	PDU Session Management function

The PDU Session function is responsible for establishing, modifying and releasing the involved PDU sessions NG-RAN resources for user data transport and the associated MBS sessions NG-RAN resources, once a UE context is available in the NG-RAN node.

NGAP supports transparent relaying of PDU Session/MBS session related information by the AMF as described in TS 23.502 [6].

--------------------------------------------------------------End of the change --------------------------------------------------------

--------------------------------------------------------------Start of the change---------------------------------------------------------

6.1	PDU Session Management Procedures

The following list of PDU Session management procedures are used to establish, release or modify NG-RAN resources for a PDU session and associated MBS session:

-	PDU Session Resource Setup;

-	PDU Session Resource Release;

-	PDU Session Resource Modify;

-	PDU Session Resource Notify;

-	PDU Session Resource Modify Indication.

--------------------------------------------------------------End of the change --------------------------------------------------------

--------------------------------------------------------------Start of the change---------------------------------------------------------

6	Signalling procedures of the NG interface

//skip unchanged part

## 6.xx	NR MBS Session Management Procedures

The following list of MBS Session management procedures are used to establish, release, or modify NG-RAN resources for a NR MBS session:

-	Broadcast Session Resource Setup;

-	Broadcast Session Resource Release;

-	Multicast Distribution Setup;

-	Multicast Distribution Release;

-	Multicast Session Activation;

-	Multicast Session Deactivation;

--------------------------------------------------------------End of the change --------------------------------------------------------

--------------------------------------------------------------Start of the change---------------------------------------------------------

7	NG interface protocol structure

7.2	NG User Plane

The NG user plane (NG-U) interface is defined between a NG-RAN node and a UPF. The NG-U interface provides non guaranteed delivery of PDU Session /MBS session user plane PDUs between the NG-RAN node and the UPF.

Editor’s note: Changes are expected for MBS session.

The protocol stack for NG-U is shown in Figure 7.2-1.

Figure 7.2-1: NG-U protocol structure for PDU Session

Figure 7.2-2: NG-U protocol structure for MBS Session

Editor’s note: If NG-U protocol structure is the same for both PDU Session and MBS Session, the Figure 7.2-2 shall be removed.

--------------------------------------------------------------End of the change --------------------------------------------------------