3GPP TSG-RAN WG3 #113e                                                                               R3-214121

Online, 16th – 26th August 2021

Agenda Item:	22.2.2

Source:	CMCC, Huawei

Title:	(TP to TS 38.300) MBS session management over NG

Document for:	Approval

1	Introduction

This is a text proposal for TS 38.300 BLCR on the MBS session management over NG based on the discussion paper.

- Text proposal to TS 38.300 BLCR

--------------------------------------------------------------Start of the change---------------------------------------------------------

16.x	NR MBS

16.x.1	General

Editor’s Note: General aspects to be covered here.

16.x.2	Architecture

Editor’s Note: Architecture aspects to be covered here.

The overall NG-RAN architecture specified in section 4 applies for NR MBS.

16.x.3	Session Management

Editor’s Note: Session Management aspects to be covered here.

### 16.x.3.1		General

An MBS Session comprises one or more QoS flows. Each MBS QoS flow is associated to a QoS profile.

### 16.x.3.2		MBS Session Resources for Multicast

The 5GC sets up MBS Session Resources and the linkage between MBS session and associated PDU session in the NG-RAN node using the PDU Session Resource Modify procedure.

MBS Session Resources consist of an MBS context in the NG-RAN node, control plane resources, NG-U resources and radio resources.

There are two types of delivery modes as defined in TS 23.501:

-	MBS shared delivery mode

-	MBS individual delivery mode

In MBS shared delivery mode, the MBS context is setup in the NG-RAN node when the first UE joins the MBS session in the NG-RAN node and released when the last UE leaves the MBS session in this NG-RAN node.

In MBS shared delivery mode, shared NG-U resource is used. The NG-RAN node triggers the setup of the shared NG-U resources when the MBS context is setup and triggers the release of the shared NG-U resources when the MBS context is removed using the MBS distribution procedure.

The shared NG-U resources consist of one shared NG-U tunnel per MBS session which can be of the following types:

-	one shared GTP-U tunnel per NG-RAN node if unicast transport is used.

-	one shared GTP-U tunnel across all involved NG-RAN nodes if multicast transport is used. The multicast transport is identified by one multicast IP address and one source IP address.

### 16.x.3.2		MBS Session Resources for Broadcast

--------------------------------------------------------------End of the change---------------------------------------------------------