3GPP TSG-RAN WG3 Meeting #113electronic	R3-214461

Online, 16th – 26th Aug 2021

Agenda Item:	18.4.3

Source:	CMCC (moderator), ZTE

Title:	TP for AIRAN_MobilitySolution

Document for:	Discussion and Decision

1	TP to TR 37.817

-----------------------------------Start of Changes-----------------------------------

5.3 Mobility Optimization

5.3.1	Use case description

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

5.3.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

Considering the locations of AI/ML Model Training and AI/ML Model Inference for mobility solution, following two options are considered:

- The AI/ML Model Training function is deployed in OAM, while the Model Inference function resides within the RAN node

- Both the AI/ML Model Training function and the AI/ML Model Inference function reside within the RAN node

Furthermore, for CU-DU split scenario, following option is possible:

- AI/ML Model Training is located in CU-CP or OAM, and AI/ML Model Inference function is located in CU-CP

5.3.2.1 AI/ML Model Training in OAM and AI/ML Model Inference in NG-RAN node

Step 1: The RAN is assumed to have in use a trained AI/ML model for inference

Step 2. Model Inference. Required measurements are leveraged into Model Inference to output the prediction, e.g.  UE trajectory prediction, target cell prediction, target NG-RAN node prediction, etc.

Step 3. According to the prediction, recommended actions or configuration are executed for Mobility Optimization.

          - 5.3.2.2 AI/ML Model Training and AI/ML Model Inference in NG-RAN node

Figure 5.3-1: Model Training and Model Inference both located in RAN node

Step 1. NG-RAN node1 configures the measurement information on the UE side and sends configuration message to UE including configuration information.

Step 2. UE collects the indicated measurement, e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3. UE sends measurement report message to NG-RAN node1 including the required measurement.

Step 4. Model training. Required measurements are leveraged to training ML model for mobility optimization.

Step 5. NG-RAN node1 obtains the measurement report as inference data for real-time UE mobility optimization.

Step 6. Model Inference. Required measurements are leveraged into Model Inference to output the prediction, including e.g., UE trajectory prediction, target cell prediction, target NG-RAN node prediction, etc.

Step 7. According to the prediction, recommended actions are executed for Mobility Optimization. NG-RAN node1 may send the predicted mobility optimization solution to NG-RAN node2.

          - 5.3.2.3 Input data

The following data is required as input data for mobility optimization.

Input Information from UE:

- FFS UE historical location information from MDT, e.g., Latitude, longitude, altitude, cell ID

- Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

- UE historical serving cells and their locations

- Moving velocity

- FFS predicted traffic

Input Information from the neighbouring RAN nodes:

- UE’s successful handover information in the past and received from neighboring RAN nodes

- UE’s history information from neighbor

- Position, resource status, FFS QoS parameters of historical HO-ed UE (e.g., loss rate, delay, etc.)

- Predicted load

- Resource status and utilization prediction/estimation

- SON Reports of handovers that are successful, too-early, too-late, or handover to wrong (sub-optimal) cell

- FFS Information about the performance of handed over UEs

Input Information from the local node:

- UE trajectory prediction output (will be used by the RAN node internally)

- Local load prediction

          - 5.3.2.4 Output data

- FFS UE trajectory prediction (Latitude, longitude, altitude of UE over a future period of time)

- Estimated arrival probability in CHO and relevant confidence interval

- Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication

-----------------------------------End of Changes-----------------------------------