3GPP TSG-RAN WG3 Meeting #113-e	R3-214482

16-26 Aug 2021, E-meeting

Title: 	TP to 37.817 on AI/ML based network energy saving

Source: 	Intel

Agenda item:	18.4.1

Document Type:	Other

# 1. Introduction

This TP tries to reflect the common understanding on the solution of AI/ML-based network energy saving from CB # AIRAN3_ESSolution, including general descriptions of agreed solution.

# 5. Reference

- R3-214221, SoD_CB # AIRAN3_ESSolution

# Annex – TP for TR 37.817

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 5.1	Network Energy Saving

### 5.1.1	Use case description

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 5.1.2	Solutions and standard impacts

-----------------------------------Start of Changes-----------------------------------

#### 5.1.2.1	Model Training at OAM and Model Inference at NG-RAN

In this solution, NG-RAN predicts energy saving decisions by AI/ML model trained from OAM.

Figure 5.1.2.1-1. Model Training at OAM, Model Inference at NG-RAN

Step 0: NG-RAN node 1 is assumed to have a AI/ML model trained by OAM, NG-RAN node 2 is assumed to have a AI/ML model trained by OAM optionally.

Step 1: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 2: UE sends UE measurement report to NG-RAN node 1. (FFS on if triggered)

Step 3: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output(s) (e.g. energy saving strategy, handover strategy, etc).

Step 4: NG-RAN node 1 selects the most appropriate target cell for each UE before it performs handover and goes to the predicted energy state.

Step 5: NG-RAN node 1 and NG-RAN node 2 provide feedback to OAM.

#### 5.1.2.2	Model Training and Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and generates energy saving decisions.

Editor’s Notes: FFS on data collection.

Figure 5.1.2.2-1. Model Training and Model Inference at NG-RAN

Step 1: NG-RAN node 1 trains AI/ML model for AI/ML-based energy saving based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based energy saving optionally, which can also generate predicted results/actions.

Step 2: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 3: UE sends UE measurement report to NG-RAN node 1. (FFS on if triggered)

Step 4: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g. energy saving strategy, handover strategy, etc).

Step 5: NG-RAN node 1 selects the most appropriate target cell for each UE before it performs handover and goes to the predicted energy state.

Step 6: NG-RAN node 2 provides feedback to NG-RAN node 1.

#### 5.1.2.3	Input of AI/ML-based Network Energy Saving

To predict the optimized network energy saving decisions, NG-RAN may need following information as input data for AI/ML-based network energy saving:

- Current/Predicted resource status of ES-Cell and its neighbor nodes

- Current/Predicted energy information of ES-Cell and its neighbor nodes

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc)

If existing UE measurements are needed by a gNB for AI/ML-based network energy saving, RAN3 shall reuse the existing framework (including MDT and RRM measurements). FFS on whether new UE measurements are needed.

Editor’s Note: FFS other input information required for AI/ML-based network energy saving. FFS energy information is exact energy consumption value or energy efficiency gain.

#### 5.1.2.4	Output of AI/ML-based Network Energy Saving

AI/ML-based network energy saving model can generate following information as output:

- Energy saving strategy

- Handover strategy, including recommended candidate cells for taking over the traffic

- Predicted energy information

Editor’s Note: FFS other output information expected from AI/ML-based network energy saving. FFS detailed granularity and action of energy saving strategy. FFS on accuracy of predicted energy saving decision.

#### 5.1.2.5	Feedback of AI/ML-based Network Energy Saving

To optimize the performance of AI/ML-based network energy saving model, following feedback can be considered to be collected from NG-RAN nodes:

- Load measurement

- Energy information

Editor’s Note: FFS other feedback expected from AI/ML-based network energy saving.

-----------------------------------End of Changes-----------------------------------