3GPP TSG-RAN3 Meeting #113-e	R3-213129

Electronic meeting, 16 - 26 August 2021

SA WG2 Meeting #S2-145E	S2-2105150

17 - 28 May, 2021, Electronic meeting

Title:	Reply LS on NAS-based busy indication

Response to:	S2-2103768 / R2-2104354

Release:	Release 17

Work Item:	MUSIM / LTE_NR_MUSIM-Core

Source:	SA2

To:	RAN2, CT1, RAN3

Cc:	SA3

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.502 CR2558

1. Overall Description:

SA2 thanks RAN2 for their LS on NAS-based busy indication for Idle and RRC Inactive state (S2-2103768 / R2-2104354).

The attached CR implements the RAN2 agreement to enable RAN2 to further analyse the topic and come back to SA2 with any additional feedback. Several companies in SA2 (13) have a concern about the use of NAS-based busy indication from RRC Inactive state as described hereafter. Several other companies in SA2 (12) do not share this concern.

- The UE resumes from RRC-Inactive when sending the Paging Reject in NAS level. - The RAN is unaware of the content of the NAS message and forwards the NAS message to AMF. The RAN node starts scheduling the DL data or signalling within its buffers for the UE. - Depending upon UE implementation, the UE may discard any received packet or NAS PDU, which would lead to use of Uu resources for these discarded packets or NAS PDUs. - This may continue until the UE is released. - RAN receives the N2 release request from the AMF and then releases the UE to CM-IDLE/RRC-IDLE.

SA2 would like to ask RAN2 to evaluate this CR and provide feedback on the above concerns.

Given that SA2 work on MUSIM is due for completion in June 2021, SA2 agreed to proceed with the attached CR, but including an Editor’s note. A response from RAN2 is desired so SA2 can decide how to proceed.

Regarding the specific RAN2 questions in the LS:

- Question 1: Are the impacts identified by RAN2 valid?

- Question 2: Are there any other impacts beyond those identified by RAN2?

- Question 3: If the ANS to Q1 and/or to Q2 is yes, can they be specified within Rel-17 timeframe?

SA2 would like to offer the following answers:

-	A1: As indicated above.

-	A2: See the concern description above expressed by several companies, noting that not all companies share this concern.

-	A3: The attached CR needs only the Editor's note to be removed if RAN2 does not change its Working assumption. If the RAN2 assumptions change (e.g. AS-level busy), SA2 needs to update the corresponding solution.

SA2 would also like to bring to RAN2’s attention the attached CR implies that at the end of the 5GS NAS Leaving procedure the UE is always put in RRC Idle state.

In addition, SA2 would like to check with RAN2 what range of absence time RAN2 considers to use in the procedure for “switching without leaving RRC Connected state”.

2. Actions:

To RAN2 group.

ACTION:

1) 	SA2 respectfully asks RAN2 to consider the above and provide feedback.

2) 	SA2 would also like to ask RAN2 what range of absence time RAN2 considers to use in the procedure for “switching without leaving RRC Connected state”.

To CT1 and RAN3 groups.

ACTION: 	SA2 respectfully asks CT1 and RAN3 to take the feedback above into consideration for their work on MUSIM.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#146-e			16 - 27 August 2021			Electronic Meeting

3GPP SA2#147-e			18 - 22 October 2021			Electronic Meeting