3GPP TSG-RAN3 Meeting #114-e	R3-214701

Electronic meeting, 1- 11 November 2021

3GPP TSG-RAN WG4 Meeting # 100-e	R4-2115354

Electronic Meeting, Aug. 16-27, 2021

Title:	Reply LS on inter-donor migration

Reply To:	R3-212981 (R4-2111715)

Release:	Rel-17

Work Item:	NR_IAB_enh-Core

Source:	RAN WG4

To:	RAN WG3

Cc:			RAN WG1, RAN WG2

Contact Person:

Name:	Richie Leo

E-mail Address:	richie.leo@zte.com.cn

Attachments:

1. Overall Description:

RAN4 would like to thank RAN3 for their LS on inter-donor migration [1]. RAN4 would like to provide the following observation and responses to the RAN3 questions:

On implementation alternative # 1:

- Alternative 1 can be supported without impact to RAN4 specification TS 38.133.

On alternative # 2,

- Q1: Whether the current specification enables a RRC CONNECTED UE remains connected, while observing the change of NCGI, and no change to the PCI?

- [RAN4 Response]: Current RAN4 specifications do not define whether a RRC CONNECTED UE remains connected, while observing the change of NCGI, and no change to the PCI. During NCGI acquisition time if the NCGI changes then the UE may not meet NCGI acquisition delay requirements defined in clause 9.11, TS 38.133.

- Q2: is it possible to use same PCI for cell1 and cell2, and support the HO from cell1 to cell2 without new impact to the UE (e.g. a legacy UE)?

- [RAN4 Response]: The UE is not expected to meet handover requirements if the PCIs of cell1 and cell2 are the same.

From RAN4 perspective, it is not clear how cell1 and cell 2 are configured to use the same PCI. At least two options are possible:

  - SSBs carrying PCI are identical, only SIB1 carrying NCGI changes,

  - SSB configurations are separate but PCI is the same for those.

  - In both cases the UE is not expected to meet handover requirements.

- Q3: when cell1 and cell2 use different PCI/NCGI, is it possible to use one set of shared resource, without new impact to the UE?

- [RAN4 Response]: The UE is not expected to meet handover requirements if the same resources are used in cell1 and cell2 even if they use different PCI/NCGI.

- Actions:

To RAN WG3 group.

ACTION: 	RAN4 kindly asks RAN3 to take the above feedback into account on their future work.

- Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #101-e			1st – 11th November 2021 		Online

- References:

- R4-2111715