3GPP TSG-RAN WG3 #114-e																		   R3-214753

1-11 Nov 2021

Online

Agenda Item:	8.1

Source:	ZTE

Title:	(TP for BL CR for 38.413) Multicast group paging

Document for:	Approval

- Introduction

In the last RAN2 #115 e-meeting, for Multicast Paging, one LS (R2-2109177) was approved as below.

1. Overall Description: RAN2 considered two options for paging for multicast session activation notification for RRC_IDLE/ RRC_INACTIVE UEs as - Option 1: Paging for multicast session activation notification is used in all legacy Paging Occasions (POs). - Option 2: Paging for multicast session activation notification is used in the relevant legacy POs for the UEs with non-activated multicast session(s). Further, RAN2 understands that option 2 is paging resource efficient and has made agreement for option 2, subject to RAN3 confirmation. 2. Actions: To RAN3, SA2 RAN2 respectfully asks RAN3 and SA2 to take above information into account in future work and provide feedback if needed.

- Discussion

For option 2, the CN paigng message includes each idle UEs, the message needs more signalling overhead. However, the option2 can decrease more Uu resouces. So that, RAN2 prefer solution 2.

From our view, both option 1 and option 2 can work for support of multicast paging. So, RAN3 can confirm that option 2 is feasible and RAN3 will provide interface signalling to support option 2.

Proposal 1: RAN3 shall confirm that option 2 is feasible and will provide interface signalling to support option 2.

Proposal 2: Reply LS to RAN2 to provide RAN3’s feedback.

- Conclusion

Based on above analysis, we provide the following observations and proposals.

Proposal 1: RAN3 shall confirm that option 2 is feasible and will provide interface signalling to support option 2.

Proposal 2: Reply LS to RAN2 to provide RAN3’s feedback.

- Text Proposal for TS36.413

----------Start of the First Change--------------

### 8.5.X	Multicast Group Paging

#### 8.5.X.1	General

The purpose of the Multicast Group Paging procedure is to enable the AMF to notify all involved UEs of the activation of an MBS Session.

The procedure uses non-UE associated signalling.

#### 8.5.X.2	Successful Operation

Figure 8.5.X.2-1: Multicast Group Paging

The AMF initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the NG-RAN node supporting MBS Session.

At the reception of the MULTICAST GROUP PAGING message, the NG-RAN node shall perform paging of the MBS Session identified by the MBS Session ID IE in cells which belong to the indicated Multicast Paging Area IE.

If the Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 38.304 [12] and TS 36.304 [29].

For each cell that belongs to any of the tracking areas indicated in the TAI List for Paging IE, the NG-RAN node shall generate one page on the radio interface.

If the Paging Priority IE is included in the PAGING message, the NG-RAN node may use it according to TS 23.247 [x].

#### 8.5.1.3	Abnormal Conditions

Void.

*****************Next changes*******************

#### 9.2.4.X	MULTICAST GROUP PAGING

This message is sent by the AMF and is used to notify involved UEs about the activation of a multicast session in one or several tracking areas.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.x |  | YES | ignore |
| MBS Service Area | O |  | (FFS) |  | YES | ignore |
| Paging Priority (FFS) | O |  | 9.3.1.78 |  | YES | ignore |
| UE Paging List |  | 0..1 |  |  | YES | ignore |
| >UE Paging Item |  | 1..<maxnoofUEsforPaging> |  |  | - |  |
| >>UE Paging Identity | M |  | 9.3.3.18 |  | - |  |
| >>Paging DRX | O |  | 9.3.1.90 |  |  |  |
| >>TAI List for Paging |  | 1 |  |  |  |  |
| >>>TAI List for Paging Item |  | 1..<maxnoofTAIforPaging> |  |  |  |  |
| >>>>TAI | M |  | 9.3.3.11 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforGroupPaging | Maximum no. of TAIs for group paging. Value is FFS. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEsforPaging | Maximum no. of UEs for paging. Value is 1024. |

9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

----------End of the Change--------------