3GPP TSG-RAN WG3 #114-e																		   R3-214754

1-11 Nov 2021

Online

Agenda Item:	8.1

Source:	ZTE

Title:	(TP for BL CR for 38.423) Multicast group paging

Document for:	Approval

- Introduction

In the last RAN2 #115 e-meeting, for Multicast Paging, one LS (R2-2109177) was approved as below.

1. Overall Description: RAN2 considered two options for paging for multicast session activation notification for RRC_IDLE/ RRC_INACTIVE UEs as - Option 1: Paging for multicast session activation notification is used in all legacy Paging Occasions (POs). - Option 2: Paging for multicast session activation notification is used in the relevant legacy POs for the UEs with non-activated multicast session(s). Further, RAN2 understands that option 2 is paging resource efficient and has made agreement for option 2, subject to RAN3 confirmation. 2. Actions: To RAN3, SA2 RAN2 respectfully asks RAN3 and SA2 to take above information into account in future work and provide feedback if needed.

- Discussion

For option 2, the CN paigng message includes each idle UEs, the message needs more signalling overhead. However, the option2 can decrease more Uu resouces. So that, RAN2 prefer solution 2.

From our view, both option 1 and option 2 can work for support of multicast paging. So, RAN3 can confirm that option 2 is feasible and RAN3 will provide interface signalling to support option 2.

Proposal 1: RAN3 shall confirm that option 2 is feasible and will provide interface signalling to support option 2.

Proposal 2: Reply LS to RAN2 to provide RAN3’s feedback.

- Conclusion

Based on above analysis, we provide the following observations and proposals.

Proposal 1: RAN3 shall confirm that option 2 is feasible and will provide interface signalling to support option 2.

Proposal 2: Reply LS to RAN2 to provide RAN3’s feedback.

- Text Proposal for TS38.423

<<<<<<<<<<<<<<<<<<<< Start Change >>>>>>>>>>>>>>>>>>>>

8.X1.Y1	RAN Multicast Group Paging

8.X1.Y1.1	General

The purpose of the RAN Multicast Group Paging procedure is to enable the NG-RAN node1 to request paging of UEs that have joined an MBS Session in the NG-RAN node2.

The procedure uses non UE-associated signalling.

8.X1.Y1.2	Successful operation

Figure 8.X1.Y1.2-1: RAN Multicast Group Paging: successful operation

The RAN Multicast Group Paging procedure is triggered by the NG-RAN node1 by sending the RAN MULTICAST GROUP PAGING message to the NG-RAN node2, in which the necessary information e.g. the MBS Session Identity IE associated with the MBS session should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.X2.Y2	RAN MULTICAST GROUP PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page UEs for a multicast session.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| MBS Session ID | M |  | FFS |  | YES | reject |
| UE Paging List |  | 0..1 |  |  | YES | ignore |
| >UE Paging Item |  | 1..<maxnoofUEsforPaging> |  |  | - |  |
| >>CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >>>Length-10 |  |  |  |  |  |  |
| >>>>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| >>UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| >>Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| >>RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< End Change >>>>>>>>>>>>>>>>>>>>