3GPP TSG-RAN WG3#114-e	R3-214771

# E-meeting, 1 – 11 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0642 | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of 4g-5g Direct Data Forwarding over E1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, CATT, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Direct_data_fw_NR-Core |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For inter-system HO,direct data forwarding is not supported in case the source node or target node is CP/UP separation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Data Forwarding from E-UTRAN Request List IE in PDU Session Resource To Setup List. Add Data Forwarding from E-UTRAN Response List IE in PDU Session Resource Setup List. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the Inter-system handover function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Direct data forwarding could not be supported in case the source node or target node has CP/UP separation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 9.3.3.2, 9.3.3.5, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.413 CR 1738 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT SETUP RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	For each established PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each established PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each established DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each established DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Existing Allocated NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may re-use the indicated resources already allocated for this bearer context. If the gNB-CU-UP decides to re-use the indicated resources, it shall include the NG DL UP Unchanged IE in the BEARER CONTEXT SETUP RESPONSE message.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store and use the information for the down link traffic policing for the Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [20].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE, Data Forwarding from E-UTRAN Request List IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall include the requested forwarding information respectively in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE, Data Forwarding from E-UTRAN Response List IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT SETUP RESPONSE message.

If the DL UP Parameters IE is contained in the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall configure the corresponding information.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup List IE of the BEARER CONTEXT SETUP RESPONSE message.

Not modified

#### 9.3.3.2	PDU Session Resource To Setup List

This IE contains PDU session resource related information used at Bearer Context Setup Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when at least one Non-GBR QoS Flows is being setup. | - | - |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Existing Allocated NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >Network Instance | O |  | 9.3.1.62 | This IE is ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Contains the PDCP SN Status at setup after Resume. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB. | YES | ignore |
| >>>DAPS Request Information | O |  | 9.3.1.91 |  | YES | ignore |
| >>>Ignore Mapping Rule Indication | O |  | ENUMERATED (True, …) | Included if the QoS flow mapping rule for the DRB has not been decided by gNB-CU-CP. | YES | reject |
| >>>Data Forwarding from EUTRAN Request List |  | 0..<maxnoofDataForwardingTunnelfromE-UTRAN> |  |  | YES | ignore |
| >>>>Data Forwarding Information Request | M |  | 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP for inter-system handover. | - | - |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |
| >Redundant PDU Session Information | O |  | 9.3.1.80 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Not modified

#### 9.3.3.5	PDU Session Resource Setup List

This IE contains setup PDU session resource related information used at Bearer Context Setup Response

| IE/Group Name |  | Presence |  | Range |  | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Setup Item |  |  |  | 1..<maxnoofPDUSessionResource> |  |  |  | - | - |  |
| >PDU Session ID |  | M |  |  |  | 9.3.1.21 |  | - | - |  |
| >Security Result |  | O |  |  |  | 9.3.1.52 |  | - | - |  |
| >NG DL UP Transport Layer Information |  | M |  |  |  | UP Transport Layer Information 9.3.2.1 |  | - | - |  |
| >PDU Session Data Forwarding Information Response |  | O |  |  |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |  |
| >NG DL UP Unchanged |  | O |  |  |  | ENUMERATED (True, …) |  | - | - |  |
| >DRB Setup List |  |  |  | 1 |  |  |  | - | - |  |
| >>DRB Setup Item |  |  |  | 1..<maxnoofDRBs> |  |  |  | - | - |  |
| >>>DRB ID |  | M |  |  |  | 9.3.1.16 |  | - | - |  |
| >>>DRB Data forwarding information Response |  | O |  |  |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |  |
| >>>UL UP Parameters |  | M |  |  |  | UP Parameters 9.3.1.13 |  | - | - |  |
| >>>Flow Setup List |  | M |  |  |  | QoS Flow List 9.3.1.12 |  | - | - |  |
| >>>Flow Failed List |  | O |  |  |  | Flow Failed List 9.3.1.45 |  | - | - |  |
| >>>Data Forwarding from EUTRAN Response List |  |  |  | 0..<maxnoofDataForwardingTunnelfromE-UTRAN> |  |  |  | YES | ignore |  |
| >>>>Data Forwarding Information |  |  | M |  |  | 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP for inter-system handover. | - | - |  |
| >DRB Failed List |  |  |  | 0.. 1 |  |  |  | - | - |  |
| >>DRB Failed Item |  |  |  | 1..<maxnoofDRBs> |  |  |  | - | - |  |
| >>>DRB ID |  | M |  |  |  | 9.3.1.16 |  | - | - |  |
| >>>Cause |  | M |  |  |  | 9.3.1.2 |  | - | - |  |
| >Redundant NG DL UP Transport Layer Information |  | O |  |  |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |  |
| >Used Redundant PDU Session Information |  | O |  |  |  | 9.3.1.80 |  | YES | ignore |  |
| Range bound |  |  |  | Explanation |  |  |  |  |
| maxnoofDRBs |  |  |  | Maximum no. of DRBs for a UE. Value is 32. |  |  |  |  |
| maxnoofPDUSessionResource |  |  |  | Maximum no. of PDU Sessions for a UE. Value is 256. |  |  |  |  |

Not modified

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-MaxCIDEHCDL,

id-ignoreMappingRuleIndication,

id-DataForwardingfromE-UTRANRequestList,

id-DataForwardingfromE-UTRANResponseList,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofExtNRCGI,

maxnoofDataForwardingTunnelfromE-UTRAN

FROM E1AP-Constants

not modified

DRB-To-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-NG-RAN

DRB-To-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

qos-flow-Information-To-Be-Setup					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer	OPTIONAL,

pDCP-SN-Status-Information								PDCP-SN-Status-Information								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS						CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters		PRESENCE optional}|

{ID id-DAPSRequestInfo				CRITICALITY ignore	EXTENSION DAPSRequestInfo					PRESENCE optional}|

{ID id-ignoreMappingRuleIndication	CRITICALITY reject	EXTENSION IgnoreMappingRuleIndication	PRESENCE optional}|

{ID id- DataForwardingfromE-UTRANRequestList				CRITICALITY ignore	EXTENSION 	DataForwardingfromE-UTRANRequest-List		PRESENCE optional	},

...

}

Not modified

DRB-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-NG-RAN

DRB-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DataForwardingfromE-UTRANResponseList				CRITICALITY ignore	EXTENSION 	DataForwardingfromE-UTRANResponse-List		PRESENCE optional	},

...

}

Not modified

DataForwardingfromE-UTRANRequest-List	::= SEQUENCE (SIZE(1.. maxnoofDataForwardingTunnelfromE-UTRAN)) OF Data-Forwarding-Information-Request

DataForwardingfromE-UTRANResponse-List	::= SEQUENCE (SIZE(1.. maxnoofDataForwardingTunnelfromE-UTRAN)) OF Data-Forwarding-Information

Not modified

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Not modified

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofExtNRCGI 							INTEGER	::= 16384

maxnoofDataForwardingTunnelfromE-UTRAN		INTEGER ::= 16

-- **************************************************************

Not modified

id-Number-of-tunnels											ProtocolIE-ID ::= 127

id-DRB-Measurement-Results-Information-List						ProtocolIE-ID ::= 128

id-Extended-GNB-CU-CP-Name										ProtocolIE-ID ::= 129

id-Extended-GNB-CU-UP-Name										ProtocolIE-ID ::= 130

id-DataForwardingtoE-UTRANInformationList						ProtocolIE-ID ::= 131

id-QosMonitoringReportingFrequency								ProtocolIE-ID ::= 132

id-QoSMonitoringDisabled										ProtocolIE-ID ::= 133

id-AdditionalHandoverInfo										ProtocolIE-ID ::= 134

id-Extended-NR-CGI-Support-List									ProtocolIE-ID ::= 135

id-DataForwardingtoNG-RANQoSFlowInformationList					ProtocolIE-ID ::= 136

id-MaxCIDEHCDL													ProtocolIE-ID ::= 137

id-ignoreMappingRuleIndication									ProtocolIE-ID ::= 138

id-DirectForwardingPathAvailability								ProtocolIE-ID ::= 139

id-DataForwardingfromE-UTRANRequestList							ProtocolIE-ID ::= xxx

id-DataForwardingfromE-UTRANResponseList						ProtocolIE-ID ::= xxx

END

-- ASN1STOP