3GPP TSG-RAN WG3 Meeting #114-e 	R3-214802

# E-meeting, -, 1. - 11.11.2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1621 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Path status information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 20.10.2021 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The S1-U path fails toward particular SGW, the UE that uses this SGW can’t be admitted for EN-DC operation. However, when selecting the en-gNB, the MeNB does not know if the node has operable path towards the SGW selected for the UE. Therefore, it may try to establish EN-DC with a non-operating gNB (and be unaware of the reason for rejection or release). |  |  |  |  |  |  |  |  |
| Summary of change: |  | S1-U path status is added to the gNB Status Indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN-DC operation is slowed down and service may be subject to interruptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.17.2, 9.1.4.27, 9.2.A1 (new), ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change, ommited text not changed

### 8.7.17	gNB Status Indication

#### 8.7.17.1	General

The purpose of the gNB Status Indication procedure is to inform the eNB that the en-gNB is overloaded so that overload reduction actions can be applied. The procedure uses non-UE associated signalling.

#### 8.7.17.2	Successful Operation

Figure 8.7.17.2-1: gNB Status Indiciation procedure, successful operation

If the gNB Overload Information IE in the GNB STATUS INDICATION message is set to "overloaded", the eNB shall apply overload reduction actions until it receives a subsequent GNB STATUS INDICATION message with gNB Overload Information IE set to "not-overloaded".

The detailed overload reduction policy is up to eNB implementation.

If case of network sharing with multiple cell ID broadcast with shared X2-C signalling transport, as specified in TS 36.300 [15], the GNB STATUS INDICATION message shall contain the Interface Instance Indication IE to identify the corresponding interface instance.

If the S1-U Path Status Information IE is included in the GNB STATUS INDICATION message, the eNB shall consider the status of the S1-U path between the IP addresses indicated in the IE has changed. The new status is indicated in the Status IE in the S1-U Path Status Information IE.

Next change, ommited text not changed

#### 9.1.4.27	GNB STATUS INDICATION

This message is sent by the en-gNB to indicate to the eNB its status of overload.

Direction: en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| gNB Overload Information | M |  | ENUMERATED (overloaded, not-overloaded, ...) |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |
| S1-U Path Status Information | O |  | 9.2.A1 |  | YES | ignore |

Next change, ommited text not changed

### 9.2.A1	S1-U Path Status Information

The S1-U Path Status Information IE indicates the status of the S1-U path towards given SGW.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S1U Path List |  | 1 |  |  |
| >Path Item |  | 1..< maxnoofS1uPath |  |  |
| >>SGW IP Address | M |  | BIT STRING (1..160, ...) | SGW’s Transport Layer Address |
| >>en-gNB IP Address | M |  | BIT STRING (1..160, ...) | en-gNB’s Transport Layer Address |
| >>Status | M | ENUMERATED (available, not-available, ...) |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofS1uPath | Maximum number of paths to indicate. Value is 256. |

Next change, ommited text not changed

Next change, ommited text not changed

-- **************************************************************

--

#### -- GNB STATUS INDICATION

--

-- **************************************************************

GNBStatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{ { GNBStatusIndicationIEs} },

...

}

GNBStatusIndicationIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GNBOverloadInformation			CRITICALITY ignore	TYPE GNBOverloadInformation			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional }|

{ ID id-S1U-path-status					CRITICALITY ignore 	TYPE S1UPathStatus					PRESENCE optional },

...

}

Next change, ommited text not changed

#### -- S

S1TNLLoadIndicator ::= SEQUENCE {

dLS1TNLLoadIndicator			LoadIndicator,

uLS1TNLLoadIndicator			LoadIndicator,

iE-Extensions					ProtocolExtensionContainer { {S1TNLLoadIndicator-ExtIEs} } OPTIONAL,

...

}

S1TNLLoadIndicator-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

S1UPathStatusList ::= SEQUENCE (SIZE(1..maxnoofS1Upaths)) OF ProtocolIE-Single-Container {{ S1UPathStatus-ItemIEs }}

S1UPathStatus-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-S1UPathStatus-Item		CRITICALITY reject	TYPE S1UPathStatus-Item		PRESENCE mandatory},

...

}

S1UPathStatus-Item ::= SEQUENCE {

sGW-address						BIT STRING (SIZE(1..160, ...)),

en-gNB-address					BIT STRING (SIZE(1..160, ...)),

status							ENUMERATED {available, not-available, ...},

iE-Extensions				ProtocolExtensionContainer { { S1UPathStatus-Item-ExtIEs} } OPTIONAL,

...

}

S1UPathStatus-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Next change, ommited text not changed

Remaining text not changed