3GPP TSG-RAN3 Meeting #114-e	R3-214806

Online, 1 – 11 November 2021

Title:	[draft] Response LS on inter-MN handover without SN change

Response to:	R2-2106682

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Nokia, Nokia Shanghai Bell [TSG RAN3]

To:	TSG RAN2

Cc:

Contact Person:

#### Name:	Krzysztof Kordybach

E-mail Address:	krzysztof.kordybach@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R3-214804

1. Overall Description:

RAN3 would like to thank RAN2 for the LS on inter-MN handover without SN change! RAN3 has discussed the questions and concluded as follows:

Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN?

Answer 1: RAN3 understands the existing description in the TS 37.340 that the MN shall include the SN UE X2/XnAP ID when it wishes to enable the SN to identify the UE context. Therefore, it is not mandatory to include the ID always in this scenario. However, if the MN does not include the SN UE X2/XnAP ID, the SN has no mean to identify the existing UE context and must prepare the Addition anew.

Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration?

Answer 2: RAN3 understands the existing description in the TS 37.340 that purpose of the SN UE X2/XnAP ID, if included in the Addition, is to enable the SN to find the existing UE context and thus the SCG configuration. Also, the MN is obliged to provide the SCG configuration separately only if the SN changes. Therefore, the SN may use the SN UE X2/XnAP ID alone to prepare the delta configuration.

The existing specification seems clear in the context of the use of the SN UE X2/XnAP ID. However, considering that a doubt has arisen, RAN3 added a note to the stage-2 description of the Addition procedure (please, see the attached draft CR).

2. Actions:

To RAN3 group.

ACTION: 	RAN3 respectfully asks RAN2 to take the above answers into consideration.

3. Date of Next TSG-RAN WG3 Meeting:

TSG-RAN3 Meeting #114-bis-e 		17 – 26 Jan 2022			e-Meeting

TSG-RAN3 Meeting #115-e 		21 Feb – 3 Mar 2022			e-Meeting