3GPP TSG-RAN WG3 Meeting #114-e	   				   R3-214837

Online, November 1st – 12th 2021

Agenda Item:		23.2

Source:			China Telecom

Title:				Discussion on Relay and Remote UE authorization

Document for:	 	Discussion

- Introduction

In the Sidelink Relay Rel-17 WID [1], the following mechanisms related to RAN3 should be specified (see RP-212601):

- Specify mechanisms for Relay and Remote UE authorization for L3 and L2 relaying [RAN3]

  - Re-use LTE as baseline

- Specify Control Plane procedures for U2N, including RRC connection management, system information delivery, paging mechanism and access control for Remote UE [RAN2, RAN3]

- Specify mechanisms for 5G ProSe Direct Discovery [RAN2, RAN3, RAN4]

In this contribution, we will provide our opinions on Relay and Remote UE authorization for Sidelink Relay.

- Discussion

In order to support NR sidelink relay, 5G ProSe service Authorisation information need to be made available in NG-RAN. SA2 has already specified the procedures for service authorization and the corresponding conclusions are documented in TS 23.304 [2]:

- If the UE is authorised to use 5G ProSe services, then the AMF shall include in a NGAP message sent to NG-RAN:

- "5G ProSe authorised" information, including one or more of the following:

-	whether the UE is authorized to use 5G ProSe Direct Discovery;

-	whether the UE is authorized to use 5G ProSe Direct Communication;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE.

- ProSe NR UE-PC5-AMBR, used by NG-RAN for the resource management of UE's PC5 transmission for 5G ProSe services in network scheduled mode.

- the PC5 QoS parameters for 5G ProSe used by the NG-RAN for the resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

- If the UE is authorised to use 5G ProSe services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

……

A new IE needs to be defined to indicate whether UE is authorized to use 5G ProSe services, the contents of 5G Prose Authorization information are highlighted as described above.

Proposal：A new IE needs to be defined to indicate whether UE is authorized to use 5G ProSe services, and the type of authorization information shall include one or more items as below:

-	5G ProSe Direct Discovery;

-	5G ProSe Direct Communication;

-	5G ProSe Layer-2 UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE.

- Conclusions

In this paper, we have the following proposal:

Proposal：A new IE needs to be defined to indicate whether UE is authorized to use 5G ProSe services, and the type of authorization information shall include one or more items as below:

-	5G ProSe Direct Discovery;

-	5G ProSe Direct Communication;

-	5G ProSe Layer-2 UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE.

- References

[1]	RP-212601, WID on NR sidelink relay, OPPO.

[2]	TS 23.304, Technical Specification Group Services and System Aspects; Proximity based Services (ProSe) in the 5G System (5GS).