3GPP TSG-RAN WG3 Meeting #114e	R3-214877

1~11 Nov. 2021

Online

Agenda item:	14.2

Source:	Samsung

Title:	(TP to SCG BL CR TS38.401) Discussion on SCG activation and deactivation

Document for:	Discussion & Decision

1 Introduction

The discussion part can be seen in R3-214878 (TP to SCG BL CR TS38.401) Discussion on SCG activation and deactivation.

This paper provides the corresponding TP for TS38.401.

2 Discussions

Based on above analysis, we provide the following observations and proposals.

Observation 1: The criteria for SCG (de)activation and RRC INACTIVE could be different. Network could get the benefit from the difference.

Observation 2: The MN-CU-CP/SN-CU-CP feedbacks the SCG (de)activation request based on the previous information provide by CU-UP(s) and DU immediately, instead of inquiry them one by one or parallel. It could match the UE dynamic data rate requirement better.

Observation 3: The MN-CU-CP/SN-CU-CP should collect information at each side from multiple entities (MN-CU-UP(s), SN-CU-UP(s), SN-DU) in order to trigger the SCG (de)activation procedure.

Proposal 1: Change the “FFS how to obtain the assisting information from DU or CU-UP and the content of the assisting information” to “CU/CU-CP makes the final decision of SCG (de)activation based on the assisting information obtained from CU-UP and DU, FFS the content of the assisting information”

Proposal 2: Use the exiting inactivity procedure as the base line for CU-CP to obtain the assisting information on F1/E1. FFS the detail of enhancement message could be discussed further.

Proposal 3: CU-CP set the specific criteria to CU-UP and DU for SCG (de)activation. It could be different with that for RRC INACTIVE.

Proposal 4: CU-UP shall be informed about the status of the SCG resources.

Annex – TP to TS 38.401

--------------------------------Start of the First Change-----------------------------

8.4.x	SCG Deactivation and Activation

This clause gives the NR SCG deactivation and activation in EN-DC and MR-DC with NR SN given that the en-gNB and SgNB consists of a gNB-CU and gNB-DU(s).

8.4.x.1	SN Addition with SCG Deactivation

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG deactivation.

2. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and indicates that the SCG is to be deactivated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG deactivation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU.

6. The SN sends the SN addition Request Acknowledge towards the MN.

8.4.x.2	MN initiated SN Modification with SCG Deactivation

Figure 8.4.x.2-1: SCG Deactivation in MN initiated SN Modification procedure

0. The SN-CU-CP collects the assisting information from the SN-CU-UP and the SN-DU.

1. SCG is activated.

2. The MN sends the SN Modification Request towards the SN, indicates the request of SCG deactivation.

3. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP, indicates that the SCG is to be deactivated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

4. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

5. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

6. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

7. The SN sends the SN Modification Request Acknowledge towards the MN, indicates that the SCG is deactivated.

8. The SN-CU-CP sends the SCG STATUS INDICATION message to the SN-CU-UP, indicates that the SCG is deactivated.

8.4.x.3	MN initiated SN Modification with SCG Activation

Figure 8.4.x.3-1: SCG Activation in MN initiated SN Modification procedure

0. The SN-CU-CP collects the assisting information from the SN-CU-UP and the SN-DU.

1. SCG is deactivated.

2. MN sends the SN Modification Request towards the SN, indicates the request of SCG activation.

3. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP, indicates that the SCG is to be activated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

4. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP

5. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

6. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is activated.

7. The SN sends the SN Modification Request Acknowledge towards the MN, indicates that the SCG is activated.

8. The SN-CU-CP sends the SCG STATUS INDICATION message to the SN-CU-UP, indicates that the SCG is activated.

8.4.x.4	SN initiated SN Modification with SCG Deactivation

Figure 8.4.x.4-1: SCG Deactivation in SN initiated SN Modification procedure

0. The SN-CU-CP collects the assisting information from the SN-CU-UP and the SN-DU.

1. SCG is activated.

2. The SN sends the SN Modification Require towards the MN, indicates the request of SCG deactivation.

3. The MN sends the SN Modification Confirm towards the SN.

4. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP, indicates that the SCG is to be deactivated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

5. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

6. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

7. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

Editor's note:	The order of the step 2, 3 and step 4,5,6,7 is FFS.

8. The SN-CU-CP sends the SCG STATUS INDICATION message to the SN-CU-UP, indicates that the SCG is deactivated.

8.4.x.5	SN initiated SN Modification with SCG Activation

Figure 8.4.x.5-1: SCG Activation in SN initiated SN Modification procedure

0. The SN-CU-CP collects the assisting information from the SN-CU-UP and the SN-DU.

1. SCG is deactivated.

2. The SN sends the SN Modification Require towards the MN, indicates the request of SCG activation.

3. The MN sends the SN Modification Confirm towards the SN.

4. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP, indicates that the SCG is to be activated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

5. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

6. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

7. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

Editor's note:	The order of the step 2, 3 and step 4,5,6,7 is FFS.

8. The SN-CU-CP sends the SCG STATUS INDICATION message to the SN-CU-UP, indicates that the SCG is activated.