3GPP TSG-RAN WG3 Meeting #114-e	R3-214918

E-meeting, Nov 1st – Nov 11th, 2021

Agenda item:	9.3.2.2

Source:	Qualcomm Incorporated

Title:	SN direct data forwarding

WID/SID:	Direct Data Forwarding Between NG-RAN and E-UTRAN: With Mobility between DC and SA – Release 16

Document for:	Discussion and decision

# Introduction

Agreements (RAN3 #113-e):

The additional scenarios (i.e. intra-system SN change, and intra-system handover involving MR-DC) are considered, and it is expected to have a common design for all handover scenarios involving MR-DC.

NR SA to EN-DC Handover:

Direct data forwarding is possible between the source NG-RAN node and target SgNB (SN) node.

RAN3 acknowledges that for EN-DC, the SgNB could have direct data forwarding information with its neighbouring nodes without extra configuration effort.

- One of reasons is that RAN3 specification already supports the SgNB/gNB logical nodes coexistence case via the 5GS-TAC IE in TS 36.423, in which case both could share the direct data forwarding configuration information.

WA: the option 3 below could be pursued between the following options.

- Option 2: target MN - the target MeNB is configured by OAM with the direct forwarding path availability information between the neighboring source nodes and the neighboring target SgNB nodes (i.e. not relevant to itself).

- Option 3: target SN - the target SN is configured by OAM with direct forwarding path availability information between itself and neighboring source nodes

EN-DC to NR SA Handover:

Direct data forwarding is possible between the source SN and the target NG-RAN node.

WA: the option2/3 below could be pursued among the following options

- Option 1: source MN - the source MN is configured by OAM with the direct forwarding path availability information between the source SN and the target node (i.e. not relevant to itself).

- Option 2/3: source SN/target node - the source SN or the target NG-RAN node is configured by OAM with the direct forwarding path between itself and neighboring nodes.

It was agreed by RAN2 and RAN3 that NR SA to EN-DC inter-system handover will be supported in Rel-16. In previous RAN3 meetings, companies provided contributions discussing signalling aspects to enable DL direct data forwarding between source and target nodes in NR SA to EN-DC and EN-DC to NR SA handovers. The goal was to address certain MR-DC inter-system handover scenarios that are not covered in the existing specifications. In the last RAN3 meeting (RAN3 #113-e) [1], an email discussion [2] was set up to collect the views from various companies on this topic. The agreements from the last RAN3 meeting (RAN3 #113-e) [1] are as shown above.

Agreement (RAN3 #111-e): Consider solution for all the following data forwarding scenarios of handover between SA and NSA:

- Scenario 1: both MN and SN have direct forwarding.

- Scenario 2: MN has direct forwarding, SN has no direct forwarding.

- Scenario 3 (FFS): MN has no direct forwarding, SN has direct forwarding.

- Scenario 4: neither MN nor SN has direct forwarding.

It was previously agreed in RAN3 #111-e meeting [3] that solutions should be developed for the following four identified scenarios of handover between SA and NSA.

In this contribution, we discuss the various solution options that have been considered so far for the problem and which try to address the above outstanding issues.

# DL direct data forwarding in NR SA to EN-DC HO

We first discuss the options with regard to configuration of direct path availability in the network nodes. There seem to be two options:

Option A: Target MeNB is configured by OAM with all the direct path availability information for data forwarding:

- Whether it has a direct path to the source NG-RAN node.

- Whether target SgNB has a direct path to the source NG-RAN node.

Option B: Target MeNB and target SgNB are configured by OAM on whether each node has a direct path available to the source NG-RAN node.

In case of Option B, in the email discussions and contribution papers in the previous RAN3 meetings [2], [4], [5], [6], companies proposed signaling based solutions so that target MeNB can obtain from the target SgNB the direct path information. An example signaling based solution is the following [6]: During the handover procedure, in SgNB Addition Request, target MeNB includes the source NG-RAN node ID and requests the target SgNB to check if it has a direct path to the source NG-RAN node. If target SgNB has a direct path to the source NG-RAN node, it includes in SgNB Addition Request Acknowledge an SN Direct Forwarding indicator.

The following table summarizes the two options and their advantages and disadvantages.

Table 2-1: Options for configuration of direct path information

|  | Option A: Target MeNB is configured with direct path information of itself and of the target SgNB to source NG-RAN node | Option B: Target MeNB and target SgNB are both configured with direct path information to source NG-RAN node |
| --- | --- | --- |
| Advantages | No signaling mechanism is required for target MeNB to obtain target SgNB’s direct path availability information to source NG-RAN. | Avoids the OAM burden of configuring neighbor’s neighbor information on direct path availability. |
| Disadvantages | Target MeNB needs to be configured with neighbor’s neighbor information, i.e., direct path availability of each of the possible target SgNBs. Depending on number of possible SgNBs which could be large in certain deployments – for example, a macro MN node with a number of pico SN nodes – the OAM configuration burden can be significant. | - Signaling solution is required for target MeNB to obtain from target SgNB its direct path availability information. - There is likely specification impact – ASN.1 changes and text changes. |

NR SA to EN-DC Handover:

Direct data forwarding is possible between the source NG-RAN node and target SgNB (SN) node.

RAN3 acknowledges that for EN-DC, the SgNB could have direct data forwarding information with its neighbouring nodes without extra configuration effort.

- One of reasons is that RAN3 specification already supports the SgNB/gNB logical nodes coexistence case via the 5GS-TAC IE in TS 36.423, in which case both could share the direct data forwarding configuration information.

WA: the option 3 below could be pursued between the following options.

- Option 2: target MN - the target MeNB is configured by OAM with the direct forwarding path availability information between the neighboring source nodes and the neighboring target SgNB nodes (i.e. not relevant to itself).

- Option 3: target SN - the target SN is configured by OAM with direct forwarding path availability information between itself and neighboring source nodes

In the last RAN3 meeting (RAN3 #113-e), the following agreement was reached on this case.

We would like RAN3 to confirm the Working Assumption (WA) that Option 3 could be pursued. This option corresponds to Option B in our discussion above.

Proposal 1. RAN3 to confirm the WA from the last RAN3 meeting (RAN3 #113-e). Then, Option B from Table 2-1 above, which corresponds to Option 3, is used for configuration of direct path availability in the involved nodes.

Proposal 2. In case RAN3 confirms Option B from Table 2-1 above, we propose the following signaling solution for target MeNB to obtain from target SgNB its direct path availability information:

- During the handover procedure, in SgNB Addition Request, target MeNB includes the source NG-RAN node ID and requests the target SgNB to check if it has a direct path to the source NG-RAN node.

- If target SgNB has a direct path to the source NG-RAN node, it includes in SgNB Addition Request Acknowledge an SN Direct Forwarding indicator.

Please see the CR to TS 36.423 [8] accompanying this discussion paper that provides the standards changes corresponding to Proposal 2.

We assume that the target MeNB knows whether target SgNB has a direct path to the source NG-RAN node in the discussion below for further signaling support for data forwarding.

In the case of NR SA to EN-DC handover, the following are the data forwarding scenarios to consider:

Scenario 1: Source NG-RAN has direct paths to target MeNB and target SgNB.

Scenario 2: Source NG-RAN has a direct path to target MeNB, but not to target SgNB.

Scenario 3: Source NG-RAN does not have a direct path to target MeNB but has a direct path to target SgNB.

Scenario 4: Source NG-RAN does not have a direct path to target MeNB or to target SgNB.

For Scenario 1 and Scenario 2, the NG-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from the source NG-RAN to the target MeNB.

For Scenario 4, the NG-AP Handover Required message does not include the Direct Forwarding Path Availability IE since there is no direct path available from the source NG-RAN to the target MeNB.

Proposal 3. For scenarios 1 and 2, NG-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from source NG-RAN node to target MeNB. For Scenario 4, Handover Required does not include the Direct Forwarding Path Availability IE. For Scenario 3, it is FFS whether this IE is included.

We now discuss further proposals and observations regarding signaling for supporting data forwarding in Scenarios 1, 2, and 4.

Scenario 1 and Scenario 2

The following is a brief sketch of the steps of the handover procedure (see Figure 2) that are relevant to data forwarding.

- Upon receiving the Handover Request message from the MME, target MeNB decides whether an SgNB should be added. Target MeNB selects an SgNB if it decides to add.

- Target MeNB performs an SN Addition procedure with the target SgNB requesting it to configure and reserve resources for SN terminated and MN terminated bearers that require SCG resources. In SgNB Addition Request Acknowledge, target SgNB provides data forwarding addresses for established SN terminated bearers.

- In case of Scenario 1, in Handover Request Acknowledge message to the MME, target MeNB includes the data forwarding addresses provided by target SgNB for the E-RABs corresponding to SN terminated bearers. For the E-RABs corresponding to MN terminated bearers, target MeNB includes its own addresses.

- In case of Scenario 2, target MeNB can forward data for SN terminated bearers from source NG-RAN node to the target SgNB. In this way, advantage can be taken of the fact that there is a direct path between the target MeNB and source NG-RAN node, to enable more efficient data forwarding for SN terminated bearers. In this case therefore, in Handover Request Acknowledge message to the MME, target MeNB includes only its own data forwarding addresses for all E-RABs.

Proposal 4. In case of Scenario 1, in Handover Request Acknowledge message to the MME, target MeNB includes the data forwarding addresses provided by target SgNB for the E-RABs corresponding to SN terminated bearers.

In the HANDOVER REQUEST ACKNOWLEDGE message in TS 36.413, the DL GTP-TEID IE in E-RABs in E-RABs Admitted Item IEs can be used for providing the data forwarding addresses in Proposal 4. The IE hierarchy is:

HANDOVER REQUEST ACKNOWLEDGE message: E-RABs Admitted List > E-RABs Admitted Item IEs > DL GTP-TEID.

Proposal 5. In case of Scenario 2, target MeNB forwards data from source NG-RAN node to the target SgNB, for the SN terminated bearers.

Proposal 6. In case of Scenario 2, in Handover Request Acknowledge message to the MME, target MeNB includes its own data forwarding addresses for all E-RABs.

In Proposal 5, target MeNB uses the data forwarding addresses provided in SgNB Addition Request Acknowledge to forward data to the target SgNB for the SN terminated bearers.

Scenario 3

For Scenario 3, a partial solution to the problem is discussed in [9].

Scenario 4

In case of Scenario 4, since the the NG-AP Handover Required message does not include the Direct Forwarding Path Availability IE, CN (Core Network) tunnels for indirect data forwarding are established as part of the handover procedure for all bearers; see Section 4.11.1.2.1, TS 23.502. No further changes are required in the standards to support this scenario.

Observation 1. No further changes are required in the standards to support Scenario 4, and no room for any improvements in data forwarding is foreseen.

Table 2-2 describes the specification impact of Proposals 3-6. We see that there are no ASN.1 changes involved but text changes for clarification are likely required.

Observation 2. Regarding specification impact of Proposals 3-6 for supporting direct data forwarding, there are no ASN.1 changes involved but text changes for clarification are likely required (Table 2-2).

Observation 3. Direct forwarding path availability configuration and information acquisition (Table 2-1) is independent of the signaling of the data forwarding tunnel addresses for the scenarios (Table 2-2). Proposals 3-6 are valid for both configuration options in Table 2-1.

Figure 2: Call-flow showing signalling for DL direct forwarding set up

Table 2-2: Specification impact of proposals for Scenarios 1, 2, and 4

|  | Proposal | Specification impact |  |
| --- | --- | --- | --- |
|  |  | ASN.1 changes | Text changes for clarification |
| Scenario 1 | NG-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from source NG-RAN to target MeNB. | No | Yes |
|  | In Handover Request Acknowledge message to the MME, target MeNB includes the data forwarding addresses provided by target SgNB for admitted E-RABs corresponding to SN terminated bearers. | No | Yes |
| Scenario 2 | NG-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from source NG-RAN to target MeNB. | No | Yes |
|  | Target MeNB forwards data from source NG-RAN node to the target SgNB, for the SN terminated bearers. | No | Yes |
|  | In Handover Request Acknowledge message to the MME, target MeNB includes its own data forwarding addresses for all admitted E-RABs. | No | Yes |
| Scenario 4 | NG-AP Handover Required message does not include the Direct Forwarding Path Availability IE. | No | Yes |

# DL direct data forwarding in EN-DC to NR SA HO

Similar to the discussion for NR SA to EN-DC handover, in this case there seem to be the following two options for configuring direct path availability in the network nodes:

Option A: Source MeNB is configured by OAM with all the direct path availability information for data forwarding:

- Whether it has a direct path to the target NG-RAN node.

- Whether source SgNB has a direct path to the target NG-RAN node.

Option B: Source MeNB and source SgNB are configured by OAM on whether each node has a direct path available to the target NG-RAN node.

In case of Option 2, in the email discussions and contribution papers in the previous RAN3 meetings [2], [4], [5], [7], companies proposed signaling based solutions so that source MeNB can obtain from the source SgNB the direct path information. An example signaling based solution is the following [7]: Source MeNB uses the SN Modification procedure to obtain from source SgNB information regarding whether source SgNB has a direct path to the target NG-RAN.

The following table summarizes the two options and their advantages and disadvantages. This table is similar to the table for NR SA to EN-DC handover and is included here for completeness.

Table 3-1: Options for configuration of direct path information

|  | Option A: Source MeNB is configured with direct path information of itself and of the source SgNB to target NG-RAN node | Option B: Source MeNB and Source SgNB are both configured with direct path information to target NG-RAN node |
| --- | --- | --- |
| Advantages | No signaling mechanism is required for source MeNB to obtain source SgNB’s direct path availability information to target NG-RAN. | Avoids the OAM burden of configuring neighbor’s neighbor information on direct path availability. |
| Disadvantages | Source MeNB needs to be configured with direct path availability of each of the possible source SgNBs. The OAM configuration burden can then be significant. | - Signaling solution is required for source MeNB to obtain from source SgNB its direct path availability information. - There is likely specification impact – ASN.1 changes and text changes. |

EN-DC to NR SA Handover:

Direct data forwarding is possible between the source SN and the target NG-RAN node.

WA: the option2/3 below could be pursued among the following options.

- Option 1: source MN - the source MN is configured by OAM with the direct forwarding path availability information between the source SN and the target node (i.e. not relevant to itself).

- Option 2/3: source SN/target node - the source SN or the target NG-RAN node is configured by OAM with the direct forwarding path between itself and neighboring nodes.

In the last RAN3 meeting (RAN3 #113-e), the following agreement was reached on this case.

Option 2 corresponds to Option B in our discussion above.

Proposal 7. RAN3 to confirm the WA from the last RAN3 meeting (RAN3 #113-e). We think that Option 2 is more suitable than Option 3. Then, Option B from Table 3-1 above, which corresponds to Option 2, is used for configuration of direct path availability in the involved nodes.

Proposal 8. In case RAN3 confirms Option B from Table 3-1 above, the following signaling solution for target MeNB to obtain from target SgNB its direct path availability information was proposed in [7] which we think is suitable: Source MeNB uses the SN Modification procedure to obtain from source SgNB information regarding whether source SgNB has a direct path to the target NG-RAN.

We assume that the source MeNB knows whether source SgNB has a direct path to the target NG-RAN node in the discussion below for further signaling support for data forwarding.

In the case of EN-DC to NR SA handover, the following are the data forwarding scenarios to consider:

Scenario 1: Both source MeNB and source SgNB have direct path to target NG-RAN node.

Scenario 2: Source MeNB has direct path to target NG-RAN node, but source SgNB does not.

Scenario 3: Source MeNB does not have direct path to target NG-RAN node, but source SgNB does.

Scenario 4: Both source MeNB and source SgNB do not have direct path to target NG-RAN node.

We have a similar proposal as Proposal 3 in case of NR SA to EN-DC handover.

Proposal 9. For scenarios 1 and 2, S1-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from source MeNB to target NG-RAN node. For Scenario 4, Handover Required does not include the Direct Forwarding Path Availability IE. For Scenario 3, it is FFS whether this IE is included.

We now discuss further proposals regarding signaling in Scenarios 1, 2, and 4.

Scenario 1

In Scenario 1, in the Handover Command message from the MME, source MeNB receives direct data forwarding addresses for all the bearers that have been admitted by the target NG-RAN node. Source MeNB forwards the addresses for the SN terminated bearers to the source SgNB. Source MeNB may use the SN Modification procedure for this purpose. Source SgNB uses these addresses for direct data forwarding to the target NG-RAN node.

Proposal 10. In case of Scenario 1, source MeNB forwards the data forwarding addresses for the SN terminated bearers received in the Handover Command message, to the source SgNB.

Please see the CR to TS 36.423 [8] accompanying this discussion paper that provides the standards changes corresponding to Proposal 10.

Scenario 2

In Scenario 2, source MeNB can forward data for SN terminated bearers from source SgNB to the target NG-RAN node. Source MeNB provides its own addresses to the source SgNB for this purpose.

Proposal 11. In case of Scenario 2, source MeNB forwards data for SN terminated bearers from source SgNB to the target NG-RAN node and provides its own addresses to the source SgNB for this purpose.

Scenario 3

For Scenario 3, a partial solution to the problem is discussed in [9].

Scenario 4

In case of Scenario 4, following a similar reasoning as for the NR SA to EN-DC handover case, we have the following conclusion.

Observation 4. No further changes are required in the standards to support Scenario 4, and no room for any improvements in data forwarding is foreseen.

Table 3-2 describes the specification impact of Proposals 9-11.

Observation 5. Regarding specification impact of Proposals 9-11 for supporting direct data forwarding, ASN.1 change is involved in scenario 1. For scenarios 2 and 4, there are no ASN.1 changes involved but text changes for clarification are likely required (Table 3-2).

Observation 6. Direct forwarding path availability configuration and information acquisition (Table 3-1) is independent of the signaling of the data forwarding tunnel addresses for the scenarios (Table 3-2). Proposals 9-11 are valid for both configuration options in Table 3-1.

Table 3-2: Specification impact of proposals for Scenarios 1, 2, and 4

|  | Proposal | Specification impact |  |
| --- | --- | --- | --- |
|  |  | ASN.1 changes | Text changes for clarification |
| Scenario 1 | S1-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from source MeNB to target NG-RAN node. | No | Yes |
|  | Source MeNB forwards the data forwarding addresses for the SN terminated bearers received in the Handover Command message, to the source SgNB. | Yes | Yes |
| Scenario 2 | S1-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from source MeNB to target NG-RAN node. | No | Yes |
|  | Source MeNB forwards data for SN terminated bearers from source SgNB to the target NG-RAN node and provides its own addresses to the source SgNB for this purpose. | No | Yes |
| Scenario 4 | S1-AP Handover Required message does not include the Direct Forwarding Path Availability IE. | No | Yes |

# Conclusions

Based on the discussion in the paper, we have the following observations and proposals.

DL direct data forwarding in NR SA to EN-DC HO

Proposal 1. RAN3 to confirm the WA from the last RAN3 meeting (RAN3 #113-e). Then, Option B from Table 2-1 above, which corresponds to Option 3, is used for configuration of direct path availability in the involved nodes.

Proposal 2. In case RAN3 confirms Option B from Table 2-1 above, we propose the following signaling solution for target MeNB to obtain from target SgNB its direct path availability information:

- During the handover procedure, in SgNB Addition Request, target MeNB includes the source NG-RAN node ID and requests the target SgNB to check if it has a direct path to the source NG-RAN node.

- If target SgNB has a direct path to the source NG-RAN node, it includes in SgNB Addition Request Acknowledge an SN Direct Forwarding indicator.

Proposal 3. For scenarios 1 and 2, NG-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from source NG-RAN node to target MeNB. For Scenario 4, Handover Required does not include the Direct Forwarding Path Availability IE. For Scenario 3, it is FFS whether this IE is included.

Proposal 4. In case of Scenario 1, in Handover Request Acknowledge message to the MME, target MeNB includes the data forwarding addresses provided by target SgNB for the E-RABs corresponding to SN terminated bearers.

Proposal 5. In case of Scenario 2, target MeNB forwards data from source NG-RAN node to the target SgNB, for the SN terminated bearers.

Proposal 6. In case of Scenario 2, in Handover Request Acknowledge message to the MME, target MeNB includes its own data forwarding addresses for all E-RABs.

Observation 1. No further changes are required in the standards to support Scenario 4, and no room for any improvements in data forwarding is foreseen.

Observation 2. Regarding specification impact of Proposals 3-6 for supporting direct data forwarding, there are no ASN.1 changes involved but text changes for clarification are likely required (Table 2-2).

Observation 3. Direct forwarding path availability configuration and information acquisition (Table 2-1) is independent of the signaling of the data forwarding tunnel addresses for the scenarios (Table 2-2). Proposals 3-6 are valid for both configuration options in Table 2-1.

DL direct data forwarding in EN-DC to NR SA HO

Proposal 7. RAN3 to confirm the WA from the last RAN3 meeting (RAN3 #113-e). We think that Option 2 is more suitable than Option 3. Then, Option B from Table 3-1 above, which corresponds to Option 2, is used for configuration of direct path availability in the involved nodes.

Proposal 8. In case RAN3 confirms Option B from Table 3-1 above, the following signaling solution for target MeNB to obtain from target SgNB its direct path availability information was proposed in [7] which we think is suitable: Source MeNB uses the SN Modification procedure to obtain from source SgNB information regarding whether source SgNB has a direct path to the target NG-RAN.

Proposal 9. For scenarios 1 and 2, S1-AP Handover Required message includes the Direct Forwarding Path Availability IE indicating that there is a direct path available from source MeNB to target NG-RAN node. For Scenario 4, Handover Required does not include the Direct Forwarding Path Availability IE. For Scenario 3, it is FFS whether this IE is included.

Proposal 10. In case of Scenario 1, source MeNB forwards the data forwarding addresses for the SN terminated bearers received in the Handover Command message, to the source SgNB.

Proposal 11. In case of Scenario 2, source MeNB forwards data for SN terminated bearers from source SgNB to the target NG-RAN node and provides its own addresses to the source SgNB for this purpose.

Observation 4. No further changes are required in the standards to support Scenario 4, and no room for any improvements in data forwarding is foreseen.

Observation 5. Regarding specification impact of Proposals 9-11 for supporting direct data forwarding, ASN.1 change is involved in scenario 1. For scenarios 2 and 4, there are no ASN.1 changes involved but text changes for clarification are likely required (Table 3-2).

Observation 6. Direct forwarding path availability configuration and information acquisition (Table 3-1) is independent of the signaling of the data forwarding tunnel addresses for the scenarios (Table 3-2). Proposals 9-11 are valid for both configuration options in Table 3-1.

# References

[1] Chair notes, RAN3 #113-e.

[2] R3-214151: 	RAN3 #113-e email discussion on Direct Data Forwarding in inter-system handovers.

[3] Chair notes, RAN3 #111-e.

[4] R3-212625: 	RAN3 #112-e email discussion on Direct Data Forwarding in inter-system handovers.

[5] R3-210963: 	RAN3 #111-e email discussion on Direct Data Forwarding in inter-system handovers.

[6] R3-206182:	SN direct data forwarding.

[7] R3-206673: 	Discussion on inter-system handover from EN-DC to SA.

[8] R3-214919:    RAN3 #113-e, CR to TS 36.423, SN direct data forwarding, Qualcomm Inc.

[9] R3-xxxxxx: 	RAN3 #113-e, Discussion on direct data forwarding for mobility between DC and SA, CATT, Qualcomm Inc.