3GPP TSG-RAN WG3 Meeting #114-e	R3-214958

E-meeting, 01-11 Nov 2021

Title: 	(TP for SON BLCR for 38.423, 38.300) MRO for SN Change Failure

Source: 	Huawei

Agenda item:	10.2.1.6

Document Type:	other

# 1. Introduction

The MRO for SN Change Failure was discussed during RAN3#113-e.

# 2. Background

In RAN3-113-e meeting, there were some progresses and open issues on the MRO for SN Change Failure. Some issues as listed below still need more discussion:

Proposal: Include the following IEs in the new XnAP message besides SCGFailureInformation

a)	PSCell failure type

b)	Source PSCell CGI, if avaliable in MN

c)	Failed PSCell CGI, if available in MN

d) 	Suitable PSCell CGI

Open issues (already captured in the chair minute after the first round of discussion):

Issue 1: FFS how to support Intra-SN PSCell change after MN/SN initiated SN change for pre-R17 UEs.

Issue 2: FFS for the following IEs, and discuss whether the source SN has the UE context when it receives SCG failure information:

Mobility Information

S-NG-RAN node UE X2AP ID

M-NG-RAN node UE XnAP ID

Issue 3: FFS whether to support EN-DC and NR-DC scenarios for SN change failure in Rel-17.

Issue 4: FFS for whether there is ambiguity in SCG failure cases

To be continued...

In this paper, we’ll give our considerations based on the agreements and open issues.

# 3. Discussion

## 3.1 Intra-SN PSCell change

Issue 1: FFS how to support Intra-SN PSCell change after MN/SN initiated SN change for pre-R17 UEs.

For this issue 1, there were lots of discussions in the offline summary [1]. There are three options:

Option 1: MN always forward SCG failure report to last serving SN. If no intra-SN PSCell change, last serving SN indicates it to MN, so it’s SN change failure. A class-1 procedure should be defined. [1][2][7]

Option 2: MN asks first the last serving SN whether intra-SN PSCell change occurs. If yes, MN forward SCG failure report to last serving SN; Otherwise it’s SN change failure. A class-1 procedure should be defined. [9]

Option 3: No enhancement is needed for this specific case. The MN can depend on the measurement results and other information to decide the node that caused the SCG failure. [5][11]

In our understanding, the MN can know the measurement results from the SCG Failure Information message from the UE. Together with the overall information stored in the MN, the MN can know the right SN node responsible for the SCG failure. Here we’d like to give more detailed analysis.

First of all, we believe the MN can at least record the time elapsed since the SN node change until the SCG failure. With the following assumption in RAN3#110-e, it is desirable that the MN can know the Tstore_UE_cntxt without any transmission from the SN node. This threshold can be used for the right SN node decision at MN side.

WA: No need to transmit Time threshold (i.e. the Tstore_UE_cntxt) over network interface.

In addition, the MN can also record the source SN node information, e.g., SN1, and target SN node, e.g., SN2, where the UE detects SCG failure.

Upon receiving the measurement results from the UE in the SCG Failure Information message, the MN can know whether there is good cell in SN1 or SN2.

Case 0: long staying time in SN2 before PSCell change failure-(SN2)

In this case, the time elapsed since SN node change until the SCG failure in SN2 is larger than the threshold Tstore_UE_cntxt. Technically speaking, it is straight-forward for the MN to know that the parameters in the SN2 should be optimized to avoid the SCG failure.

In the below use cases, we assume that the time recorded in MN is below the Tstore_UE_cntxt. And the possible use cases based on the above information are listed. This can be used for the MN to decide the right SN node. Subsequently, the MN can send the initial analysis and the forward the SCG Failure Information to the right SN node for further analysis on the potential issue.

Case 1: (no) good cell in SN1, good/best cell in SN2, short time-(SN2)

There are at least one good or best cell(s) in SN2. No matter whether there is good cell in SN1, the SN1 has made suitable PSCell change decision to SN2.  As a result, the MN can decide the SN2 as the right node. Based on the stored UE context and whether there has been an intra-SN PSCell, the SN2 can further decide the SCG failure reason, e.g., too late PSCell change, change to wrong PSCell or other reasons.

Case 2: good/best cell in SN1, no good cell in SN2, short time-(SN1)

Obviously, this is kind of too early PSCell change. The MN decides the SN1 as the right node.

Case 3: no good cell in SN1, no good cell in SN2, short time-(SN1)

It is worth noting that there is no any good cell in SN1 or SN2. Therefore, it is desirable for SN1 to initiate SN change procedure. However, the SN1 made wrong decision to SN2. With this in mind, the MN can know that this is kind of change to wrong PSCell failure due to suboptimal mobility parameters in SN1.

On basis of the above analysis, it is desirable that the MN can make reasonable initial analysis for the SCG failure based on the measurement results and the recorded time. Here we also give a logical summary on the MN actions.

Fig-1 MN decision

Based on the measurement results, or the previous notification from SN to MN to indicate the serving PSCell, the MN can also determine the source PSCell and failed PSCell information. The information of source PSCell or failed PSCell may be not so exact considering potential intra-SN PSCell change. However, since the MN has identified the right SN node, it is quite possible for the right SN node to store or get the related UE context upon receiving the SCG Failure information from the MN. Therefore, it is preferred that the right SN node can further modify the initial analysis and derive the suitable and reasonable adjustment. Therefore, we believe that the option 3 can work well for the intra-SN PSCell change after MN/SN initiated SN change for pre-R17 UEs.

Proposal 1: The MN can depend on the measurement results and stored time information to support intra-SN PSCell change for pre-Rel-17 UE.

Therefore, the MN can know the right SN which caused the failure and it is also worth noting that a class 2 procedure is enough to transmit the SCGFailureInformation from the MN to the SN.

Proposal 2: Class 2 procedure is defined for transmitting SCGFailureInformation form the MN to the right SN that caused the failure.

## 3.2 Information in the Xn

Issue 2: FFS for the following IEs, and discuss whether the source SN has the UE context when it receives SCG failure information:

Mobility Information

S-NG-RAN node UE X2AP ID

M-NG-RAN node UE XnAP ID

Besides, there is no progress on the IEs a)PSCell failure type and d)suitable PSCell CGI in the new XnAP message from MN to the right SN node.

Here we’ll give our analysis on the required IEs among the above information from the MN to the SN.

First of all, as described in section 3.1 for all case 0~3 for the PSCell change failure, the MN can decide a)PSCell failure type, e.g., Too late PSCell change, Too early PSCell change or Triggering PSCell change to wrong PSCell, based on the time elapsed since the last PSCell change initialization until SCG failure and the possibly suitable cell(s).

Besides, upon the PSCell change failure, if the MN decides to continue the DC for the UE, it is highly anticipated that the MN selects a new target PSCell based on the measurement results in the SCGFailureInformation message. This can quickly recover the SCG configuration and ensure the UE experience compared to the SN decision solution.

If we wait for the source SN to re-decide the new target PSCell, it will bring additional latency due to the delivery of the SCGFailureInformation and the SN Modification Required messages. Besides, it is a bit questionable how the source SN can know whether the new PSCell belonging to a new SN is a suitable candidate for SCG connection, e.g. whether there is an Xn connection between the MN and the new SN. In this way, the MN can also send the new suitable target PSCell decision to the initiating SN node.

In other hand, if the MN decides not to continue the DC configuration for the UE. It depends on MN implementation whether to include the new suitable target PSCell in the XnAP message SCG Failure Information. Alternatively, the MN can send additional indicators to the initiating SN node to indicate whether the corresponding cell in the measurement results has direct Xn connectivity with the MN. This can assist the initiating SN node to decide the suitable PSCell if the SN node initiates SN node modification procedure.

Proposal 3: Include a)PSCell failure type in the new XnAP message to the right SN.

Proposal 4: If needed, include the following IEs in the new XnAP message:

- d) Suitable PSCell CGI if the MN decides to configure DC; or,

- the indicator to indicate whether there is Xn connectivity between the cell(s) in the measurement results and the MN if the MN decides not to configure DC.

In RAN3-113-e meeting, it was agreed that:

If the sufficient time has passed between the SN change and the report of SCG failure, the source SN may has released the UE context when it receives SCG Failure Information

To assist the initiating SN to retrieve more relevant information collected from the UE context, the MN can also provide the UE ID for the initiating SN to identify the UE. But in MR-DC, the MN does not know the C-RNTI of UE in the initiating SN. Therefore it is preferred to use the S-NG-RAN node UE XnAP ID as a reference to the UE context in the source SN.

For M-NG-RAN node UE XnAP ID, this is another kind of UE ID and can work similar as the S-NG-RAN node UE XnAP ID to assist the failure SN node to identify the UE. If the S-NG-RAN node UE XnAP ID is provided, it is not needed to duplicately transmit M-NG-RAN node UE XnAP ID.

According to the above analysis and consideration, it is preferred to include the following information:

Proposal 5: Include S-NG-RAN node UE XnAP ID for the initiating SN to identify the UE in the new XnAP message.

Here we show one possible flow chart in case of too early PSCell change if the MN decides to continue the DC configuration.

Fig-2 the SCG Failure Information

For the other remaining IEs, all of them are not needed. For the e) Mobility Information, if the SCG failure information is reported immediately, it is desirable that the failure SN node can keep the UE context, including the mobility information. For f) PSCell selection assistant information, e.g., UE history information, this is being discussed in the UE history information topic to support the report of the PSCell related UHI. Besides, there are the latest measurements results provided from the UE, which is workable for the SN node to perform the PSCell selection. For g) Initiating node type, if the failure is caused by the MN, no SCGFailureInformation will be delivered to the SN node. The transmission of the SCGFailureInformation is kind of implicit indicator of the initiating node type.

## 3.3 EN-DC and NR-DC scenarios

Issue 3: FFS whether to support EN-DC and NR-DC scenarios for SN change failure in Rel-17.

In RAN3#110-e meeting, we have agreed to support NR-DC in Rel-17:

Prioritize NR-NR DC only

Besides, in the LS [2], RAN2 also wanted RAN3 to confirm the necessary scenarios for PSCell change failure report.

RAN2 is working on detailed signalling to support such report. Furthermore, RAN2 would like to ask RAN3’s confirmation on the necessary scenarios of such report. More specifically, RAN2 kindly requests RAN3 to confirm whether all (and if not which of) the following scenarios need to be supported for MRO in SCG Failure Report:

- NR-DC,

- NE-DC,

- EN-DC, NGEN-DC.

For both NR-DC and NE-DC, they will bring modification on the NR air interface specification. As for EN-DC or NGEN-DC scenarios, the impact on LTE specification need more consideration. Considering the massive modification work and the limited TU in the future meeting, we prefer to prioritize NR-DC first in Rel-17.

Proposal 6: All MR-DC cases can be considered and prioritize NR-NR DC.

## 3.4 other SCG failure cases

Issue 4: FFS for whether there is ambiguity in SCG failure cases

Ambiguity in SCG failure cases

In [3], a scenario is introduced: after RRC Reconfiguration message for SN change initiated by SN is received by a UE, a random access procedure occurs at the serving PSCell due to other reason, e.g. for timing synchronization, then it fails. So an SCG failure report is produced. It will make source SN confused whether it’s SN change failure or not.

Fig 3: message sequence chart for RLF vs HOF failure ambiguity

In our understanding, if the UE receives the RRCConnectionReconfiguration in step 4a, it will stop any RA with the source DU. On the other hand, if the CU has triggered PSCell change but the UE detects RA failure with the DU before receiving the RRCConnectionReconfiguration in step 4a, the UE will record the RLF in source PSCell. Together with other information reported by UE, e.g., the time Since PSCell change to failure or the source PSCell, the CU can realize that the RLF occurs before the UE receives the RRCConnectionReconfiguration.

Therefore, we think the case is clear and no enhancement is needed.

Proposal 7: No ambiguity in SCG failure case.

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: The MN can depend on the measurement results and stored time information to support intra-SN PSCell change for pre-Rel-17 UE.

Proposal 2: Class 2 procedure is defined for transmitting SCGFailureInformation form the MN to the right SN that caused the failure.

Proposal 3: Include a)PSCell failure type in the new XnAP message to the right SN.

Proposal 4: If needed, include the following IEs in the new XnAP message:

- d) Suitable PSCell CGI if the MN decides to configure DC; or,

- the indicator to indicate whether there is Xn connectivity between the cell(s) in the measurement results and the MN if the MN decides not to configure DC.

Proposal 5: Include S-NG-RAN node UE XnAP ID for the initiating SN to identify the UE in the new XnAP message.

Proposal 6: All MR-DC cases can be considered and prioritize NR-NR DC.

Proposal 7: No ambiguity in SCG failure case.

# 5. Reference

- R3-214332, CB SONMDT5_MROSNchange_summary

- R2-2109208, Reply LS on RACH report for SgNB and information needed for MRO in SCG Failure Report

- R3-213817, (TP for SON BL CR for TS 38.473) MRO for SN Change Failure (Ericsson)

# Annex 1 – TP on TS 38.423

Start of the first change

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SCG Failure Report | SCG FAILURE REPORT |

Start of the next change

### 8.4.x	SCG Failure Report

#### 8.4.x.1	General

The purpose of the SCG Failure Report procedure is to transfer SCG mobility related information from M-NG-RAN node to S-NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SCG Failure Report, successful operation

An M-NG-RAN node initiates the procedure by sending an SCG FAILURE REPORT message to S-NG-RAN node. By sending the message M-NG-RAN node indicates to S-NG-RAN node that a PSCell mobility-related problem was detected.

If the PSCell Change Failure Type IE is set to "too early PSCell change" or "triggering PSCell change to wrong PSCell", then the M-NG-RAN node indicates to S-NG-RAN node that, following a PSCell change from a source PSCell to a target PSCell, an SCG failure occurred and the UE established connection either at the original PSCell (too early PSCell change), or at another PSCell (triggering PSCell change to wrong PSCell). If the PSCell Change Failure Type IE is set to "too late PSCell change", then the M-NG-RAN node indicates to S-NG-RAN node that an SCG failure occurred and there was no recent PSCell change before the SCG failure.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

Start of the next change

#### 9.1.3.x	SCG FAILURE REPORT

This message is sent by the M-NG-RAN node to report a PSCell change failure event to S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| PSCell UE Context ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PSCell Change Failure Type | C- ifUESCGfailureinformationReportContainerabsent |  | ENUMERATED (too early PSCell change, too late PSCell change, triggering PSCell change to wrong PSCell, …) |  |  |  |
| Source PSCell CGI | C- ifUESCGfailureinformationReportContainerabsentand ifPSCellChangeFailureType tooearlyPSCellchangeortriggeringPSCellchangetowrongPSCell |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failure PSCell CGI | C- ifUESCGfailureinformationReportContainerabsent |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where the failure occurred | YES | ignore |
| Suitable PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of the new suitable PSCell | YES | ignore |
| UE SCGfailureinformation Report Container | O |  | OCTET STRING | The received SCGFailureInformation or SCGFailureInformationEUTRA message defined in TS 38.331 [10] from UE. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifUESCGfailureinformationReportContainerabsent | This IE shall be present if the UE SCGfailureinformation Report Container IE is absent. |
| ifUESCGfailureinformationReportContainerabsentandifPSCellChangeFailureType tooearlyPSCellchangeortriggeringPSCellchangetowrongPSCell | This IE shall be present if the UE SCGfailureinformation Report Container IE is absent and if the PSCell Change Failure Type IE is set to the value "too early PSCell change" or "triggering PSCell change to wrong PSCell". |

Start of the next change

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication,

SCGFailureReport

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-sCGFailureReport

-------- skip unchanged part ----------

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport 			|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

sCGFailureReport							,

...

}

-------- skip unchanged part ----------

accessAndMobilityIndication XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY 				ignore

}

sCGFaiureReport XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SCGFailureReport

PROCEDURE CODE			id-sCGFailureReport

CRITICALITY				ignore

}

END

-- ASN1STOP

Start of the next change

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

-------- skip unchanged part ----------

HandoverReportType,

TargetCellinEUTRAN,

C-RNTI,

UERLFReportContainer,

Measurement-ID,

RegistrationRequest,

ReportCharacteristics,

CellToReport,

ReportingPeriodicity,

CellMeasurementResult,

UEHistoryInformationFromTheUE,

MobilityParametersInformation,

MobilityParametersModificationRange,

RACHReportInformation,

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

UESCGFailureInformationReportContainer,

PSCellChangeFailureType

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

-------- skip unchanged part ----------

id-UEHistoryInformationFromTheUE,

id-HandoverReportType,

id-HandoverCause,

id-SourceCellCGI,

id-TargetCellCGI,

id-ReEstablishmentCellCGI,

id-TargetCellinEUTRAN,

id-SourceCellCRNTI,

id-UERLFReportContainer,

id-NGRAN-Node1-Measurement-ID,

id-NGRAN-Node2-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReport,

id-ReportingPeriodicity,

id-CellMeasurementResult,

id-NG-RANnode1CellID,

id-NG-RANnode2CellID,

id-NG-RANnode1MobilityParameters,

id-NG-RANnode2ProposedMobilityParameters,

id-MobilityParametersModificationRange,

id-RACHReportInformation,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-PSCellChangeFailureType,

id-SourcePSCellCGI,

id-FailurePSCellCGI,

id-SuitablePSCellCGI,

id-UESCGFailureInformationReportContainer,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows

FROM XnAP-Constants;

-------- skip unchanged part ----------

-- **************************************************************

--

#### -- ACCESS AND MOBILITY INDICATION

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ AccessAndMobilityIndication-IEs}},

...

}

AccessAndMobilityIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RACHReportInformation			CRITICALITY ignore		TYPE RACHReportInformation			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SCG FAILURE REPORT

--

-- **************************************************************

SCGFailureReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SCGFailureReport-IEs}},

...

}

SCGFailureReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-PSCellChangeFailureType			CRITICALITY ignore		TYPE PSCellChangeFailureType				PRESENCE mandatory}|

{ ID id-SourceCellCGI				CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE conditional }|

-- This IE shall be present if the UESCGFailureInformationReportContainer IE is absent and if the PSCell Change Failure Type IE is set to the value "too early PSCell change" or "triggering PSCell change to wrong PSCell". --

{ ID id-FailureCellCGI            	CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE conditional }|

-- This IE shall be present if the UESCGFailureInformationReportContainer IE is absent.

{ ID id-SuitablePSCellCGI   	CRITICALITY ignore		TYPE GlobalNG-RANCell-ID					PRESENCE conditional }|

-- This IE shall be present if the UESCGFailureInformationReportContainer IE is absent.

{ ID id-UESCGFailureInformationReportContainer   		CRITICALITY ignore		TYPE UESCGFailureInformationReportContainer			PRESENCE optional },

...

}

END

-- ASN1STOP

Start of the next change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

-------- skip unchanged part ----------

#### -- P

-------- skip unchanged part ----------

PDUSession-List ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-ID

PDUSession-List-withCause ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-List-withCause-Item

PDUSession-List-withCause-Item ::= SEQUENCE {

pduSessionId		PDUSession-ID,

cause				Cause				OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withCause-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSession-List-withDataForwardingFromTarget ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingFromTarget-Item

PDUSession-List-withDataForwardingFromTarget-Item ::= SEQUENCE {

pduSessionId						PDUSession-ID,

dataforwardinginfoTarget			DataForwardingInfoFromTargetNGRANnode,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse		CRITICALITY reject	EXTENSION DRB-List	PRESENCE optional},

...

}

PDUSession-List-withDataForwardingRequest ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF

PDUSession-List-withDataForwardingRequest-Item

PDUSession-List-withDataForwardingRequest-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

dataforwardingInfofromSource			DataforwardingandOffloadingInfofromSource					OPTIONAL,

dRBtoBeReleasedList						DRBToQoSFlowMapping-List									OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {PDUSession-List-withDataForwardingRequest-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSession-List-withDataForwardingRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PSCellChangeFailureType ::= ENUMERATED {

tooEarlyPSCellChange,

tooLatePSCellChange,

triggeringPSCellChangeToWrongPSCell,

...

}

-------- skip unchanged part ----------

#### -- U

-------- skip unchanged part ----------

UERLFReportContainer ::= CHOICE {

nR-UERLFReportContainer			UERLFReportContainerNR,

lTE-UERLFReportContainer		UERLFReportContainerLTE,

choice-Extension		ProtocolIE-Single-Container { {UERLFReportContainer-ExtIEs} }

}

UERLFReportContainer-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UERLFReportContainerLTE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-r9 IE contained in the UEInformationResponse message (TS 36.331 [14])

UERLFReportContainerNR ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the nr-RLF-Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10])

UESecurityCapabilities ::= SEQUENCE {

nr-EncyptionAlgorithms					BIT STRING {nea1-128(1),

nea2-128(2),

nea3-128(3)} (SIZE(16, ...)),

nr-IntegrityProtectionAlgorithms		BIT STRING {nia1-128(1),

nia2-128(2),

nia3-128(3)} (SIZE(16, ...)),

e-utra-EncyptionAlgorithms				BIT STRING {eea1-128(1),

eea2-128(2),

eea3-128(3)} (SIZE(16, ...)),

e-utra-IntegrityProtectionAlgorithms	BIT STRING {eia1-128(1),

eia2-128(2),

eia3-128(3)} (SIZE(16, ...)),

iE-Extension			ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} } OPTIONAL,

...

}

UESCGFailureInformationReportContainer ::= CHOICE {

nR-UESCGFailureInformationReportContainer		UESCGFailureInformationReportContainerNR,

lTE-UESCGFailureInformationReportContainer		UESCGFailureInformationReportContainerLTE,

choice-Extension		ProtocolIE-Single-Container { {UESCGFailureInformationReportContainer-ExtIEs} }

}

UESCGFailureInformationReportContainer-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

UESCGFailureInformationReportContainerLTE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the SCGFailureInformationEUTRA message (TS 38.331 [10])

UESCGFailureInformationReportContainerNR ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the SCGFailureInformation message (TS 38.331 [10])

Start of the next change

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-sCGFailureReport												ProcedureCode ::= xx

-------- skip unchanged part ----------

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

-------- skip unchanged part ----------

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-PagingeDRXInformation																			ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-SuccessfulHOReportInformation																		ProtocolIE-ID ::= xxx

id-SliceRadioResourceStatus-List																		ProtocolIE-ID ::= yyy

id-CompositeAvailableCapacitySupplementaryUplink														ProtocolIE-ID ::= zzz

id-PSCellChangeFailureType																				ProtocolIE-ID ::= xxy

id-SourcePSCellCGI																					ProtocolIE-ID ::= xxz

id-FailurePSCellCGI																					ProtocolIE-ID ::= xyy

id-SuitablePSCellCGI																			ProtocolIE-ID ::= xyz

id-UESCGFailureInformationReportContainer																				ProtocolIE-ID ::= xzz

END

-- ASN1STOP

Start of the next change

End of the last change

# Annex 2– TP on TS 38.300

Start of the first change

#### 15.5.2.x	PSCell change failure

##### 15.5.2.x.1	General

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. The MN performs an initial analysis and may transfer the XnAP message SCG Failure Information to the relevant SN (see section 13.x in TS 37.340 [21]).

End of the last change