3GPP TSG-RAN WG3 Meeting #114-e	R3-215003

# E-meeting, 1-11 Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0184 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of NR Small Data Transmissions in INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of NR Small Data Transmissions in INACTIVE state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduction of SDT abbreviation; - SDT related handling before moving the UE to inactive states for CG based SDT - SDT access procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SDT cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.6.1, 8.9.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********************************

Skip to the first change

********************************

3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

BH	Backhaul

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

DAPS	Dual Active Protocol Stack

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

NAS	Non-Access Stratum

NID	Network identifier

NPN	Non-Public Network

PNI-NPN	Public Network Integrated Non-Public Network

O&M	Operation and Maintenance

PWS	Public Warning System

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SAP	Service Access Point

SCTP	Stream Control Transmission Protocol

SDT	Small Data Transmission

SFN	System Frame Number

SM	Session Management

SMF	Session Management Function

SNPN	Stand-alone Non-Public Network

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

********************************

Skip to the next change

********************************

8.6	RRC state transition

8.6.1	RRC connected to RRC inactive

********************************

Skip to the next change

********************************

This section gives the RRC connected to RRC inactive state transition for the UE supporting small data transmission given that gNB consists of gNB-CU and gNB-DU(s), as shown in Figure 8.6.1-x.

Figure 8.6.1-x: RRC connected to RRC inactive state transition procedure

1.	The gNB-CU decides to move  the UE to enter to RRC_INACTIVE  state from RRC_CONNECTED state .

2.	The gNB-CU sends UE CONTEXT MODIFICATION REQUEST message including the Configured Grant Request to allocate the Configured Grant for SDT for the UE in the gNB-DU.

3.	The gNB-DU responds with the UE CONTEXT MODIFICATION RESPONSE message including the Configured Grant configuration.

4.	The gNB-CU generates RRCRelease message which includes suspend configuration towards UE. The RRC message is encapsulated in UE CONTEXT MODIFICATION REQUEST or UE CONTEXT RELEASE COMMAND [FFS] message to the gNB-DU.

5.	The gNB-DU forwards RRCRelease message to UE.

6.	The gNB-DU responds with UE CONTEXT MODIFICATION RESPONSE or UE CONTEXT RELEASE COMPLETE [FFS] message.

********************************

Skip to the next change

********************************

8.9.x	Small data transmission

The procedure for supporting the small data transmission in RRC-inactive is shown in Figure 8.9.x.-1.

Figure 8.9.x-1: Small Data Transmission in RRC Inactive state.

1.	The UE sends RRCResumeRequest message and small data.

2.	The gNB-DU buffer the small data.

3.	The gNB-DU sends the INITIAL UL RRC MESSAGE TRANSFER message to the gNB-CU-CP, provides indication of SDT access and the related assistance information.

4.	The gNB-CU-CP verifies the UE, and sends UE CONTEXT SETUP REQUEST message including the stored F1 UL TEIDs to the gNB-DU.

5.	The gNB-DU responds with the UE CONTEXT SETUP RESPONSE message including the F1 DL TEIDs allocated for the SDT DRBs.

6.	The gNB-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message with a RRC Resume indication, which indicates that the UE is resuming from RRC-inactive state. The gNB-CU-CP also includes the F1 DL TEIDs received from the gNB-DU in step 5.

7.	The gNB-CU-UP responds with the BEARER CONTEXT MODIFICATION RESPONSE message.