3GPP TSG-RAN WG3  Meeting # 114-e 	R3-215008

# E-Meeting, November 1st - 11th 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | other | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IAB Congestion Mitigation MPS exemption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Multimedia Priority Service (MPS) provides priority treatment to increase the probability of an authorized Service User’s Voice, Video, and Data communication. During IAB operation under congestion the MPS traffic could be subject to IAB congestion mitigation mechanisms. To satisfy requirements per TS 22.153 and as implemented in TS 29.274 this CR proposes normative text against TS 38.473 to take those requirements into account in the IAB congestion mitigation mechamisms. Specifically, TS 29.274 Section 12.3.9.3.1 states "GTP requests related to priority traffic (i.e. eMPS as described in 3GPP TS 22.153 [62]) and emergency have the highest priority. Depending on regional/national requirements and network operator policy, these GTP requests shall be the last to be throttled, when applying traffic reduction, and the priority traffic shall be exempted from throttling due to GTP overload control up to the point where the requested traffic reduction cannot be achieved without throttling the priority traffic". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a clause that the IAB-gNB-CU should exempt priority traffic (e.g. MPS) from throttling or re-routing on a congested IAB-DU or at intermediate IAB-nodes up to the point where backhaul congestion mitigation cannot be achieved without throttling the priority traffic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A user authorized to receive MPS priority service may not receive MPS priority handling when in an IAB session during congestion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Text Proposal is provided against BL CR R3-213186 (R3-213186 CR TS 38.473 CP-based Congestion Indication for IAB Networks) This CR is resubmission of R3-211889 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R3-211889 |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

### 8.2.7	gNB-DU Status Indication

#### 8.2.7.1	General

The purpose of the gNB-DU Status Indication procedure is informing the gNB-CU that the gNB-DU is overloaded so that overload reduction actions can be applied. This procedure is also used to inform the IAB-donor-CU about a downlink congestion at an IAB-DU or an IAB-donor-DU. The procedure uses non-UE associated signalling.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: gNB-DU Status Indication procedure

If the gNB-DU Overload Information IE in the GNB-DU STATUS INDICATION message indicates that the gNB-DU is overloaded, the gNB-CU shall apply overload reduction actions until informed, with a new GNB-DU STATUS INDICATION message, that the overload situation has ceased.

The detailed overload reduction policy is up to gNB-CU implementation.

If the IAB Congestion Indication IE is contained in the GNB-DU STATUS INDICATION message, the gNB-CU shall, if supported, take it into account for backhaul congestion mitigation. Depending on regional/national requirements and network operator policy, priority traffic (e.g. MPS) at the IAB-DU and at intermediate IAB-nodes shall be exempted from overload reduction policy throttling/re-routing at the gNB-CU up to the point where the backhaul congestion mitigation cannot be achieved without throttling/re-routing the priority traffic.

If the IAB Congestion Indication IE in the GNB-DU STATUS INDICATION message only includes the Child Node Identifier IE, the gNB-CU shall, if supported, consider that the link to the child node is congested. If the IAB Congestion Indication IE in the GNB-DU STATUS INDICATION message includes the Child Node Identifier IE and the BH RLC CH ID IE, the gNB-CU shall, if supported, consider that congestion occurs to the corresponding BH RLC channel(s) over the link towards the node identified by the Child Node Identifier IE.

NOTE: The handling with respect to simultaneous presence of IAB Congestion Indication IE and the gNB-DU Overload Information IE is up to gNB-CU implementation.

#### 8.2.7.3	Abnormal Conditions

Void.

-------------------------------------------End of Change-------------------------------------------