3GPP TSG-RAN WG3 Meeting #114-e	R3-215011

E-meeting, 1- 11 November 2021

Online

Agenda Item:	9.3.6.1

Source:	CATT

Title:	Discussion on removing the restriction of cause value of “procedure cancelled”

Document for:	Discussion

Introduction

In this contribution we will discuss meaningless handover and especially the case of handover which need to be cancelled as soon as possible. In the following we will further elaborate on the use case, raise the benefit of removing the restriction on scenario and bring forward our proposals.

Discussion

The use case we would like to explore starts with a UE in handover from source NG-RAN to target NG-RAN. At some point the source NG-RAN received context release command from CN. Or NG reset procedure starts, which means that it needs initialise or re-initialise the RAN, or part of RAN NGAP UE-related contexts. At reception of the NG RESET message the NG-RAN node shall release all allocated resources on NG and Uu related to the UE association(s) [1].Correspondingly, the source NG-RAN has to cancel handover procedure via handover cannel message to avoid unnecessary resource consumption in handover procedure. Since even the handover is successful, UE cannot work in target NG-RAN.

Although cause IE is added to the handover cancellation, there is no specific/appropriate cause value for the above cases. “Unspecified” is written in handover cancel procedure as a compromise choice. But it does not helpful for identify the real reason for handover cancel procedure.

Observation 1: No specific/appropriate cause value for handover cancel procedure when sending node cancelled the procedure due to other urgent actions to be performed.

In NR, we introduced Procedure cancelled as a cause value. However, it applies to Dual Connectivity only in the current version of TS 38.423 [2]. We consider remove the restriction on use case of the cause value to cover more scenarios. In other words, we propose to use Procedure cancelled not only in NR-DC but also in single connectivity. Procedure cancelled is the most suitable cause value to represent handover cancel procedure when context release or NG reset happened. The benefit is that it allows target NG-RAN to collect information about why handover have to be cancelled for a clear reason.

It should be highlighted that, the extension of the cause value about application range does not result in a significant protocol modifications and does not affect NR-DC.

In conclusion we believe the proposed modification is a beneficial way to collect the reason for handover cancellation.

- In light of the above the following is proposed:

Proposal 1: Remove the restriction on cause value of Procedure cancelled that is used in DC only.

Conclusion

Observation 1: No specific/appropriate cause value for handover cancel procedure when sending node cancelled the procedure due to other urgent actions to be performed.

Proposal 1: Remove the restriction on cause value of Procedure cancelled that is used in DC only.

- CR reflects the proposal above is available in [3].

References

[1] 3GPP TS 38.413 V16.5.0 (2021-03)

[2] 3GPP TS 38.423 V16.5.0 (2021-03)

[3] R3-215012, Remove the restriction of cause value “procedure cancelled” for 38.423 (R15), CATT