3GPP TSG-RAN WG3 Meeting #114-e	R3-215107

E-meeting, 1-11 Nov 2021

Title: 	(TP to 38.401 SCG BL CR) SCG activation and deactivation

Source: 	Huawei

Agenda item:	14.2

Document Type:	Other

1. Introduction

The SCG activation and Deactivation were discussed several meetings, there are several open issues left:

FFS whether to support partial rejection for SCG (de)activation during SN addition. WA: Define a general cause value, e.g., Failure due to SCG (de)activation, to indicate that the request is rejected due to the rejection of SCG (de)activation. FFS whether specific reasons shall be defined.

In this contribution, we would like to discuss these open issues one by one, and provide corresponding TP for TS 38.401 baseline CR.

2. Discussion

2.1 Partial Rejection for SCG (de)activation during SN addition

In the last RAN3 meeting, there are two sub-scenarios [R3-214181] for partial rejection or full rejection:

- Sub-scenario 1: SCG activation during SN addition - Sub-scenario 2: SCG deactivation during SN addition

During SN addition procedure, the first selected SN should be the best SN for the UE, but if the full rejection is supported and the (de)activation is rejected, the MN has to select a suboptimal SN which may accept SCG (de)activation. When the best SN are available for SCG (de)activation, SN change procedure may be needed to change the SN of the UE to the best SN. In this scenario, full rejection lead to unnecessary SN change. However, if the partial rejection is supported, that the best SN will be able to accept the Conditional SN addition while reject the SCG (de)activation.

In previous RAN3 meeting, it was agreed to include a new IE with two code points in in the SN modification request message to indicate the SCG activate or de-activate is requested, and a new IE with two code points in in the SN modification response message to indicate the SCG is activated or de-activated. Also, SN can reject the SCG (de)activation when accepting SN modification request. In RAN3#112-e meeting, it was agreed to include “SCG activation result” with two code points in the SN Addition Request, but the code points design of SN Addition Request Acknowledge is depending on whether partial rejection for SCG (de)activation during SN addition is supported or not. If partly rejection is supported, the code points for SN modification response message can be reused in the SN Addition Request Acknowledgement. It is straightforward to support partial rejection for both SN addition and MN initiated SN modification.

Proposal 1: Support partial rejection for SCG activation and deactivation during SN addition.

Proposal 2: Include two code points (i.e. one for SCG activated, another for SCG deactivated) in X2AP/XnAP: SN Addition Request Acknowledgement.

2.2 New cause value

In RAN3#113-e meeting, there was an agreement achieved about cause value:

WA: Define a general cause value, e.g., Failure due to SCG (de)activation, to indicate that the request is rejected due to the rejection of SCG (de)activation. FFS whether specific reasons shall be defined.

In order to inform the sending node that the rejection is caused by the failure of SCG (de)activation, not by other cases, it is necessary to introduce a general cause to indicate that. We do not see the need to provide a more detailed reason to indicate the detailed reason which lead to the failure of SCG (de)activation considering of the complexity and usage.

Proposal 3: for X2/Xn/F1 interfaces, introduce a new cause value “SCG activation deactivation failure” to indicate that the rejection of the request is due to failure of SCG (de)activation.

As discussed in [2], the CU-UP may be able to reject the SCG deactivation as there is ongoing DL transmission via SCG, it is only needed to add a new cause value to indicate the rejection of SCG deactivation over E1 interface.

Proposal 4: For E1 interface, introduce a new cause value “SCG deactivation failure” to indicate that the rejection of the request is due to failure of SCG deactivation.

3. Proposals

- In this contribution, we discussed the leftover issues about partial rejection, CU-UP awareness, and new cause value, get the following proposals:

Proposal 1: Support partial rejection for SCG activation and deactivation during SN addition.

Proposal 2: Include two code points (i.e. one for SCG activated, another for SCG deactivated) in X2AP/XnAP: SN Addition Request Acknowledgement.

Proposal 3: for X2/Xn/F1 interfaces, introduce a new cause value “SCG activation deactivation failure” to indicate that the rejection of the request is due to failure of SCG (de)activation.

Proposal 4: For E1 interface, introduce a new cause value “SCG deactivation failure” to indicate that the rejection of the request is due to failure of SCG deactivation.

- The corresponding TP to BL CR of TS 38.401 is provided in section 5.

4. Reference

- R3-214181 Summary of Offline Discussion on MRDC2 - SCG (de)activation, ZTE (Moderator)

- R3-215108 (TP to 38.463 38.425 SCG BL CR) SCG activation and deactivation, Huawei.

5. TP to BL CR of TS 38.401

--------------------------------Start of the First Change-----------------------------

8.4.x	SCG Deactivation and Activation

This clause gives the NR SCG deactivation and activation in EN-DC and MR-DC with NR SN given that the en-gNB and SgNB consists of a gNB-CU and gNB-DU(s).

8.4.x.1	SN Addition with SCG Deactivation

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG deactivation.

2. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and indicates that the SCG is to be deactivated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP, indicates that if the SN-CU-UP accepts the SCG deactivation or not..

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG deactivation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates that if the SN-DU accepts the SCG deactivation or not.

6. The SN sends the SN addition Request Acknowledge towards the MN, indicates that if the SN accepts the SCG deactivation or not.

8.4.x.1a	SN Addition with SCG Activation

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG deactivation.

2. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and indicates that the SCG is to be activated.

Editor’s note: It is FFS if the UP shall be informed about the status of the SCG resources.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG activation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates that if the SN-DU accepts the SCG activation or not.

6. The SN sends the SN addition Request Acknowledge towards the MN, indicates that if the SN accepts the SCG activation or not.

-------------------End of the Change-------------------