3GPP TSG-RAN WG3 Meeting #114-e	R3-215139

E-meeting, 1-11 Nov 2021

Title: 	(TP to TS 38.410 BL CR) Multicast Session Management

Source: 	Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom

Agenda item:	22.2.2

Document Type: Other

1. Introduction

Based on the discussion in [1], there are several new NGAP procedures to be introduced:

Proposal 2: Introduce non UE associated class1 NGAP: Multicast Session Activation/Deactivation procedures. With NGAP: MULTICAST SESSION ACTIVATION REQUEST/RESPONSE/ FAILURE messages and MULTICAST SESSION DEACTIVATION REQUEST/RESPONSE messages, and MBS Session ID (TMGI) is included in these messages.

Proposal 3: The MBS Session ID, MBS Service Area(s) shall be provided in the highest level of NGAP: MULTICAST GROUP PAGING message.

Proposal 4: The NGAP: MULTICAST GROUP PAGING message shall consist of a list of paging areas, only page the relevant UEs in each paging area (one or several TAIs).

The Text Proposal to TS 38.410 BL CR is provided in section 3.

2. Reference

- R3-215138 Consideration on Multicast Session Management, Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom

3. Text Proposal to BL CR of TS 38.410

-----------------Start of the Changes-------------------

5	Functions of the NG interface

5.5	PDU Session Management function

The PDU Session Management function is responsible for establishing, modifying and releasing the involved PDU sessions NG-RAN resources for user data transport once a UE context is available in the NG-RAN node.

NGAP supports transparent relaying of PDU Session and MBS Session related information by the AMF as described in TS 23.502 [6].

//skip unchanged part

## 5.xx	NR MBS Session Management function

The MBS Session Management function is responsible for establishing, modifying and releasing the involved NR MBS sessions NG-RAN resources for user data transport once an MBS context is available in the NG-RAN node.

## 5.yy	Multicast Group Paging Function

The Multicast group paging function supports the sending of multicast group paging requests to the NG-RAN nodes in order to group-page UEs that have joined the Multicast MBS Session

6	Signalling procedures of the NG interface

//skip unchanged part

## 6.xx	NR MBS Session Management Procedures

The following list of MBS Session management procedures are used to establish, release, or modify NG-RAN resources for a NR MBS session:

-	Broadcast Session Resource Setup;

-	Broadcast Session Resource Release;

-	Multicast Session Activation;

-	Multicast Session Deactivation;

-	Multicast Distribution Setup;

-	Multicast Distribution Release

## 6.yy	Multicast Group Paging Procedures

The following Multicast Group Paging procedure is used to send multicast group paging requests to the NG-RAN nodes:

-	Multicast Group Paging.

-----------------End of the Changes-------------------