3GPP TSG-RAN WG3 Meeting #114-e	R3-215140

E-meeting, 1-11 Nov 2021

Title: 	(TP to TS 38.413 BL CR) Multicast Session Management

Source: 	Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom, CMCC

Agenda item:	22.2.2

Document Type:  Other

1. Introduction

- As discussed in [1], the following proposals are achieved:

Proposal 1: Update NGAP: PDU Session Resource Modify Request Transfer IE and PDU Session Resource Setup Request Transfer IE to include detailed MBS Context.

Proposal 2: Introduce non UE associated class1 NGAP: Multicast Session Activation/Deactivation procedures. With NGAP: MULTICAST SESSION ACTIVATION REQUEST/RESPONSE/ FAILURE messages and MULTICAST SESSION DEACTIVATION REQUEST/RESPONSE messages, and MBS Session ID (TMGI) is included in these messages.

Proposal 3: The MBS Session ID, MBS Service Area(s) shall be provided in the highest level of NGAP: MULTICAST GROUP PAGING message.

Proposal 4: The NGAP: MULTICAST GROUP PAGING message shall consist of a list of paging areas, only page the relevant UEs in each paging area (one or several TAIs).

Proposal 5: The NGAP: MULTICAST GROUP PAGING message carries the following information:

- MBS Session ID

- List of (Paging Area (a list of TAI) + an optional list of (UE Identity Index value and optional DRX value))

Proposal 8: Introduce non-UE associated class1 NGAP procedures:

- Multicast Distribution Setup procedure, triggered by the gNB

  - in case the gNB decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Setup Request.

  - in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the core network shall provide IP multicast DL tunnel info to the gNB, to enable IP multicast transport.

- Multicast Distribution Release procedure, triggered by the gNB or CN

- In this paper, we provide the text proposal to BL CR of TS 38.413 in Section 3.

2. Reference

- R3-215138 Consideration on Multicast Session Management, Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom

- R3-215139 (TP to TS 38.410 BL CR) Multicast Session Management, Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom

3. Text Proposal to BL CR of TS 38.413

*****************First change*******************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture description".

[3]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[11]	3GPP TS 32.422: "Trace control and configuration management".

[12]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode and in RRC inactive state".

[13]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[14]	3GPP TS 38.414: "NG-RAN; NG data transport".

[15]	3GPP TS 29.281: "General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[16]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[17]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[18]	3GPP TS 38.331: "NG-RAN; Radio Resource Control (RRC) Protocol Specification".

[19]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[20]	3GPP TS 23.007: "Technical Specification Group Core Network Terminals; Restoration procedures".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[25]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[26]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[27]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[28]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[29]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[30]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3".

[31]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[32]	3GPP TS 37.340: " Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[33]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[34]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[35]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[36]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[37]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[38]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[39]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[40]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP) ".

[41]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA), Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[42]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[xx]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2”.

*****************Next changes*******************

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Multicast Session Activation | MULTICAT SESSION ACTIVATION REQUEST | MULTICAT SESSION ACTIVATION RESPONSE | MULTICAT SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAT SESSION DEACTIVATION REQUEST | MULTICAT SESSION DEACTIVATION RESPONSE |  |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE REQUEST | MULTICAST DISTRIBUTION RELEASE RESPONSE |  |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |

*****************Next changes*******************

### 8.5.X	Multicast Group Paging

#### 8.5.X.1	General

The purpose of the Multicast Group Paging procedure is to enable the AMF to notify all involved UEs of the activation of an MBS Session.

#### 8.5.X.2	Successful Operation

Figure 8.5.X.2-1: Multicast Group Paging

The AMF initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the NG-RAN node.

At the reception of the MULTICAST GROUP PAGING message, the NG-RAN node shall perform paging of the UEs jointed MBS Session identified by the MBS Session ID IE in cells which belong to the tracking areas as indicated in the TAI List for Group Paging IE.

If the MBS Service Area Info IE is included in the MULTICAST GROUP PAGING message, the NG-RAN node shall page the UEs jointed the MBS session inside the indicated service Area within the indicated tracking areas.

If the UE Paging Info List IE is included in the Paging Area List IE in the MULTICAST GROUP PAGING message, the NG-RAN node shall, if supported, page the UEs in the indicated TAI List on the paging resources of the relevant UEs, as specified in TS 38.304 [12].

#### 8.5.1.3	Abnormal Conditions

Void.

*****************Next changes*******************

8.xx	MBS Session Management Procedures

8.xx.a	Multicast Session Activation

8.xx.a.1	General

The purpose of the Multicast Session Activation procedure is to request a NG-RAN node to activate the MBS resources of one MBS session and the corresponding MBS QoS flows.

The procedure uses non-UE-associated signalling.

8.xx.a.2	Successful Operation

Figure 8.xx.a.2-1: Multicast Session Activation, successful operation

The AMF initiates the procedure by sending a MULTICAST SESSION ACTIVATION REQUEST message to the NG-RAN node.

Upon receipt of this message, the NG-RAN node should activate the MBS resources indicated in the MULTICAST SESSION ACTIVATION REQUEST message and shall indicate in the MULTICAST SESSION ACTIVATION RESPONSE message for which MBS Session the request was fulfilled.

8.xx.a.3	Unsuccessful Operation

Figure 8.xx.a.3-1: Multicast Session Activation, unsuccessful operation

If the NG-RAN node cannot activate the MBS resources indicated in the MULTICAST SESSION ACTIVATION REQUEST message, it shall respond with a MULTICAST SESSION ACTIVATION FAILURE message with an appropriate cause value.

8.xx.a.4	Abnormal Conditions

Not applicable.

8.xx.b	Multicast Session Deactivation

8.xx.b.1	General

The purpose of the Multicast Session Deactivation procedure is to request a NG-RAN node to deactivate the MBS resources of one MBS session and the corresponding MBS QoS flows.

The procedure uses non-UE-associated signalling.

8.xx.b.2	Successful Operation

Figure 8.xx.b.2-1: Multicast Session Deactivation, successful operation

The AMF initiates the procedure by sending a MULTICAST SESSION DEACTIVATION REQUEST message to the NG-RAN node.

Upon receipt of this message, the NG-RAN node should deactivate the MBS resources indicated in the MULTICAST SESSION DEACTIVATION REQUEST message and shall indicate in the MULTICAST SESSION DEACTIVATION RESPONSE message for which MBS Session the request was fulfilled.

8.xx.b.3	Unsuccessful Operation

Not applicable.

8.xx.b.4	Abnormal Conditions

Not applicable.

8.xx.c	Multicast Distribution Setup

8.xx.c.1	General

The purpose of the Multicast Distribution Setup procedure is to assign NG-U resources for one MBS session and the corresponding MBS QoS flows. The procedure uses non-UE-associated signalling.

8.xx.c.2	Successful Operation

Figure 8.xx.c.2-1: Multicast Distribution Setup: successful operation

The NG-RAN node initiates the procedure by sending a MULTICAST DISTRIBUTION SETUP REQUEST message to the AMF. The AMF responds with a MULTICAST DISTRIBUTION SETUP RESPONSE message.

If the DL Transport Layer Information IE is included in the Multicast Distribution Setup Request Transfer IE in the MULTICAST DISTRIBUTION SETUP REQUEST message, the SMF shall consider that the included information as the downlink termination point for the included associated QoS flows for this MBS session.

If the DL Transport Layer Information IE is not included in the Multicast Distribution Setup Request Transfer IE in the MULTICAST DISTRIBUTION SETUP REQUEST message, the SMF shall interpret that the IP multicast is used for this MBS Session and provide IP Multicast Address Information IE in the Multicast Distribution Setup Response Transfer IE in the MULTICAST DISTRIBUTION SETUP RESPONSE message.

8.xx.c.3	Unsuccessful Operation

Figure 8.xx.c.3-1: Multicast Distribution Setup: unsuccessful operation.

In case the Multicast Distribution cannot be setup successfully, the AMF shall response with the MULTICAST DISTRIBUTION SETUP FAILURE message to the NG-RAN node with an appropriate cause value in the Cause IE.

8.xx.c.4	Abnormal Conditions

Not applicable.

8.xx.d	Multicast Distribution Release

8.xx.d.1	General

The purpose of the Multicast Distribution Release procedure is to enable the release of already established NG-U resources for a given MBS Session. The procedure uses non-UE-associated signalling.

8.xx.d.2	Successful Operation

Figure 8.xx.d.2-1: Multicast Distribution Release initiated by the NG-RAN node

Figure 8.xx.d.2-2: Multicast Distribution Release initiated by the AMF

The NG-RAN node or the AMF initiates the procedure by sending a MULTICAST DISTRIBUTION RELEASE REQUEST message.

Upon receipt of the MULTICAST DISTRIBUTION RELEASE REQUEST message, the receiving node shall send the MULTICAST DISTRIBUTION RELEASE RESPONSE message after successfully remove the corresponding NG-U resource for the MSB Session.

8.xx.d.3		Unsuccessful Operation

Not applicable.

8.xx.d.4	Abnormal Conditions

Not applicable.

*****************Next changes*******************

#### 9.2.4.X	MULTICAST GROUP PAGING

This message is sent by the AMF and is used to notify involved UEs about the activation of a multicast session in one or several tracking areas.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.x |  | YES | ignore |
| Paging Area List |  |  |  | 1 |  |  |  |  |  | YES |  | ignore |  |
| > Paging Area Item |  |  |  | 1..<maxnoofPagingArea> |  |  |  |  |  | - |  | - |  |
| >> TAI List for Group Paging |  |  |  | 1 |  |  |  |  |  | - |  | - |  |
| >>>TAI List for Group Paging Item |  |  |  | 1..<maxnoofTAIforGroupPaging> |  |  |  |  |  | - |  | - |  |
| >>>>TAI |  | M |  |  |  | 9.3.3.11 |  |  |  | - |  | - |  |
| >> UE Paging Info List |  |  |  | 0..1 |  |  |  |  |  | - |  | - |  |
| >>> UE Paging Info Item |  |  |  | 1..<maxnoofUEPagingInfos> |  |  |  |  |  | - |  | - |  |
| >>>> UE Identity Index Value |  | M |  |  |  | 9.3.3.23 |  |  |  | - |  | - |  |
| >>>> Paging DRX |  | O |  |  |  | 9.3.1.90 |  |  |  | - |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingArea | Maximum no. of Paging area for group paging. Value is FFS. |
| maxnoofTAIforGroupPaging | Maximum no. of TAIs for a group paging area. Value is FFS. |
| maxnoofUEPagingInfo | Maximum no. of UE Paging Information. Value is FFS. |

*****************Next changes*******************

9.2.x	MBS Session Management Messages

9.2.x.a1	MULTICAST SESSION ACTIVATION REQUEST

This message is sent by the AMF to a NG-RAN node to request previously deactivated MBS resources to be re-activated.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |

9.2.x.a2	MULTICAST SESSION ACTIVATION RESPONSE

This message is sent by the NG-RAN node to the AMF to indicate that the MBS resources previously deactivated have been activated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.a3	MULTICAST SESSION ACTIVATION FAILURE

This message is sent by the NG-RAN node to the AMF to indicate multicast session activation failure.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.b1	MULTICAST SESSION DEACTIVATION REQUEST

This message is sent by the AMF to a NG-RAN node to request to deactivate the MBS resources of a MBS Session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |

9.2.x.b2	MULTICAST SESSION DEACTIVATION RESPONSE

This message is sent by the NG-RAN node to the AMF to indicate that the MBS resources have been deactivated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.c1	MULTICAST DISTRIBUTION SETUP REQUEST

This message is sent by the NG-RAN node to request the setup of the NG-U Transport for a MBS Session.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Setup Request Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Setup Request Transfer IE specified in subclause 9.3.4.a. | YES | reject |

Editor’s note: It is FFS whether to include the MB-UPF/gNB ID in the multicast distribution setup/release related message.

9.2.x.c2	MULTICAST DISTRIBUTION SETUP RESPONSE

This message is sent by the AMF to confirm the setup of the NG-U Transport for a MBS Session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Setup Response Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Setup Response Transfer IE specified in subclause 9.3.4.b. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.c3	MULTICAST DISTRIBUTION SETUP FAILURE

This message is sent by the AMF to indicate that the setup of the NG-U Transport for a MBS Session was unsuccessful.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Setup Unsuccessful Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Setup Unsuccessful Transfer IE specified in subclause 9.3.4.c. |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.d1	MULTICAST DISTRIBUTION RELEASE REQUEST

This message is sent by the AMF and the NG-RAN node to request the release of the NG-U Transport for a MBS Session.

Direction: AMF  NG-RAN node and NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Release Request Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Request Transfer IE specified in subclause 9.3.4.d. | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.x.d2	MULTICAST DISTRIBUTION RELEASE RESPONSE

This message is sent by the NG-RAN node and the AMF to confirm the release of the NG-U Transport for a MBS Session.

Direction: NG-RAN node  AMF and AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Multicast Distribution Release Response Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Response Transfer IE specified in subclause 9.3.4.e. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

*****************Next changes*******************

9.3.1.51	QoS Flow Identifier

This IE identifies a QoS flow within a PDU Session or a MBS QoS flow within a MBS Session. The definition and use of the QoS Flow Identifier is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0..63, …) |  |

*****************Next changes*******************

9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

9.3.1.y	MBS Supported

This IE indicates MBS is supported

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Supported | M |  | ENUMERATED (true, …) |  |

9.3.1.z	Area Session ID

This IE identifies a Area Session. The definition and use of the Area Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Area Session ID | M |  | FFS |  |

*****************Next changes*******************

9.3.4.1	PDU Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 | This IE shall be present when at least one Non-GBR QoS flow is being setup and is ignored otherwise. | YES | reject |
| //skip the unchanged part |  |  |  |  |  |  |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Redundant PDU Session Information | O |  | 9.3.1.136 |  | YES | ignore |
| MBS Session Context Setup Request List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Context Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS QoS Flow Add or Modify Request List |  | 0..1 |  |  | - | - |
| >>>MBS QoS Flow add or Modify Request Item | M | 1..<maxnoofMBSQoSflows> |  |  | - | - |
| >>>>MBS QoS Flow Identifier | Ｍ |  | 9.3.1.51 |  | - | - |
| >>>>MBS QoS Flow Level QoS Parameters | Ｏ |  | 9.3.1.12 |  | - | - |
| >>>>Associated unicast QoS Flow Identifier | Ｏ |  | 9.3.1.51 |  | - | - |
| >>MBS Service Area Info | O |  | FFS |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMBSSessions | Maximum no. of MBS Sessions allowed within one PDU session. Value is FFS. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is FFS. |

*****************Next changes*******************

9.3.4.2	PDU Session Resource Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL QoS Flow per TNL Information | M |  | QoS Flow per TNL Information 9.3.2.8 | NG-RAN node endpoint of the NG-U transport bearer for delivery of DL PDUs, together with associated QoS flows. | - |  |
| //skip the unchanged part |  |  |  |  |  |  |
| Used RSN Information | O |  | Redundant PDU Session Information 9.3.1.136 |  | YES | ignore |
| Global RAN Node ID of Secondary NG-RAN Node | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| QoS Flow Failed to setup within MBS Session List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS QoS Flow Failed to Setup List | M |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows Failed to Setup. | - | - |
| MBS Supported | O |  | 9.3.1.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS Sessions allowed within one PDU session. Value is FFS. |

*****************Next changes*******************

9.3.4.3	PDU Session Resource Modify Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 |  | YES | reject |
| //skip the unchanged part |  |  |  |  |  |  |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission of the Redundant QoS Flow(s). | YES | ignore |
| Security Indication | O |  | 9.3.1.27 |  | YES | ignore |
| MBS Session Context Setup or Modify List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Context Setup or Modify Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS QoS Flow Add or Modify Request List |  | 0..1 |  |  | - | - |
| >>>MBS QoS Flow add or Modify Request Item | M | 1..<maxnoofMBSQoSflows> |  |  | - | - |
| >>>>MBS QoS Flow Identifier | Ｍ |  | 9.3.1.51 |  | - | - |
| >>>>MBS QoS Flow Level QoS Parameters | Ｏ |  | 9.3.1.12 |  | - | - |
| >>>>Associated unicast QoS Flow Identifier | Ｏ |  | 9.3.1.51 |  | - | - |
| >>MBS QoS Flow to Release List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows to be released. | - | - |
| >>MBS Service Area Info | O |  | FFS |  | - | - |
| MBS Session Release List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Release Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMultiConnectivity | Maximum no. of connectivity allowed for a UE. Value is 4. The current version of the specification supports up to 2 connectivity. |
| maxnoofMBSSessions | Maximum no. of MBS Sessions allowed within one PDU session. Value is FFS. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is FFS. |

*****************Next changes*******************

9.3.4.4	PDU Session Resource Modify Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| //skip the unchanged part |  |  |  |  |  |  |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Additional Redundant NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer for delivery of redundant DL PDUs corresponding to the modified UPF endpoint(s) received in the UL NG-U UP TNL Modify List IE of the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| MBS Context Setup or Modify Accepted List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS QoS Flow Add or Modify Accepted List |  | 0..1 |  |  | - | - |
| >>>MBS QoS Flow add or Modify Accepted Item | M | 1..<maxnoofMBSQoSflows> |  |  | - | - |
| >>>>MBS QoS Flow Identifier | Ｍ |  | 9.3.1.51 |  | - | - |
| MBS Context Failed to Setup or Modify List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS QoS Flow Failed to Setup or Modify List | M |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows Failed to Setup/Modify. | - | - |
| MBS Supported | O |  | 9.3.1.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMBSSessions | Maximum no. of MBS Sessions allowed within one PDU session. Value is FFS. |

*****************Next changes*******************

9.3.4.a	Multicast Distribution Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |
| DL Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. |

9.3.4.b	Multicast Distribution Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |
| IP Multicast Address Information |  | 0..1 |  |  |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |
| >GTP DL TEID | M |  | GTP-TEID 9.3.2.5 |  |

9.3.4.c	Multicast Distribution Setup Unsuccessful Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.4.d	Multicast Distribution Release Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |
| Cause | M |  | 9.3.1.2 |  |

9.3.4.e	Multicast Distribution Release Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Area Session ID | O |  | 9.3.1.y |  |

*****************Next changes*******************

//asn.1 part to be added

*****************End of the changes*******************