3GPP TSG-RAN WG3 Meeting #114e	R3-215183

# Online,  1st – 11th November, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0130 | Rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling PDCP Duplication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Intel Corporation, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a need to indicate if the Rel 16 dynamic MAC UE control for UL PDCP duplication is used in the gNB-DU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add in DDDS if the MAC CE control is received. Impact Analysis: The CR has limited impact on the context setup and modification procedure. The change is backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel 16 dynamic MAC CE control is not possible to use. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 0502 TS 38.473 CR 0690 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.2	DL DATA DELIVERY STATUS (PDU Type 1)

This frame format is defined to transfer feedback to allow the receiving node (i.e. the node that hosts the NR PDCP entity) to control the downlink user data flow via the sending node (i.e. the corresponding node).

The following shows the respective DL DATA DELIVERY STATUS frame. The Figure shows an example of how a frame is structured when all optional IEs (i.e. those whose presence is indicated by an associated flag) are present.

Absence of such an IE changes the position of all subsequent IEs on octet level.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 |  |  | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  | Highest Transmitted NR PDCP SN Ind | Highest Delivered NR PDCP SN Ind | Final Frame Ind. | Lost Packet Report | 1 |
| Spare |  |  | Mac CE Control received | Delivered NR PDCP SN Range Ind |  | Data rate Ind. | Retransmitted NR PDCP SN Ind | Delivered Retransmitted NR PDCP SN Ind | Cause Report | 1 |
| Desired buffer size for the data radio bearer |  |  |  |  |  |  |  |  |  | 4 |
| Desired Data Rate |  |  |  |  |  |  |  |  |  | 0 or 4 |
| Number of lost NR-U Sequence Number ranges reported |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Start of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |  | 0 or (6* Number of reported lost NR-U SN ranges) |
| End of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |  |  |
| Highest successfully delivered NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Highest transmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Cause Value |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Successfully delivered retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Number of successfully delivered out of sequence PDCP Sequence Number range |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Start of successfully delivered out of sequence PDCP Sequence Number range |  |  |  |  |  |  |  |  |  | 0 or (6* Number of successfully delivered out of sequence PDCP Sequence Number range) |
| End of successfully delivered out of sequence PDCP Sequence Number range |  |  |  |  |  |  |  |  |  |  |
| Padding |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: DL DATA DELIVERY STATUS (PDU Type 1) Format

***********************************************

SkipThe Unchanged

***********************************************

#### 5.5.3.54	Request OutOfSeq Report

Description: This parameter indicates that the node hosting the NR PDCP entity requests the corresponding node to report the NR PDCP PDU sequence number successfully delivered out of sequence in downlink delivery status report.

Value range: {0= NR PDCP PDU sequence number successfully delivered out of sequence reporting in downlink delivery status report not requested, 1= NR PDCP PDU sequence number successfully delivered out of sequence reporting in downlink delivery status report requested}.

Field length: 1 bit.

#### 5.5.3.54	MAC CE Control Received

Description: This parameter indicates that the assisting node has received the MAC CE Control from the Node hosting the PDCP entity.

Value range: {0=Not Received, 1= Received}.

Field length: 1 bit.