3GPP TSG-RAN WG3 Meeting #114-e	R3-215241

Online, 16 - 27 August 2021

Agenda item:	  22.2.2

Source:	  Qualcomm Incorporated, Huawei, Lenovo, Motorola Mobility

Title:		(TP to TS 38.473 BL CR) Multicast Session Management

Document for:		Other

# 1	Introduction

- As discussed in [1], new group paging procedure needs to be introduced:

Proposal 7: Introduce non UE associated Class 2 F1AP: Multicast Group Paging procedure to support group paging from gNB-CU to gNB-DU. And the F1AP: MULTICAST GROUP PAGINGE message includes:

- MBS Session ID

- Paging Cell List

- (Optional) Paging UE List { UE Identity Index value, DRX value, Paging UE Cell List}

- In this paper, we provide the text proposal to BL CR of TS 38.470 in Section 3.

# 2 Reference

- R3-215138	Consideration on Multicast Session Management	Huawei, CBN, China Unicom, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Telecom

- R3-215139	(TP to TS 38.410 BL CR) Multicast Session Management	Huawei, CBN, China Unicom, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Telecom

- R3-215140	(TP to TS 38.413 BL CR) Multicast Session Management	Huawei, Lenovo, Motorola Mobility, Qualcomm Incorporated

- R3-215272	(TP to TS 38.460 BL CR) Support of Multicast Distribution Setup and Release	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

- R3-215273	(TP to TS 38.463 BL CR) Support of Multicast Distribution Setup and Release	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

- R3-215271	(TP to TS 38.423 BL CR) Support of Multicast Distribution Setup and Release	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

# 3	Text Proposal

-----------------Start of the Changes-------------------

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup |  | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update |  | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update |  | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup |  | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) |  | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) |  | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) |  | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning |  | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel |  | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination |  | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal |  | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  | BAP MAPPING CONFIGURATION FAILURE |  |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  | GNB-DU RESOURCE CONFIGURATION FAILURE |  |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  | IAB TNL ADDRESS FAILURE |  |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |  |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |  |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |  |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |  |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Multicast Group Paging |  | MULTICAST GROUP PAGING |  |

-----------------Next Changes-------------------

## 8.x	NR MBS Procedures

### 8.x.1	Multicast Group Paging

#### 8.x.1.1	General

The purpose of the Multicast Group Paging procedure is to provide the paging information to enable the gNB-DU to page UEs joined the MBS session. The procedure uses non-UE associated signalling.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: Multicast Group Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a MULTICAST GROUP PAGING message.

At the reception of the MULTICAST GROUP PAGING message, the gNB-DU shall perform paging of UEs jointed MBS Session identified by the MBS Session ID IE in cells which belong to cells as indicated in the Paging Cell List IE.

If Paging UE List IE is received, gNB-DU pages each UE only in cells identified by Paging UE Cell List at UE specific paging occasions calculated using Paging DRX IE and UE Identity Index value IE.

#### 8.x.1.3	Abnormal Conditions

Not applicable.

-----------------Next Changes-------------------

#### 9.2.y.1	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| > Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| Paging UE List |  | 0...1 |  |  | YES | ignore |
| > Paging UE Item |  | 1..<maxnoofPagingUE> |  |  | - | - |
| >> UE Identity Index value | M |  | 9.3.1.39 |  | YES | reject |
| >> CHOICE Paging Identity | M |  |  |  | YES | reject |
| >>> RAN UE Paging identity | M |  | 9.3.1.43 |  | - |  |
| >>> CN UE paging identity | M |  | 9.3.1.44 |  | - |  |
| >> Paging DRX | O |  | 9.3.1.40 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| >> Paging UE Cell List |  | 1 |  |  | YES | ignore |
| >>> Paging UE Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>>> NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingUE | Maximum no. of Paging UE for group paging. Value is FFS. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |