3GPP TSG-RAN WG3 Meeting #114-e	R3-215288

# Online, Nov 1st – Nov 11th 2021

Title:	RRC configuration during inter-MN handover without SN change

Source:	Ericsson

Agenda Item:	8.3.1

Document for:	Discussion

# 1	Introduction

In the last RAN2#114-e meeting, an LS was sent to RAN3 about the scenario of inter-MN without SN change. RAN2 would ask RAN3 to clarify the interpretation of SN UE X2/XnAP ID in [1].

For the inter-MN handover without SN change scenario, RAN2 discussed how MN can indicate whether SN should apply delta signalling or full configuration. For one of the options proposed, the signalling of the fields is as shown by the example below:

- Inter-MN HO without SN change (implies SN may provide delta configuration)

  - SN UE X2/XnAP ID        present

  - sourceConfigSCG          not present

  - scg-RB-Config               not present

- Inter-MN HO without SN change (implies SN must apply full configuration)

  - SN UE X2/XnAP ID        not present

  - sourceConfigSCG          not present

  - scg-RB-Config               not present

Questions are listed below:

Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN?

Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration?

RAN3 did not reach the consensus yet in last meeting. The agreed part is:

If the target MN decides to keep the SN, the target MN sends SN Addition Request to the SN including the SN UE X2AP ID as a reference to the UE context in the SN.

This paper would continue the discussion.

# 2	Discussion

It has been confirmed by the group that as being stated in TS 37.340, the SN UE X2/XnAP ID within SN ADDITION REQUET message is used as a reference to UE context during inter-MN without SN change. Though the remaining issue is about how to interpret the presence of this IE provided that sourceConfigSCG and scg-RB-Config are absent in the CG-ConfigInfo, i.e., target MN does not have to transfer the SCG config info since no SN change.

Three options are given below:

- Option 1: change stage-2 by saying that the SN UE X2/XnAP ID is not mandatory in case of inter-MN without SN change.

- Option 2: leave to RAN2 to decide if change the absence/presence of IE sourceConfigSCG and scg-RB-Config.

- Option 3: introduce an indicator over X2AP/XnAP to explicitly indicate delta or full configuration for the SN considering the SN UE X2/XnAP ID is always present.

History needs to be examined why full configuration is applied in this scenario. During inter-MN HO without SN change, the purpose of having SN UE X2/XnAP ID is to minimize the efforts for the MN to find the UE context considering the SN is not changed. Delta configuration is another option though no benefit is seen. In other words, the SN has the flexibility to choose full or delta configuration upon its own decision.

- Observation	Full configuration is preferred in the scenario of inter-MN handover without SN change.

However, if companies insist there is potential inter-operability issue, a clear standardized way would be the final solution.  Compared the above three, Option 1 intends to change the legacy behavior which is NBC. Option 2 would need similar changes in RAN2 specification. Option 3 is more straightforward and backward compatible. Thus, Option 3 is preferred from RAN3 point of view.

Introduce a new indicator in the SN Addition Request message over X2/XnAP to explicitly indicate full or delta configuration during inter-MN without SN change.

Regarding the questions from RAN2, the answer is yes to the 1st one. For the second one, SN would not be able to understand the request is for full or delta configuration if only the SN UE X2/XnAP ID is included.

Reply to RAN2’s questions as above.

# Conclusion

We propose to solve the issue by proposing:

Observation	Full configuration is preferred in the scenario of inter-MN handover without SN change.

Proposal 1	Introduce a new indicator in the SN Addition Request message over X2/XnAP to explicitly indicate full or delta configuration during inter-MN without SN change.

Proposal 2	Reply to RAN2’s questions as above.

A reply LS is in [2]. CRs to X2AP and XnAP for both R15 and R16 are provided in [3]-[6].

# References

R3-213121, LS on inter-MN handover without SN change

R3-215289, [DRAFT] Reply LS on inter-MN handover without SN change, Ericsson

R3-215290, Introducing RRC configuration indicator during inter-MN handover without SN change, TS 36.423, R15, Ericsson

R3-215291, Introducing RRC configuration indicator during inter-MN handover without SN change, TS 36.423, R16, Ericsson

R3-215292, Introducing RRC configuration indicator during inter-MN handover without SN change, TS 38.423, R15, Ericsson

R3-215293, Introducing RRC configuration indicator during inter-MN handover without SN change, TS 38.423, R16, Ericsson