3GPP TSG-RAN WG3 #114-e	R3-215295

1-11 Nov 2021

Online

Agenda item:		22.2.2

Source:	LG Electronics

Title:	(TP for NR_MBS BL CR for TS 38.413) Discussion on multicast group paging

Document for:		Other

# 1. Text Proposal for TS 38.413

Beginning of Text Proposal to TS 38.413

#### 9.2.4.X	MULTICAST GROUP PAGING

This message is sent by the AMF and is used to notify involved UEs about the activation of a multicast session.

Editor’s Note:	 procedure text is FFS

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.x |  | YES | ignore |
| UE ID List for Paging |  | 0..1 |  |  | YES | Ignore |
| >UE ID List for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  |  |  |
| >>UE Paging Identity | M |  | 9.3.3.18 |  |  |  |
| Further IEs FFS |  |  |  | Editor’s Note: in which way and whether POs, paging area, DRX, Paging Priority etc are included and structured is FFS |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE IDs for paging. Value is FFS. |

End of Text Proposal to TS 38.413