3GPP TSG-RAN WG3 Meeting #114-e	R3-215334

1-11 Nov 2021

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell, Vodafone

Title:	Discussion on support the UE Radio Capability for Paging in RACS context

Document for:	Discussion & decision

# 1	Introduction

RAN3 received a LS ([1]) from SA2 on supporting the UE Radio Capability for Paging in RACS context. This contribution discusses the impact to RAN3 specifications.

# 2	Discussion

Per the SA2 LS, SA2 agreed the storage of the UE Radio Capability For Paging (URCFP) in the UCMF so that the UE Radio Capability ID a UE provides is sufficient for the MME and AMF to also obtain these from the UCMF, without requiring the upload of the Radio capabilities and related URCFP from the RAN. The following SA2 agreement may affect RAN3:

---

Text from S2-2106693:

An NG-RAN which supports RACS can be configured to operate with one of two modes of operation when providing the UE radio capabilities to the AMF when the NG-RAN executes a UE Radio Capability Enquiry procedure (see TS 38.331 [28]) to retrieve UE radio capabilities from the UE:

-	Mode of operation A): The NG-RAN provides to the AMF both formats (i.e the TS 38.331 [28] format and TS 36.331 [51] format). The NG-RAN derives one of the UE Radio Capability formats using local transcoding of the other format it receives from the UE and extracts the E-UTRAN UE Radio Capability for Paging and NR UE Radio Capability for Paging from the UE Radio capabilities.

Text from S2-2106819:

An E-UTRAN which supports RACS can be configured to operate with one of two modes of operation when providing the UE radio capabilities to the MME when the E-UTRAN executes a UE Radio Capability Enquiry procedure (see TS 36.331 [37]) to retrieve UE radio capabilities from the UE.

-	Mode of operation A): The E-UTRAN provides to the MME both formats (i.e the TS 36.331 [37] format and TS 38.331 [89] format).The E-UTRAN derives one of the UE Radio Capability formats using local transcoding of the other format it receives from the UE and extracts the E-UTRAN UE Radio Capability for Paging and NR UE Radio Capability for Paging from the UE Radio capabilities.

---

For NGAP, the UE Radio Capability for Paging IE (copied as below) can include both the UE Radio Capability for Paging of NR IE (encoded in TS 38.331 format) and the UE Radio Capability for Paging of E-UTRA IE (encoded in TS 36.331 format).

#### 9.3.1.68	UE Radio Capability for Paging		 (NGAP)

This IE contains paging specific UE Radio Capability information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Radio Capability for Paging of NR | O |  | OCTET STRING | Includes the RRC UERadioPagingInformation message as defined in TS 38.331 [18]. |
| UE Radio Capability for Paging of E-UTRA | O |  | OCTET STRING | Includes the RRC UERadioPagingInformation message as defined in TS 36.331 [21]. |
| UE Radio Capability for Paging of NB-IoT | O |  | OCTET STRING | Includes the RRC UERadioPagingInformation-NB message as defined in TS 36.331 [21]. |

However, there is no behaviour defined for the AMF upon the reception of the optional UE Radio Capability for Paging of NR IE and the UE Radio Capability for Paging of E-UTRA IE. We propose to add a simple text clarifying the AMF shall behave as defined in TS23.501.

Proposal 1: Update NGAP to clarify the AMF behaviour, upon the reception of the optional UE Radio Capability for Paging of NR IE and the UE Radio Capability for Paging of E-UTRA IE.

For S1AP, the UE Radio Capability for Paging IE (copied as below) can only be the UE Radio Capability for Paging (encoded in TS 36.331 format). So the SA2 agreement requires to enhance the UE CAPABILITY INFO INDICATION message to also transfer the UE Radio Capability for Paging (encoded in TS 38.331 format).

#### 9.2.1.98	UE Radio Capability for Paging 		(S1AP)

This IE contains paging specific UE Radio Capability information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Radio Capability for Paging | M |  | OCTET STRING | Includes either the UERadioPagingInformation message as defined in 10.2.2 of TS 36.331 [16], or the UERadioPagingInformation-NB message as defined in 10.6.2 of TS 36.331 [16]. |

Observation 1: Current S1AP only support the UE Radio Capability for Paging encoded in TS 36.331 format.

Possible solution is to add a new UE Radio Capability for Paging – NR Format IE in the S1AP UE CAPABILITY INFO INDICATION message, similar to the UE Radio Capability – NR Format IE. An example is shown below:

---

### 9.1.10	UE CAPABILITY INFO INDICATION

This message is sent by the eNB to provide UE Radio Capability information to the MME.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Radio Capability | M |  | 9.2.1.27 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |
| UE Application Layer Measurement Capability | O |  | BIT STRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement capability, refer to TS 25.331[10]. Bit 0 = QoE Measurement for streaming service Bit 1 = QoE Measurement for MTSI service Value ‘1’ indicates “Capable” and value ‘0’ indicates “not Capable”. Unused bits are reserved for future use. | YES | ignore |
| LTE-M Indication | O |  | 9.2.1.135 |  | YES | ignore |
| UE Radio Capability – NR Format | O |  | 9.2.1.154 |  | YES | ignore |
| UE Radio Capability for Paging – NR Format | O |  | 9.2.1.xxx |  | YES | ignore |

---

The draft NGAP CR and S1AP CR can be found in [2][3].

Proposal 2: Introduce the UE Radio Capability for Paging encoded in TS 38.331 format in the S1AP UE CAPABILITY INFO INDICATION message.

If the above proposal is agreed, we propose to send a reply LS to SA2 on RAN3 agreements.

Proposal 3: RAN3 send a reply LS to SA2/CT4 on the agreed RAN3 CRs.

# 3	Conclusions

In this contribution, we briefly analyzed the impact to RAN3 specification on support the UE Radio Capability for Paging in RACS context. Our proposals are:

Observation 1: Current S1AP only support the UE Radio Capability for Paging encoded in TS 36.331 format.

Proposal 1: Update NGAP to clarify the AMF behaviour, upon the reception of the optional UE Radio Capability for Paging of NR IE and the UE Radio Capability for Paging of E-UTRA IE.

Proposal 2: Introduce the UE Radio Capability for Paging encoded in TS 38.331 format in the S1AP UE CAPABILITY INFO INDICATION message.

Proposal 3: RAN3 send a reply LS to SA2/CT4 on the agreed RAN3 CRs.

The draft NGAP CR, S1AP CR and reply LS can be found in [2][3][4].

# References

- R3-214709/S2-2106821, LS on the UE Radio Capability for Paging in RACS context

- R3-215335, (NGAP CR) support the UE Radio Capability for Paging in RACS context

- R3-215336, (S1AP CR) support the UE Radio Capability for Paging in RACS context

- R3-215337, (draft) reply LS on the UE Radio Capability for Paging in RACS context