3GPP TSG-RAN WG3 Meeting #114-e	R3-215377

E-meeting, 01-11 Nov 2021

Title: 	Discussion on PRS measurement outside the management gap

Source: 	Huawei

Agenda item:	8.1

Document Type:	Other

# 1. Introduction

A LS from RAN1 is received [1]. In the LS, the following RAN1 working assumption are included:

Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.) - For the purpose of this feature, PRS-related conditions are expected to be specified, with the following to be down-selected: - Alt. 1: Applicable to serving cell PRS only - Alt. 2: Applicable to all PRS under conditions to PRS of non-serving cell. - Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. - Further study - Further details of which other DL signals/channels to be prioritized - How the UE determines DL PRS’s priority based on one or more of the following: - Opt. 1: Based on indication/configuration from serving gNB - Opt. 2: Other options (e.g., implicit, signalling from LMF, etc) - Whether UE can do the measurement for both inside MG (if MG is configured) and outside MG in a measurement period - How to do the PRS measurement when the conditions cannot be satisfied, e.g. when BWP switching happens - Prioritization conditions of processing PRS over other DL channels/signals or vice versa. Send an LS to RAN2, RAN3 and RAN4 informing them of this working assumption and requesting feedback in case they have concerns.

This paper discuss the above working assumption and the RAN3 impacts.

# 2. Discussion

From the working assumption it can be seen that it is about the UE PRS measurement, ie, the PRS measurement outside the MG and within a PRS processing window, the priority of PRS measurement, etc., which mainly impacts the LPP. For NRPPa, the possible impacts could be the following item:

  - How the UE determines DL PRS’s priority based on one or more of the following:

    - Opt. 1: Based on indication/configuration from serving gNB

    - Opt. 2: Other options (e.g., implicit, signalling from LMF, etc)

However, this option is still under FFS. It is better to let the RAN1/RAN2 to further discuss the above issue and design the whole procedure of UE PRS measurement outside the MG. RAN3 can evaluate the specification impacts after further agreements are reached in RAN1/RAN2.

Proposal: Wait for RAN1/RAN2 to further discuss on this PRS measurement priority issue and evaluate the RAN3 impacts after further agreements are reached in RAN1/RAN2.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal: Wait for RAN1/RAN2 to further discuss on this PRS measurement priority issue and evaluate the RAN3 impacts after further agreements are reached in RAN1/RAN2.

# 4. Reference

- R1-2108639, LS on PRS measurement outside the measurement gap.