3GPP TSG-RAN WG3 Meeting #114-e	R3-215401

Online, 1st November – 11th November 2021

Title:	(draft) Reply LS to RAN2 and SA5 on Report Amount for M4, M5, M6, M7 measurements

Response to:	R2-2108966, S5-214523

Release:	Rel-17

# Work Item:	     NR_ENDC_SON_MDT_enh

Source:	Ericsson

To:	3GPP RAN2, 3GPP SA5

Cc:

Contact Person:	Angelo Centonza

E-mail Address:	Angelo.Centonza (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	R3-215403 - CR for the introduction of the Report Amount in MDT measurement configurations in TS38.413, 
R3-215404 - CR for the introduction of the Report Amount in MDT measurement configurations in TS38.423, 
R3-215405 - CR for the introduction of the Report Amount in MDT measurement configurations in TS38.473, 
R3-215406 - CR for the introduction of the Report Amount in MDT measurement configurations in TS38.463, 
R3-215407 - CR for the introduction of the Report Amount in MDT measurement configurations in TS36.413_Rel15, 
R3-215408 - CR for the introduction of the Report Amount in MDT measurement configurations in TS36.413_Rel16, 
R3-215409 - CR for the introduction of the Report Amount in MDT measurement configurations in TS36.423_Rel15, 
R3-215410 - CR for the introduction of the Report Amount in MDT measurement configurations in TS36.423_Rel16, 
R3-215411 - draftCR for the introduction of the Report Amount in MDT measurement configurations in TS37.320_Rel15, 
R3-215412 - draftCR for the introduction of the Report Amount in MDT measurement configurations in TS37.320_Rel16

1. Overall Description:

RAN3 thanks RAN2 and SA5 for their Reply LS on Report Amount for M4, M5, M6, M7 measurements.

RAN2 confirmed that reportAmount is necessary to configure the UE with D1 measurement (part of M6 measurement) reports from the UE.

In addition RAN2 indicated that reprotAmount configuration for M4, M5 and M7 measurements is not within the scope of RAN2 and left the decision to RAN3 and/or SA5.

SA5 confirmed that Report Amount is beneficial to configure for MDT measurements M4, M5, M6 and M7.

RAN3 would like in its turn to confirm that the above information is considered and has agreed to the attached CRs which enable the configuration of Report Amount for MDT measurements M4, M5, M6 and M7.

2. Actions:

To RAN2 group

ACTION: 	RAN3 kindly asks RAN2 to take the above information into account.

To SA5 group

ACTION: 	RAN3 kindly asks SA5 to take the above information into account.

3. Date of Next RAN3 Meetings:

RAN3#114bis-e	17th January – 26th January 2021	Online

RAN3#115-e	22nd February – 3rd March 2021		Online