3GPP TSG-RAN WG3 Meeting #114-e	R3-215411

# Online, 1-11 Nov 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | draftCR | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the Report Amount in MDT measurement configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding the Report Amount to the MDT measurement configurations enables to configure the network and the UE with exact instructions on how many reports to collect for each measurement, hence optimising the UE RRC state transition process and resource and battery consumption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include reference to the Report amount configuration on MDT measurement providing control over the number of measurement samples to be collected Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 specification will be missing discussing an important supported feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.1.1, 5.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS36.413CR1843, TS36.413CR1844, TS36.423CR1640, TS36.423CR1641, TS38.413CR0695, TS38.423CR0699, TS38.473CR0823, TS38.463CR0654 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 25.133: "Requirements for support of radio resource management (FDD)".

[3]	3GPP TS 36.133: "Requirements for support of radio resource management (FDD)".

[4]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[5]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[6]	3GPP TS 32.422: "Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 25.215: "Physical Layer; Measurements (FDD)".

[8]	3GPP TS 25.225: "Physical Layer; Measurements (TDD)".

[9]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Layer; Measurements".

[10]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC); Protocol Specification".

[11]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[12]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[13]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 – Measurements".

[14]	3GPP TS 25.321: "Medium Access Control (MAC) Protocol Specification".

[15]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[17]	3GPP TS 28.552: "Technical Specification Group Services and System Aspects; Management and orchestration; 5G performance measurements".

[18]	3GPP TS 38.314: "NR; Layer 2 Measurements".

[19]	3GPP TS 38.215: " NR; Physical layer measurements".

[20]	3GPP TS 38.213: " NR; Physical layer procedures for control".

[21]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[22]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[23]	3GPP TS 36.413: "(E-UTRAN); S1 Application Protocol (S1AP)".

<<<<<<<<<<<<<<<<<<<< End of 1st set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of the 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

#### 5.2.1.1	Measurements and reporting triggers for Immediate MDT

Measurements to be performed for Immediate MDT purposes involve reporting triggers and criteria utilized for RRM. An MDT specific UE-based measurement for UL PDCP delay is applied for QoS verification purpose. In addition, there are measurements performed in eNB.

In particular, the following measurements shall be supported for Immediate MDT performance:

Measurements:

-	M1: RSRP and RSRQ measurement by UE, see TS 36.214 [9].

-	M2: Power Headroom measurement by UE, see TS 36.213 [11].

-	M3: Received Interference Power measurement by eNB, see TS 36.214 [9]. This is a cell measurement. One sample is logged each measurement collection period, where one sample corresponds to a measurement period as specified in TS 36.133 [3].

-	M4: Data Volume measurement separately for DL and UL, per QCI per UE, by eNB, see TS 36.314 [13].

-	M5: Scheduled IP Throughput for MDT measurement separately for DL and UL, per RAB per UE and per UE for the DL, per UE for the UL, by eNB, see TS 36.314 [13]. QCI values of the RABs that have contributed to a measurement value are logged with the measurement values.

-	M6: Packet Delay measurement, separately for DL and UL, per QCI per UE, see UL PDCP Delay, by the UE, and Packet Delay in the DL per QCI, by the eNB, TS 36.314 [13].

NOTE:	If the UE does not detect any UL PDCP delay based on the delay threshold and delay report interval configured by the network, the UE does not report any UL PDCP delay measurement within that period.

-	M7: Packet Loss rate measurement, separately for DL and UL per QCI per UE, by the eNB, see Packet Loss rate in the UL and Packet Uu Loss rate in the DL TS 36.314 [13].

-	M8: RSSI measurement by UE, see 3GPP TS 36.331 [5].

-	M9: RTT measurement by UE, see 3GPP TS 36.331 [5].

Measurement collection triggers:

-	For M1:

-	Event-triggered measurement reports according to existing RRM configuration for events A1, A2, A3, A4, A5 A6, B1 or B2

-	Periodic, A2 event-triggered, or A2 event triggered periodic measurement report according to MDT specific measurement configuration.

-	For M2:

-	Reception of Power Headroom Report (PHR) according to existing RRM configuration.

NOTE:	PHR is carried by MAC signalling. Thus, the existing mechanism of PHR transmission applies, see TS 36.321 [10].

-	For M3:

-	End of measurement collection period

-	For M4:

-	End of measurement collection period.

-	For M5:

-	End of measurement collection period.

-	For M6:

-	End of measurement collection period.

-	For M7:

-	End of measurement collection period.

-	For M8:

-	End of measurement collection period.

-	For M9:

-	End of measurement collection period.

The reported measurements can be controlled by the Report Amount IE for the MDT measurements supporting it, see TS 36.413 [23].

<<<<<<<<<<<<<<<<<<<< End of 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>