3GPP TSG-RAN3 Meeting # 114-e	   R3-21xxxx

# 1st - 11th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0825 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Spatial Relation Information for SRS Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to RRC TS 38.331, the Spatial Relation information IE is defined per Positioning SRS Resource, which can be up to 64 values. In F1AP TS 38.473 section 9.3.1.175 however, it is included at a per SRS Set configuration level, creating thus a discrepancy between RRC and F1AP on the usage of such information when sent from gNB-CU. The same misalignment is considered by RAN2 valid for the Periodicity List IE in 9.3.1.175 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting F1AP to: - Ignore the current Spatial Relation information and Periodicity List IEs in 9.3.1.175 - Add a new Positioning SRS Resource List IE to be in-line with RRC description Impact analysis: This CR has isolated impact towards the previous version of the specification because it is addition of new IE that aligns with the RRC behaviour |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RRC spec on the use of the Spatial Relation information when gNB is configuring UE’s SRS for UL positioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.175, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.455 CR 0048 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.3.1.175	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE for positioning purposes.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of SRS transmissions. | - |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |  |
| CHOICE Bandwidth SRS | M |  |  |  | - |  |  |
| >FR1 |  |  |  |  | - |  |  |
| >>FR1 Bandwidth | M |  | ENUMERATED (5, 10, 20, 40, 50, 80, 100, ...) |  | - |  |  |
| >FR2 |  |  |  |  | - |  |  |
| >>FR2 Bandwidth | M |  | ENUMERATED (50, 100, 200, 400,…) |  | - |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |  |
| >>Periodicity List |  | 0.. 1 |  | This IE is ignored | - |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |  |
| >>Spatial Relation Information | O |  | 9.3.1.181 | This IE is ignored | - |  |  |
| >>Pathloss Reference Information | O |  | 9.3.1.201 |  | - |  |  |
| SSB Information | O |  | 9.3.1.202 |  | - |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |  |
| Positioning SRS Resource List |  | 0..1 |  |  | YES | ignore |
| > Positioning SRS Resource item |  | 1..< maxnoSRS-PosResources> |  |  |  |  |
| >> Number of Positioning SRS Resources | O |  | INTEGER (1..64,...) | The number of Positioning SRS Resources for SRS transmission. |  |  |
| >> Spatial Relation Information | O |  | 9.3.1.181 |  |  |  |
| >> PeriodicityPosSRS | O |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |  |  |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |
| maxnoSRS-PosResources | Maximum no of SRS Positioning resources. Value is 64 |

NEXT CHANGES

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

id-AggressorgNBSetID,

id-VictimgNBSetID,

id-MeasGapSharingConfig,

id-systemInformationAreaID,

id-areaScope,

id-IntendedTDD-DL-ULConfig,

id-QosMonitoringRequest,

id-BHInfo,

id-IAB-Info-IAB-DU,

id-IAB-Info-IAB-donor-CU,

id-IAB-Barred,

id-SIB12-message,

id-SIB13-message,

id-SIB14-message,

id-UEAssistanceInformationEUTRA,

id-SL-PHY-MAC-RLC-Config,

id-SL-ConfigDedicatedEUTRA-Info,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRPRACHConfig,

id-TDD-UL-DLConfigCommonNR,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-TSCTrafficCharacteristics,

id-AdditionalPDCPDuplicationTNL-List,

id-RLCDuplicationInformation,

id-AdditionalDuplicationIndication,

id-mdtConfiguration,

id-TraceCollectionEntityURI,

id-NID,

id-NPNSupportInfo,

id-NPNBroadcastInformation,

id-AvailableSNPN-ID-List,

id-SIB10-message,

id-RequestedP-MaxFR2,

id-DLCarrierList,

id-ExtendedTAISliceSupportList,

id-E-CID-MeasurementQuantities-Item,

id-ConfiguredTACIndication,

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-SrsPosResourceList,

NEXT CHANGES

#### -- R

SKIPPED TEXT UNCHANGED

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions		INTEGER (0..500, ...)		OPTIONAL,

-- The IE shall be present if the Resource Type IE is set to “periodic” --

resourceType				ENUMERATED  {periodic, semi-persistent, aperiodic,...},

bandwidthSRS				BandwidthSRS,

sRSResourceSetList 			SRSResourceSetList				OPTIONAL,

sSBInformation				SSBInformation				OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedSRSTransmissionCharacteristics-ExtIEs} } OPTIONAL

}

RequestedSRSTransmissionCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SrsFrequency		CRITICALITY ignore EXTENSION SrsFrequency		PRESENCE optional }|

{ ID id-SrsPosResourceList	CRITICALITY ignore EXTENSION SrsPosResourceList		PRESENCE optional },

...

}

NEXT CHANGES

#### -- S

SKIPPED TEXT UNCHANGED

SRSResourceSetList ::= SEQUENCE (SIZE(1.. maxnoSRS-ResourceSets)) OF SRSResourceSetItem

SRSResourceSetItem ::= SEQUENCE {

numSRSresourcesperset		INTEGER (1..16, ...)	OPTIONAL,

periodicityList				PeriodicityList			OPTIONAL,

-- This IE is ignored

spatialRelationInfo			SpatialRelationInfo		OPTIONAL,

-- This IE is ignored

pathlossReferenceInfo		PathlossReferenceInfo	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRSResourceSetItemExtIEs } }	OPTIONAL

}

SRSResourceSetItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-ResourceSets)) OF SRSResourceSet

SRSResourceTrigger ::= SEQUENCE {

aperiodicSRSResourceTriggerList					AperiodicSRSResourceTriggerList,

iE-Extensions		ProtocolExtensionContainer { {SRSResourceTrigger-ExtIEs} }	OPTIONAL

}

SRSResourceTrigger-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SrsPosResourceList ::= SEQUENCE (SIZE (1.. maxnoSRS-PosResources)) OF SRSPosResourceSet-Item

SRSPosResourceSet-Item::= SEQUENCE {

numberOfPosSRSResourcePerSet	INTEGER (1..64, ...)		OPTIONAL,

periodicityList					PeriodicityList-Item		OPTIONAL,

spatialRelationInformation		SpatialRelationInfo			OPTIONAL,

...

}

NEXT CHANGES

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

SKIPPED TEXT UNCHANGED

id-Extended-GNB-CU-Name								ProtocolIE-ID ::= 427

id-F1CTransferPath									ProtocolIE-ID ::= 428

id-SFN-Offset										ProtocolIE-ID ::= 429

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-SrsPosResourceList								ProtocolIE-ID ::= xxx

END

-- ASN1STOP