3GPP TSG-RAN3 Meeting # 114-e	   R3-215440

1st - 11th November 2021

Agenda Item:	19.2.2

Source:	Ericsson

Title: 	TP to F1AP BL CR: Mirror impacts to NRPPa

Document for:	other

# Introduction

This TP is a mirror F1AP to the NRPPA proposals in [1] and [2] that affect F1 positioning messages

# 2	References

- R3-215438 Discussion on support of RRC-Inactive Positioning	Ericsson

- R3-215441 Discussion on latency improvements for Rel-17 positioning	Ericsson

# 3	TP to F1AP BL CR

***************************Beginning of change in TS 38.473*******************************

#### 9.2.12.13	POSITIONING INFORMATION REQUEST

This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.175 |  | YES | ignore |
| Deferred Positioning Peridocity | O |  | 9.3.1.x | Corresponds to the UE Reporting Information | YES | ignore |

***************************Skip to next change*******************************

### 9.3.1.x Deferred Positioning Peridocity (FFS)

This IE contains the UE Reporting Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting Amount | M |  | ENUMERATED ( ra1, ra2, ra4, ra8, ra16, ra32, ra64, ra-Infinity) |  |
| Reporting Interval | M |  | ENUMERATED ( noPeriodicalReporting, ri0-25, ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64) |  |

***************************Skip to next change*******************************

### 9.3.1.x	Response Time

This information element contains the response time for desired positioning QoS in terms of accuracy and latency..

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested Latency | O |  | ENUMERATED (no delay, delay tolerant,…) |  |
| Requested Accuracy | O |  | ENUMERATED (low accuracy, high accuracy,…) |  |

***************************End of change in TS 38.473*******************************