3GPP TSG-RAN3 Meeting #114-e	R3-215550

Electronic meeting, 1st - 11th Nov 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1847 | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support to address pre-emption limit issue in MC systems |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh2MCPTT |  |  |  |  |  |  | Date: |  | 2021-10-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A LS from SA6 on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems indicates that due to the pre-emption rate limitation in eNB, some of the simultaneous bearer requests for the group call will be failed when eNB is high loaded. This issue is very important for MC services and needs to be solved. To ensure the bearers for MC services can be successfully setup with less latency, pending the request in eNB and/or providing suggested retry time should be supported to solve the issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To address the Bearer pre-emption rate limit issue in MC systems, the following changes are made: 1. E-RAB Setup Pending List IE is added in E-RAB SETUP RESPONSE messages 2. Back off time IE is included in E-RAB List |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The bearer requsts for MC services will be rejected and can not be setup in time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 9.1.3.2, 9.2.1.36, 9.2.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS 38.413 CR 0704 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 8.2	E-RAB Management procedures

### 8.2.1	E-RAB Setup

#### 8.2.1.1	General

The purpose of the E-RAB Setup procedure is to assign resources on Uu and S1 for one or several E-RABs and to setup corresponding Data Radio Bearers for a given UE. The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: E-RAB Setup procedure. Successful operation.

The MME initiates the procedure by sending an E-RAB SETUP REQUEST message to the eNB.

-	The E-RAB SETUP REQUEST message shall contain the information required by the eNB to build the E-RAB configuration consisting of at least one E-RAB and for each E-RAB to setup include an E-RAB to be Setup Item IE.

Upon reception of the E-RAB SETUP REQUEST message, and if resources are available for the requested configuration, the eNB shall execute the requested E-RAB configuration. For each E-RAB and based on the E-RAB level QoS parameters IE the eNB shall establish a Data Radio Bearer and allocate the required resources on Uu. The eNB shall pass the NAS-PDU IE and the value contained in the E-RAB ID IE received for the E-RAB for each established Data Radio Bearer to the UE. The eNB does not send the NAS PDUs associated to the failed Data radio bearers to the UE. The eNB shall allocate the required resources on S1 for the E-RABs requested to be established.

If the Correlation ID IE is included in the E-RAB SETUP REQUEST message towards the eNB with L-GW function for LIPA operation, then the eNB shall use this information for LIPA operation for the concerned E-RAB.

If the SIPTO Correlation ID IE is included in the E-RAB SETUP REQUEST message towards the eNB with L-GW function for SIPTO@LN operation, then the eNB shall use this information for SIPTO@LN operation for the concerned E-RAB.

If the Bearer Type IE is included in the E-RAB SETUP REQUEST message and is set to "non IP", then the eNB shall not perform IP header compression for the concerned E-RAB.

If the Ethernet Type IE is included in the E-RAB SETUP REQUEST message and is set to "True", then the eNB shall, if supported, take this into account to perform header compression appropriately for the concerned E-RAB.

The E-RAB SETUP REQUEST message may contain

-	the UE Aggregate Maximum Bit Rate IE.

If the UE Aggregate Maximum Bit Rate IE is included in the E-RAB SETUP REQUEST the eNB shall

-	replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context;

-	use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the UE Aggregate Maximum Bit Rate IE is not contained in the E-RAB SETUP REQUEST message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

The eNB shall establish or modify the resources according to the values of the Allocation and Retention Priority IE (priority level and pre-emption indicators) and the resource situation as follows:

-	The eNB shall consider the priority level of the requested E-RAB, when deciding on the resource allocation.

-	The priority levels and the pre-emption indicators may (individually or in combination) be used to determine whether the E-RAB setup has to be performed unconditionally and immediately. If the requested E-RAB is marked as “may trigger pre-emption” and the resource situation requires so, the eNB may trigger the pre-emption procedure which may then cause the forced release of a lower priority E-RAB which is marked as “pre-emptable”. Whilst the process and the extent of the pre-emption procedure are operator-dependent, the pre-emption indicators shall be treated as follows:

1.	The values of the last received Pre-emption Vulnerability IE and Priority Level IE shall prevail.

2.	If the Pre-emption Capability IE is set to “may trigger pre-emption”, then this allocation request may trigger the pre-emption procedure.

3.	If the Pre-emption Capability IE is set to “shall not trigger pre-emption”, then this allocation request shall not trigger the pre-emption procedure.

4.	If the Pre-emption Vulnerability IE is set to “pre-emptable”, then this E-RAB shall be included in the pre-emption process.

5.	If the Pre-emption Vulnerability IE is set to “not pre-emptable”, then this E-RAB shall not be included in the pre-emption process.

6.	If the Priority Level IE is set to “no priority” the given values for the Pre-emption Capability IE and Pre-emption Vulnerability IE shall not be considered. Instead the values “shall not trigger pre-emption” and “not pre-emptable” shall prevail.

-	The E-UTRAN pre-emption process shall keep the following rules:

1.	E-UTRAN shall only pre-empt E-RABs with lower priority, in ascending order of priority.

2.	The pre-emption may be done for E-RABs belonging to the same UE or to other UEs.

The eNB shall suspend the requests and handled them later when the requests for Mission Critical services cannot be handled due to the pre-emption limit in eNB.

The eNB shall report to the MME, in the E-RAB SETUP RESPONSE message, the result for all the requested E-RABs.

-	A list of E-RABs which are successfully established shall be included in the E-RAB Setup List IE.

-	A list of E-RABs which failed to be established, if any, shall be included in the E-RAB Failed to Setup List IE.

-  A list of E-RABs which are pending and will be established after a pending time, if any, shall be included in the E-RAB Setup Pending List IE.

In case of the establishment of an E-RAB the EPC must be prepared to receive user data before the E-RAB SETUP RESPONSE message has been received.

When the eNB reports unsuccessful establishment of an E-RAB, the cause value should be precise enough to enable the MME to know the reason for an unsuccessful establishment, e.g., “Radio resources not available”, “Failure in the Radio Interface Procedure”.

When the eNB reports unsuccessful establishment of an E-RAB, the back off time shall be optionally indicated to enable the MME to know the suggested retry time for the next requests if needed.

<Next change>

#### 9.1.3.2	E-RAB SETUP RESPONSE

This message is sent by the eNB and is used to report the outcome of the request from the E-RAB SETUP REQUEST message.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| E-RAB Setup List |  | 0..1 |  |  | YES | ignore |
| >E-RAB Setup Item IEs |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  |  |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | eNB TEID. | - |  |
| E-RAB Failed to Setup List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB Setup List IE and in E-RAB Failed to Setup List IE. | YES | ignore |
| E-RAB Setup Pending List | O |  | E-RAB Pending List 9.2.1.x |  |  |  |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

<Next change>

#### 9.2.1.36	E-RAB List

This IE contains a list of E-RAB IDs with a cause value. It is used for example to indicate failed bearers or bearers to be released or the suggested re-requested time for the failed bearers.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RAB List Item |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >E-RAB ID | M |  | 9.2.1.2 |  | - | - |
| >Cause | M |  | 9.2.1.3 |  | - | - |
| >Back off time | O |  | FFS |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.2.1.x	E-RAB Pending List

This IE contains a list of E-RAB IDs with a pending time. It is used to indicate requests are pending in eNB and the E-RAB will be setup after the pending time.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RAB List Item |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >E-RAB ID | M |  | 9.2.1.2 |  | - | - |
| >Pending time | M |  | FFS |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

<End of change>