3GPP TSG-RAN WG3 Meeting #114-e	R3-215566

# E-meeting, 1-11 Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1644 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Reset procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The specifications say that theX2AP: Reset procedure resets the X2 interface and does not affect the application level configuration data exchanged during interface setup procedure or configuration update procedure, but it is not it is unclear if the X2AP: Reset procedure resets the other non-UE-related context/signalling (i.e. Periodic Resourcing Status Reporting). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the X2AP: Reset procedure only impacts UE related context/signalling. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the handling of non-UE related context/signalling in case of Reset procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent processing between nodes can lead to problems in the network, such as unnecessary measurements and reports, etc. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.3.4	Reset

8.3.4.1	General

The purpose of the Reset procedure is to align the resources in eNB1 and eNB2, or the resources in eNB and en-gNB involved in the EN-DC in the event of an abnormal failure. The procedure resets all the UE related contexts over the X2 interface. This procedure doesn’t affect the application level configuration data exchanged during, e.g., the X2 Setup procedure, EN-DC X2 Setup procedure.

The procedure uses non UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: Reset, successful operation