3GPP TSG-RAN WG3 #114-e	R3-215594

E-meeting, 1st – 11th November 2021

Source:	CATT

Title:	(TP for BL CR for TS 38.300) On country policy handling

Agenda Item:	20.2.5

Document for:	Discussion and decision

- Introduction

This is a small TP already provided in the e-mail discussion last RAN3 meeting [1]. The problem is firstly raised in [2].

In the last meeting, an issue was raised in [1], how to handle the use case “UE moves across the country (within the same cell) and not support the new PLMN of the country”.

Continue to discuss whether and how to address the use case “UE moves across the country (within the same cell) and not support the new PLMN of the country”

To be continued...

Currently, we have not concluded how to handle this issue.

In this contribution, we will further discuss the issue and provide corresponding proposal and text proposal.

- Discussion

From the Mobility Restriction List info provided by 5GC and the UE location info reported by the UE, NG-RAN could detect whether the UE moves out of the country where the PLMN it supported.

Observation 1: NG-RAN could detect whether the UE moves out of the country where the PLMN it supported by the UE location info reported from UE and the Mobility Restriction List info provided by 5GC.

When NG-RAN detects the UE has moved out of the country where the PLMN it supported, it could release RRC connection directly. To be robust, it could initiates UE CONTEXT RELEASE REQUEST procedure towards the AMF to release all the resources of the UE, with the cause value “Handover target not allowed”. Then AMF may order the NG-RAN to release the UE context, corresponding RRC connection will be released by the NG-RAN.

Proposal 1: NG-RAN should initiates UE Context Release Request towards AMF when detects a UE moves out of the PLMN it supported.

Corresponding Text Proposal for stage 2 BL CR could be found in section 5.

Proposal 2: Discuss and agree the Stage 2 TP for BL CR for TS 38.300 on handling of the case where UE moves out of the PLMN it supported.

- Conclusion

Observation 1: NG-RAN could detect whether the UE moves out of the country where the PLMN it supported by the UE location info reported from UE and the Mobility Restriction List info provided by 5GC.

Proposal 1: NG-RAN should initiates UE Context Release Request towards AMF when detects a UE moves out of the PLMN it supported.

Proposal 2: Discuss and agree the Stage 2 TP for BL CR for TS 38.300 on handling of the case where UE moves out of the PLMN it supported.

- Reference

[1] R3-213344; Country-specific routing issues; Huawei.

[2] R3-214364; CB: # 2006_NTN_Country_Routing; CATT (moderator).

- TP for TS 38.300 over R3-212793

/////////////////////////////////////////////////////////////////////// Begin of change /////////////////////////////////////////////////////////////////

### 16.x.x	RAN (Re)Selection of CN Entity

The NG-RAN node implements the NAS Node Selection Function specified in TS 38.410 [16].

For a RRC_CONNECTED UE, when the NG-RAN node is configured to ensure that the UE is using an AMF that serves the country in which the UE is located:

-	If the NG-RAN node detects that the UE is in a different country to that served by the serving AMF, the NG-RAN should perform an NG handover to change to an appropriate AMF.

- If the NG-RAN node detects the UE is in a country where the corresponding PLMN(s) are not supported by the UE, the NG-RAN should initiate UE Context Release Request towards AMF.

Editor’s note: Text may need to be revised depending on RAN2/SA2/RAN3 progress.

/////////////////////////////////////////////////////////////////////// End of Change ///////////////////////////////////////////////////////////////////////