3GPP TSG-RAN WG3 Meeting #114-e	R3-215601

E-meeting, 1 – 11 November, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of RA-based SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To support small data transmission in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new procedures for SDT with and without anchor relocation. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has an impact under funtional point of view. The impact can be considered isolated because the SDT is a new functionality for R17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SDT in RRC_INACTIVE could not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | xxxx |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

16.x	Small Data Transmission in RRC_INACTIVE

16.x.1	RA-based Small Data Transmission with anchor relocation

The overall procedure for RACH based small data transmission with anchor relocationcould be illustrated with the figure below:

Figure 16.x.1-1. RA-based SDT with anchor relocation

1.  The UE sends an RRCResumeRequest as well as UL SDT data and/or ULInformationTransfer message to the receiving gNB.

2.  	The receiving gNB locates the anchor gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure.

3.  The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE.

4-6. The receiving gNB decides to keep UE in RRC Inactive state for SDT transmission, and initiates NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF. After path switch, the UL SDT data is delivered to the UPF, the UL NAS PDU is delivered to AMF.

7. After SDT transmission is completed, the receiving gNB generates and sends RRCRelease including the Suspend indication to the UE to move the UE back to RRC_INACTIVE.

8. The receiving gNB becomes the new anchor gNB, and indicates the last serving gNB to remove the UE context by sending UE CONTEXT RELEASE.

16.x.2	RA-based Small Data Transmission without anchor relocation

The overall procedure for RACH based small data transmission without anchor relocationcould be illustrated with the figure below:

Figure 16.x.2-1. RA-based SDT without anchor relocation

1.  The UE sends an RRCResumeRequest as well as UL SDT data and/or ULInformationTransfer message to the receiving gNB.

2.  	The receiving gNB locates the anchor gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure, in the Retrieve UE Context Request the receiving gNB provides the SDT assistance info.

3.  The last serving gNB responds with the RETRIEVE UE CONTEXT RESPONSE, in which it indicates the anchor is kept, and it assigns the UL GTP-U tunnel(s) for SDT DRB(s).

4. After reception of the UE context, the receiving gNB may provide the GTP-U tunnel(s) for DL SDT. The receiving gNB handles the UL small data and or the UL signalling with the received UE context and forwards them to the last serving gNB.

5. The last serving gNB indicates the receiving gNB to release the UE context after SDT transmission is completed, it also generates and provides the RRCRelease for the UE in the UE Context Release procedure.

6. The receiving gNB forwards the RRCRelease to UE, and the UE is moved to RRC_INACTIVE.

Change Ends