3GPP TSG-RAN WG3 Meeting #114-e		R3-215605

Online, 1-11 Nov, 2021

Source:	CATT

Title:	Further Consideration of UL-AOA/ZOA Assistance Information Signalling

Agenda Item:	19.2.1

Document for:	Discussion and Decision

1 Introduction

In the last RAN3 meeting, the UL-AOA/ZOA Assistance Information signaling was discussed according to the LS from RAN1 [1], and the following agreements were agreed:

Define a new Assistance Information for UL-AoA IE with the Expected AoA/ZoA values and their Uncertainties using a CHOICE structure. Stage 3 details are FFS

Agree to introduce the UL-AoA assistance information IN NRPPA MEASUREMENT REQUEST message, within the TRP Measurement Request List IE

Agree to introduce the UL-AoA assistance information IN NRPPA MEASUREMENT UPDATE message, within a new TRP Measurement Update List IE (FFS)

Agree to introduce a Zenith Angle of Arrival IE to the TRP Measurement Result IE. Adding description for linear array FFS

No indication from gNB to LMF is needed for requesting the Expected UL AoA

An F1AP TP is needed. Maintain the BL CR up to date with respect to NRPPa progress.

However, there are some remaining issues that need further discussion, e.g, FFS on the presence of the uncertainties range. FFS if LMF can request the Zenith Angle of Arrival only, and some signaling details. In this contribution, we provide further analysis and proposals on the remaining issues for support of UL-AOA/ZOA Assistance Information.

2 Discussion

Issue 1：FFS on the presence of the uncertainties range

According to the discussion of RAN1, the expected AoA/ZoA and uncertainty ranges are indicated to help gNB to know the rough angle range that AoA/ZoA is expected to be searched, so that gNB can search the angle in this range to reduce the angle error.

However, for normal UL-AOA measurement report, one company thinks that gNB is also expected to provide uncertainty range values to further improve positioning accuracy [2]. In this regard, from RAN1#104 to RAN1#105, there is no conclusion related to the parameters that gNB reports to LMF. And if there is a need to introduce Uncertainty to the normal UL-AOA measurement, is there a similar requirement for other measurements? e.g, UL RTOA? Therefore, it seems difficult to evaluate from a RAN3 perspective whether it can bring accuracy improvement, and for which case would this gain be more meaningful?

We propose to send a LS to RAN1 to evaluate whether there will be performance gains.

Proposal 1：For normal UL-AOA measurement report, whether to introduce uncertainty range value should be evaluated by RAN1

Issue 2：FFS if LMF can request the Zenith Angle of Arrival only？

According to the conclusion for UL AoA positioning enhancement at RAN1##104bis-e, the following agreement was reached:

Agreement:

- The following option is supported to enhance signaling of UL-AOA measurement report in case of a linear array

- Option 2: The z-axis of LCS is defined along the linear array axis. gNB reports only the ZoA relative to z-axis in the LCS, and the LCS-to-GCS translation function is used to set up the specific z-axis direction

- UL-AOA signalling details for support of Option 2 are left up to RAN WG3

Based on above description, the UL AOA measurement reporting is determined by gNB implementation, that is, if a linear array is deployed, gNB only reports the Zenith Angle of Arrival. Otherwise, the gNB should report the Azimuth and Zenith Angle of Arrival as much as possible. Also, in case the UL-AoA assistance information is introduced, based on the implementation, the LMF can determine whether to send the Expected UL Angle of Arrival  or the Expected UL Angle of Arrival Linear Array. Therefore, the measurement request message from LMF still includes only one code point: UL-AoA, there is no need for LMF to request Zenith Angle of Arrival Only separately.

Proposal 2：No need for LMF to request Zenith Angle of Arrival Only.

Issue 3：Introduce the UL-AoA assistance information: within a new TRP Measurement Update List IE (FFS)

According to last RAN3 meeting, it was agreed that the MEASUREMENT UPDATE message in NRPPa is reused for updating the expected UL AoA/ZoA and uncertainty window.

When it comes to specific signaling design, considering that a UL AOA measurement may involve multiple TRPs, it is proposed to adopt the following table in the NRPPa specification [3]:

| TRP Measurement Update List |  | 0..1 |  |
| --- | --- | --- | --- |
| >TRP Measurement Update Item |  | 1..<maxnoofMeasTRPs> |  |
| >>TRP ID | M |  | 9.2.24 |
| >>Angle Window Information | O |  | 9.2.X |

This table makes the MEASUREMENT UPDATE message correspond to the MEASUREMENT REQUEST message.

Proposal 3: Introduce a new TRP Measurement Update List IE in MEASUREMENT UPDATE

Issue 4:  Whether to add description for linear array FFS

According to last RAN3 meeting, it was agreed to introduce UL Angle of Arrival Linear Array IE within the TRP Measurement Result IE, as shown below [3][4]:

### 9.2.a	UL Angle of Arrival linear array

This information element contains the uplink Angle of Arrival linear array measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Zenith Angle of Arrival | M |  | INTEGER(0..1799) | TS 38.133 [16] |
| LCS to GCS Translation |  | 1 |  |  |
| >Alpha | M |  | INTEGER (0..3599) |  |
| >Beta | M |  | INTEGER (0..3599) |  |
| >Gamma | M |  | INTEGER (0..3599) |  |

Therefore, the issue is a clarification for the above agreement, i.e, adding a description: “the z-axis of LCS is defined along the linear array axis” makes it easier to understand, if possible.

Proposal 4：Add a description: “the z-axis of LCS is defined along the linear array axis” into the explanation section of UL Angle of Arrival linear array  IE

Accordingly TPs for TS 38.455 is  attached in chapter 5.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1：For normal UL-AOA measurement report, whether to introduce uncertainty range value should be evaluated by RAN1

Proposal 2：No need for LMF to request Zenith Angle of Arrival Only.

Proposal 3: Introduce a new TRP Measurement Update List IE in MEASUREMENT UPDATE

Proposal 4：Add a description: “the z-axis of LCS is defined along the linear array axis” into the explanation section of UL Angle of Arrival linear array IE

4 Reference

- R1-2106202, LS on support of UL-AOA/ZOA assistance information signalling for NR positioning, Intel Corporation

- R3-213851, Discussion on Rel-17 UL AoA enhancements with TP to NRPPa BL CR, Ericsson

- R3-213673, Consideration of UL-AOA/ZOA Assistance Information, CATT

- R3-214309, CB: # 1901_Pos_AccEnhs_AoA-AoZ_LS

5.  Text Proposals for TS 38.455

-------------------------------- Begin of Changes -------------------------------

9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>Angle Window Information | O |  | 9.2.X |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min is not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

-------------------------------------Next Change -----------------------------------

9.1.4.5	MEASUREMENT UPDATE

This message is sent by the LMF to update a previously configured measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| TRP Measurement Update List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Update Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Angle Window Information | O |  | 9.2.X |  | YES | ignore |

-------------------------------------Next Change -----------------------------------

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  |
| >>UL RTOA | M |  | 9.2.39 |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  |
| >>UL Angle of Arrival Linear Array | M |  | 9.2.Y |  |
| >Time Stamp | M |  | 9.2.42 |  |
| >Measurement Quality | O |  | 9.2.43 |  |
| >Measurement Beam Information | O |  | 9.2.57 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

------------------------------------Next Change-----------------------------------

### 9.2.X	Angle Window Information

This information element contains angle window information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Expected AOA/ZOA Value | M |  |  |  |
| >Expected UL Angle of Arrival | M |  | 9.2.Z1 |  |
| >Expected UL Angle of Arrival Linear Array | M |  | 9.2.Z2 |  |

### 9.2.Y	  UL Angle of Arrival Linear Array

This information element contains the uplink Angle of Arrival measurement that linear array antenna is deployed, correspondingly the z-axis of LCS is defined along the linear array axis.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Zenith Angle of Arrival | M |  | INTEGER(0..1799) | As defined in TS 38.133 [16]. |
| LCS to GCS Translation |  | 0..1 |  | If absent, the azimuth and zenith are provided in GCS. |
| >Alpha | M |  | INTEGER (0..3599) |  |
| >Beta | M |  | INTEGER (0..3599) |  |
| >Gamma | M |  | INTEGER (0..3599) |  |

### 9.2.Z1  Expected UL Angle of Arrival

This information element contains the expected uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL Angle of Arrival | M |  | 9.2.38 |  |
| Azimuth AoA Uncertainty range | O |  | INTEGER(-1799..1799) |  |
| Zenith AoA Uncertainty range | O |  | INTEGER(-899..899) |  |

### 9.2.Z2  Expected UL Angle of Arrival Linear Array

This information element contains the expected uplink Angle of Arrival measurement that linear array antenna is deployed.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL Angle of Arrival Linear Array | M |  | 9.2.Y |  |
| Azimuth AoA Uncertainty range | O |  | INTEGER(-1799..1799) |  |
| Zenith AoA Uncertainty range | O |  | INTEGER(-899..899) |  |

-------------------------------------Next Change-----------------------------------

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for NRPPa

--

-- **************************************************************

-------------------------------------Next Change-----------------------------------

id-TRP-MeasurementRequestList,

id-TRP-MeasurementUpdateList,

id-TRP-MeasurementResponseList,

id-TRP-MeasurementReportList,

id-MeasurementBeamInfoRequest,

id-PositioningBroadcastCells,

id-SRSType,

id-ActivationTime,

id-SRSResourceSetID,

id-TRPList,

id-SRSSpatialRelation,

id-AbortTransmission,

id-SystemFrameNumber,

id-SlotNumber,

id-SRSResourceTrigger,

id-SFNInitialisationTime

-------------------------------------Next Change-----------------------------------

-- **************************************************************

--

##### -- Positioning Measurement Update

--

-- **************************************************************

PositioningMeasurementUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementUpdateIEs} },

...

}

PositioningMeasurementUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID			CRITICALITY reject	TYPE LMF-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID			CRITICALITY reject	TYPE RAN-MeasurementID			PRESENCE mandatory	}|

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-TRP-MeasurementUpdateList	CRITICALITY ignore	TYPE TRP-MeasurementUpdateList		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT UPDATE

--

-- **************************************************************

MeasurementUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementUpdate-IEs}},

...

}

MeasurementUpdate-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-SRSConfiguration		CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-TRP-MeasurementUpdateList	CRITICALITY ignore	TYPE TRP-MeasurementUpdateList	PRESENCE optional},

...

}

-------------------------------------Next Change-----------------------------------

9.3.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-SRSSpatialRelation,

id-ResultNR,

id-ResultEUTRA,

id-UL-AngleofArrival-LinearArray,

id-Angle-Window-Information,

#### -------------------------------------Next Change-----------------------------------

Angle-Window-Information ::= SEQUENCE {

expectedAOA-ZOA			ExpectedAOA-ZOA,

iE-Extensions					ProtocolExtensionContainer { { Angle-Window-Information-ExtIEs } } OPTIONAL

}

Angle-Window-Information-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### ----------------------------------Next Change----------------------------------

ExpectedAOA-ZOA ::= CHOICE {

expectedUL-AngleOfArrival		ExpectedUL-AoA,

expectedUL-AOA-LinearArray		ExpectedUL-AOA-LinearArray,

choice-extension		ProtocolIE-SingleContainer {{ ExpectedAOA-ZOA-ExtIEs }}

}

ExpectedAOA-ZOA-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ExpectedUL-AoA ::= SEQUENCE {

uL-AngleOfArrival			UL-AoA,

azimuthAoAUncertainty 		INTEGER (-1799..1799) 	OPTIONAL,

zenithAoAUncertainty 		INTEGER (-899..899) 	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { ExpectedUL-AoA-ExtIEs } } OPTIONAL

}

ExpectedUL-AoA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUL-AoA-LinearArray ::= SEQUENCE {

uL-AOA-LinearArray			UL-AOA-LinearArray,

azimuthAoAUncertainty 		INTEGER (-1799..1799) 	OPTIONAL,

zenithAoAUncertainty 		INTEGER (-899..899) 	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ExpectedUL-AoA-LinearArray-ExtIEs } } OPTIONAL

}

ExpectedUL-AoA-LinearArray-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### ----------------------------------Next Change----------------------------------

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival-EUTRA				INTEGER (0..719),

valueTimingAdvanceType1-EUTRA			INTEGER (0..7690),

valueTimingAdvanceType2-EUTRA			INTEGER (0..7690),

resultRSRP-EUTRA						ResultRSRP-EUTRA,

resultRSRQ-EUTRA						ResultRSRQ-EUTRA,

measuredResultsValue-Extension			ProtocolIE-Single-Container {{ MeasuredResultsValue-ExtensionIE }}

}

MeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-ResultSS-RSRP		CRITICALITY ignore	TYPE ResultSS-RSRP		PRESENCE mandatory	}|

{ ID id-ResultSS-RSRQ		CRITICALITY ignore	TYPE ResultSS-RSRQ		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRP		CRITICALITY ignore	TYPE ResultCSI-RSRP		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRQ		CRITICALITY ignore	TYPE ResultCSI-RSRQ		PRESENCE mandatory	}|

{ ID id-AngleOfArrivalNR	CRITICALITY ignore	TYPE UL-AoA				PRESENCE mandatory	}|

{ ID id-UL-AngleofArrival-LinearArray	CRITICALITY ignore TYPE UL-AOA-LinearArray		PRESENCE mandatory},

...

}

----------------------------------Next Change----------------------------------

TRP-MeasurementRequestItem ::= SEQUENCE {

tRP-ID							TRP-ID,

search-window-information		Search-window-information	OPTIONAL,

iE-extensions					ProtocolExtensionContainer { { TRP-MeasurementRequestItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementRequestItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-Cell-ID		CRITICALITY ignore EXTENSION CGI-NR		PRESENCE optional }|

{ ID id-Angle-Window-Information	CRITICALITY ignore EXTENSION Angle-Window-Information		PRESENCE optional },

...

}

TRP-MeasurementUpdateList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementUpdateItem

TRP-MeasurementUpdateItem ::= SEQUENCE {

tRPID							TRPID,

angle-Window-Information			Angle-Window-Information		OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { TRP-MeasurementUpdateItem-ExtIEs } } OPTIONAL

}

TRP-MeasurementUpdateItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

----------------------------------Next Change----------------------------------

UL-AOA-LinearArray	::=	SEQUENCE {

zenithAoA					INTEGER (0..1799),

lCS-to-GCS-TranslationAoA	LCS-to-GCS-TranslationAoA		OPTIONAL,

iE-extensions				ProtocolExtensionContainer { { UL-AOA-LinearArray-ExtIEs } }

}

UL-AOA-LinearArray-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

----------------------------------Next Change----------------------------------

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<< non changed texts omitted >>>>>>>>>>>>>>

.-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-LMF-UE-Measurement-ID										ProtocolIE-ID ::= 2

id-ReportCharacteristics										ProtocolIE-ID ::= 3

id-MeasurementPeriodicity										ProtocolIE-ID ::= 4

id-MeasurementQuantities										ProtocolIE-ID ::= 5

id-RAN-UE-Measurement-ID										ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult										ProtocolIE-ID ::= 7

id-OTDOACells													ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group									ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item									ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item									ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics						ProtocolIE-ID ::= 12

id-Cell-Portion-ID												ProtocolIE-ID ::= 14

id-TRP-MeasurementRequestList									ProtocolIE-ID ::= 41

id-TRP-MeasurementResponseList									ProtocolIE-ID ::= 42

id-TRP-MeasurementReportList									ProtocolIE-ID ::= 43

id-SRSType														ProtocolIE-ID ::= 44

id-ActivationTime												ProtocolIE-ID ::= 45

id-SRSResourceSetID												ProtocolIE-ID ::= 46

id-TRPList														ProtocolIE-ID ::= 47

id-SRSSpatialRelation											ProtocolIE-ID ::= 48

id-SystemFrameNumber											ProtocolIE-ID ::= 49

id-SlotNumber													ProtocolIE-ID ::= 50

id-SRSResourceTrigger											ProtocolIE-ID ::= 51

id-TRPMeasurementQuantities										ProtocolIE-ID ::= 52

id-AbortTransmission											ProtocolIE-ID ::= 53

id-SFNInitialisationTime										ProtocolIE-ID ::= 54

id-ResultNR														ProtocolIE-ID ::= 55

id-ResultEUTRA													ProtocolIE-ID ::= 56

id-TRPInformationTypeItem										ProtocolIE-ID ::= 57

id-CGI-NR														ProtocolIE-ID ::= 58

id-SFNInitialisationTime-NR										ProtocolIE-ID ::= 59

id-Cell-ID														ProtocolIE-ID ::= 60

id-SrsFrequency													ProtocolIE-ID ::= 61

id-UL-AngleofArrival-LinearArray								ProtocolIE-ID ::= xx

id-Angle-Window-Information										ProtocolIE-ID ::= yy

id-TRP-MeasurementUpdateList									ProtocolIE-ID ::= zz

END

-- ASN1STOP

---------------------------------- End of Changes --------------------------------