3GPP TSG-RAN WG3 #114-e							          R3-215620

1-11 Nov2021

Online

Agenda item:	22.2.2

Source:	ZTE

Title: 	(TP for TS 38.413) Management of multicast session

Document for:	Approval

- Introduction

This paper is based on the latest TS23.247 v100.

According to the SA2 progress, N2 messages can be divided into three parts, as below.

- SMF initiated N2 message, between SMF and NG-RAN, for multicast session joining/establishment, leaving/release, update QoS flow parameters except ARP.

- MB-SMF initiated N2 message, between MB-SMF and NG-RAN, for multicast session activation/deactivation and update ARP of QoS flow parameters

- NG-RAN initiated N2 message, between MB-SMF and NR-RAN, for multicast distribution activation an deactivation

- Discussion

  - Multicast session establishment/ release/update

According to a latest TS23.247, the UE associated PDU session modification shall be used for a UE to join or leave a multicast session.

7.2.1.3 Multicast session join and session establishment procedure <Skip unrelated part> 6. The N2 message, which includes the multicast session information and PDU session modification information is sent to the NG-RAN. If the MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise if the MBS is supported by NG-RAN, 5GC shared MBS traffic delivery is adopted. If the NG-RAN supports MBS, the NG-RAN uses the MBS Session ID to determine that the PDU Session Modification procedures corresponds to the indicated multicast session. If the multicast QoS information is received and the NG-RAN supports MBS, the associated unicast QoS flow information is not used to allocate the radio resource. 7.2.2.2 MBS session Leave <Skip unrelated part> 7. The SMF invokes the Nsmf_PDUSession_UpdateSMContext Response (PDU Session ID, N2 SM information (MBS Session ID), N1 SM container) service operation. In the N2 SM information, the SMF informs the NG-RAN to remove the UE from this MBS session if 5GC Shared MBS traffic delivery method is used. In the N2 SM information, the SMF also informs the NG-RAN to release the mapped unicast QoS Flow(s), which carry or intend to carry the Multicast MBS traffic for 5GC individual MBS traffic delivery. The mapped QoS Flow(s) are released as defined in TS 23.502 [6] clause 4.3.3.2. 8. The AMF send N2 message (N2 SM information, N1 SM container) to the NG-RAN

Proposal 1: The existing NGAP PDU Session Resource Modification procedure shall be enhanced to support multicast session establishment/release.

7.2.6 Multicast session update procedure Multicast session update procedure is invoked by the AF to update the ARP for an ongoing multicast session. For the interaction between AF and MB-SMF, see clause 7.1.1.5 and 7.1.1.6. Figure 7.2.6-1: Multicast session update procedure. Editor's note: Names of messages are FFS. 1. This procedure is triggered by the MB-SMF receiving the updated policy for MBS, see clauses 7.1.1.5 and 7.1.1.6. 2. The MB-SMF generates the QoS profile for the multicast, and sends MBS Session update Request (N2 SM message (TMGI, QoS profiles for multicast)) to the AMF(s). 3. The involved AMF forwards the N2 SM information received from MB-SMF to the RAN nodes via NGAP Session update Request (N2 SM message (TMGI, QoS profiles for multicast)) message. Editor's note: Whether it is the AMF or the MB-SMF that stores RAN ID for the interaction with RAN in step 3 needs to align with clause 7.2.1. 4. NG-RAN updates the MBS session context, and if only the ARP of QoS parameters is updated, NG-RAN node also updates the QoS parameters of the associating PDU Sessions. Editor's note: Updating the associated PDU Session's QoS based on the update of MBS Session QoS by NG-RAN requires RAN WGs collaboration. 5. The NG-RAN acknowledges NGAP Session update Request by sending an NGAP Session update Response message to the AMF. 6. The AMF sends MBS Session update Response to the MB-SMF. 7. MB-SMF sends Session update Request (TMGI, QoS profiles for multicast) to SMF. 8. SMF determines the UE list regarding the TMGI included in the message. If the QoS parameters other than ARP needs to be updated, SMF triggers PDU Session Modification procedure for each UE as defined in TS 23.502 [6].

According to the TS23.247, if the QoS parameters other than ARP needs to be updated, SMF triggers UE associated PDU Session Modification procedure for each UE as defined in TS 23.502. Otherwise, if only the ARP of QoS parameters is updated, MB-SMF triggers non-UE associated NGAP multicast session update procedure for all joined UEs.

Proposal 2: The existing NAGP PDU Session Resource Modification procedure shall be enhanced to support multicast session update in case ARP of QoS parameters is updated.

Proposal 3:  A new NAGP procedure (e.g., multicast session update) shall be introduced to support multicast session update in case the QoS parameters other than ARP needs to be updated, it is a non-UE associated NGAP procedure triggered by MB-SMF.

  - Multicast session activation/deactivation

MBS Session activation procedure is for multicast only. MBS Session activation procedure is triggered by MB-SMF, when it receives the notification from MB-UPF for the downlink MBS DL data, or when it receives the request directly from AF or via NEF. The MBS Session activation procedure is used for activating the resources for MBS data at NG-RAN node. The multicast session state transits from inactive to active after MBS Session activation procedure.

MBS Session deactivation procedure is for multicast only. MBS Session deactivation procedure is triggered by MB-SMF, when it receives the notification from MB-UPF in the case of no downlink data to be transmitted, or when it receives the request directly from AF or via NEF. The MBS Session deactivation procedure is used for deactivating the resources for MBS data at NG-RAN node. The multicast session state transits from active to inactive after MBS Session deactivation procedure.

7.2.5.2 MBS session activation procedure <Skip unrelated part> 11. MB-SMF sends Session Activation (TMGI) to the AMF. NOTE 2: The messages in step 10 and 11 is MBS-specific messages and it is possible that the AMF(s) in step 10 are not associate to any UEs involved in the MBS Session. 12. AMF sends NGAP activation message (TMGI) to the RAN nodes.

7.2.5.3 MBS session deactivation procedure <Skip unrelated part> 3. The MB-SMF sends MBS Session deactivation Request (TMGI) to the AMFs. 4. The AMF sends NGAP deactivation request message (TMGI) to the RAN nodes. Editor's note: Whether it is the AMF or the MB-SMF that stores RAN ID for the interaction with RAN in step 3 needs to align with clause 7.2.1. 5. NG-RAN sets the MBS session state with respect to TMGI to inactive. In this procedure, the NG-RAN will not release the MBS session context, and MB-N3 tunnel for the MBS session. Editor's note: Whether the NG-RAN removes the MBS Session Context or mark it as inactive requires RAN collaboration. 6. NG-RAN acknowledges the NGAP deactivation Response message. 7. The AMF invokes MBS Session deactivation Response to acknowledge the service for MB-SMF.

Proposal 4: A new NGAP procedure (e.g., multicast session update) shall be introduced for multicast session activation/deactivation, it is non-UE associated NGPA procedure triggered by MB-SMF.

  - Multicast distribution establish/release

Based on the TS23.247, it is the NG-RAN to trigger shared N3 tunnel establish and release.

7.2.1.4 Establishment of shared delivery toward RAN node 1. A RAN node discovers that it needs to establish shared delivery for an MBS session because it serves at least one UE within the MBS session. For location dependent services, the RAN node needs to establish shared delivery for the location dependent contents of an MBS session if it serves at least one UE assigned to an MBS session ID and area session ID. 2. The RAN node sends a multicast distribution request to the AMF and provides the TMGI as MBS session ID. If the RAN node is configured to use unicast transport for the shared delivery, it allocates a GTP tunnel endpoint and provides that endpoint. For location dependent services, the RAN node also provides the area session ID. 3. The AMF discovers the MB-SMF serving the multicast session using the NRF discovery service. It sends a Multicast distribution request to the MB-SMF, passing the parameters received in message 2.

7.2.2.4 Release of shared delivery toward RAN node 1. A RAN node discovers that it needs to release shared delivery for an MBS session, e.g. because it no longer serves at least one UE within the MBS session. For location dependent services, the RAN node may release shared delivery for the location dependent contents of an MBS session if it no longer serves at least one UE assigned to an MBS session ID and area session ID. 2. The RAN node sends a multicast distribution release request to the AMF and provides the TMGI as MBS session ID. For location dependent services, the RAN node also provides the area session ID. 3. The AMF sends a Multicast distribution release request to the MB-SMF, passing the parameters received in message 2. 4. If unicast transport was used towards the RAN node, the MB-SMF configures the MB-UPF to terminate sending multicast data for the multicast session (or location dependent content of the multicast session if an area session ID was received) towards that RAN node. 5. The MB-SMF removes the RAN node ID from storage in the context of the multicast session (or location dependent part of the multicast session if an area session ID was received).

Proposal 5: New NGAP procedures(e.g. multicast distribution establishment, multicast distribution release) shall be introduced for multicast tunnel establishment/release. It is non-UE associated NGAP procedure triggered by NG-RAN, between NG-RAN and MB-SMF.

  - Broadcast Session

Both RAN and SA agreed to support broadcast service. The following is abstracted from the latest TS23.247.

7.3.1 MBS Session Start for Broadcast Editor's note: The services and messages used in this procedure are FFS. 1. To establish broadcast session, the AF performs TMGI allocation and MBS session start as specified in clause 7.1.1.1. The AF needs to set the service type to be broadcast service. The NEF/MBSF may translate the broadcast area information to MBS service area, which includes cell IDs, TAI list, or RAN node IDs. 2. The MB-SMF may use NRF to discover the AMF(s) based on the MBS service area and select the appropriate one(s). Then the MB-SMF sends the MBS Session Resource Setup Request (TMGI, LL MC Address and source host address, 5G Authorized QoS Profile, MBS service area) messages to the selected AMF(s) in parallel if the service type is broadcast service. 3. The AMF transfers the MBS Session Resource Setup Request (TMGI, LL MC and source host address, 5G Authorized QoS Profile) message to all NG-RANs which support MBS in the MBS service area. The AMF may include the MBS service area. 4. NG-RAN creates an MBS Broadcast Session Context, stores the TMGI, the QoS Profile in the MBS Session Context. The LL MC Address and Source Host Address are optional parameters and only provided by MB-SMF to NG-RAN if N3mb multicast transport is configured to be used in the 5GC. 5. If NG-RAN prefers to use N3mb multicast transport (and if LL MC Address is available in NG-RAN), the NG-RAN joins the multicast group (i.e. LL MC Address). If NG-RAN prefers to use N3mb point-to-point transport (or if the LL MC Address is not available in NG-RAN) between the NG-RAN and MB-UPF, NG-RAN provides its N3mb DL Tunnel Info. 6. The NG-RAN reports successful establishment of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Setup Response (TMGI, N3mb DL Tunnel Info) message(s) to the AMF. N3mb DL Tunnel Info is only available when point-to-point transport applies between MB-UPF and NG-RAN. Editor´s note: How to set up shared delivery potentially using commonalities with multicast signalling is FFS 7. The AMF transfers the MBS Session Start Response (TMGI, N3mb DL Tunnel Info) to the MB-SMF. The AMF should respond success when it receives the first success response form the NG-RAN(s). And if all NG-RAN(s) report failure, the AMF should respond failure. The MB-SMF store the AMF(s) which respond success. 8. If N3mb point-to-point transport is to be used (i.e. N3mb DL Tunnel Info is present in the MBS Session Start Response message from AMF), the MB-SMF sends a N4 Session Modification Request to the MB-UPF to allocate the N3mb point-to-point transport tunnel for a replicated MBS stream for the MBS Session. Otherwise, step 8 can be skipped. 9. NG-RAN advertises the TMGI representing the MBS service over radio interface. Step 9 can take place in parallel with step 6. 10. The AF starts transmitting the DL media stream to MB-UPF using the N6mb Tunnel, or optionally un-tunnelled i.e. as an IP multicast stream using the HL MC address. 11. The MB-UPF transmits the media stream to NG-RAN via N3mb multicast transport or point-to-point transport. 12. The NG-RAN transmits the received DL media stream using DL PTM resources.

7.3.2 MBS Session Release for Broadcast 1. The AF/AS may stop the media stream before sending the MBS Session Release Request (TMGI) message to the 3GPP network. 2. The AF/AS performs MBS Session Stop procedure to request release of MBS Session (step 1 ~10 in the figure 7.1.1.2-1). 3. MB-SMF sends an MBS Session Stop Request (TMGI) message to the AMF(s) that has been involved in the MBS Session. 4. The AMF sends an MBS Session Resource Release Request (TMGI) message to all RAN nodes that have been involved. If a NG-RAN node receives multiple MBS Session Resource Release Request messages for the same TMGI (e.g. from several AMFs the NG-RAN is connected to), NG-RAN only performs step 5 and step 6 once. 5. The NG-RAN stops the PTM transmission. 6. If N3mb multicast transport has been used, the NG-RAN sends a Leave message (LL MC Address) to stop the media stream to this NG-RAN node. If N3mb point-to-point transport has been used, the NG-RAN release its N3mb Tunnel Info. NG-RAN deletes its MBS Session Context. 7. The NG-RAN reports successful release of resources for the MBS Session by sending MBS Session Resource Release Response (TMGI) message(s) to the AMF(s). 8. The AMF sends MBS Session Stop Response (TMGI) to the MB-SMF. 9. The AF may start a TMGI de-allocation procedure (step 11~14 in the figure 7.1.1.2-1).

7.3.3 MBS Session Update for Broadcast 1. The AF starts MBS session update procedure by sending Modify MBS Session Request to the NEF/MBSF with TMGI (steps 1~7 in the figure 7.1.1-3). The AF may adjust service requirement and/or broadcast area. The service requirements adjustment may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s). Editor´s note: There are dependence on S2-2104190/S2-2104309 which introduce the MBS session update procedure in clause 7.1.1 2. The MB-SMF sends MBS Session Resource Update to the AMFs with TMGI, the updated 5G QoS Profile and the updated MBS service area. If the broadcast area is updated, the MB-SMF may use NRF to discover the AMF(s) based on the new broadcast area and select the appropriate one(s). Depending on the change of the MBS service area, the MB-SMF may send MBS Session Start to some AMFs in the new MBS service area and MBS Session Stop to some other AMFs in the old MBS service area. 3. The AMF sends MBS Session Resource Update to NG-RANs with TMGI, the updated 5G QoS Profile and the updated MBS service area. Depending on the change of the MBS service area, the AMF may send MBS Session Resource Setup to some NG-RANs in new MBS service area and MBS Session Resource Release to some other NG-RANs in old MBS service area. 4. The NG-RAN updates MBS Session Context. 5. The NG-RAN sends MBS Session Resource Update Response to the AMF. 6. The AMF sends MBS Session Resource Update Response to the MB-SMF. 7. The NG-RAN updates the MBS Session. It takes place in parallel with step 5 to step 6.

Proposal 6: New NGAP procedures (e.g., Broadcast Session Session Resource Setup, Broadcast Session Resource Release and Broadcast Session Update) shall be introduced for Broadcast Session setup/release/update, they are non-UE associated NGPA procedure triggered by MB-SMF.

  - MBS support information

Based on the discussion in previous RAN3 meeting, companies have discussed to add the “MBS Support information” which is encoded as an enumeration with one value in the following SMF containers:

PDU Session Resource Setup Response Transfer,

PDU Session Resource Modify Response Transfer,

Path Switch Request Transfer,

Handover Request Acknowledge Transfer.

Proposal 7: RAN3 shall add the “MBS support information” which is encoded as an enumeration with one value into the following messages:

PDU Session Resource Setup Response Transfer,

PDU Session Resource Modify Response Transfer,

Path Switch Request Transfer,

Handover Request Acknowledge Transfer.

- Conclusion

We provide the following proposals and TP for TS38.413

Proposal 1: The existing NGAP PDU Session Resource Modification procedure shall be enhanced to support multicast session establishment/release.

Proposal 2: The existing NAGP PDU Session Resource Modification procedure shall be enhanced to support multicast session update in case ARP of QoS parameters is updated.

Proposal 3:  A new NAGP procedure (e.g., multicast session update) shall be introduced to support multicast session update in case the QoS parameters other than ARP needs to be updated, it is a non-UE associated NGAP procedure triggered by MB-SMF.

Proposal 4: A new NGAP procedure (e.g., multicast session update) shall be introduced for multicast session activation/deactivation, it is non-UE associated NGPA procedure triggered by MB-SMF.

Proposal 5: New NGAP procedures(e.g. multicast distribution establishment, multicast distribution release) shall be introduced for multicast tunnel establishment/release. It is non-UE associated NGAP procedure triggered by NG-RAN, between NG-RAN and MB-SMF.

Proposal 6: New NGAP procedures (e.g., Broadcast Session Session Resource Setup, Broadcast Session Resource Release and Broadcast Session Update) shall be introduced for Broadcast Session setup/release/update, they are non-UE associated NGPA procedure triggered by MB-SMF.

Proposal 7: RAN3 shall add the “MBS support information” which is encoded as an enumeration with one value into the following messages:

PDU Session Resource Setup Response Transfer,

PDU Session Resource Modify Response Transfer,

Path Switch Request Transfer,

Handover Request Acknowledge Transfer.

- Text proposal for TS38.413

# 8	NGAP Procedures

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Multicast Distribution Establishment | MULTICAST DISTRIBUTION ESTABLISHMENT REQUEST | MULTICAST DISTRIBUTION ESTABLISHMENT RESPONSE | MULTICAST DISTRIBUTION ESTABLISHMENT FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE REQUEST | MULTICAST DISTRIBUTION RELEASE RESPONSE | MULTICAST DISTRIBUTION RELEASE FAILURE |
| Broadcast Session Start | BROADCAST SESSION START REQUEST | BROADCAST SESSION START RESPONSE | BROADCAST SESSION START FAILURE |
| Broadcast Session Stop | BROADCAST SESSION STOP REQUEST | BROADCAST SESSION STOP RESPONSE |  |
| Broadcast Session Update | BROADCAST SESSION UPDATE REQUEST | BROADCAST SESSION UPDATE RESPONSE | BROADCAST SESSION UPDATE FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |

## 8.2	PDU Session Management Procedures

8.2.1	PDU Session Resource Setup

8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling.

8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

<Skip unrelated part>

If the NG-RAN node supports MBS, it shall add the MBS Support Indicator IE into the PDU Session Resource Setup Response Transfer IE.

<Skip unrelated part>

8.2.3	PDU Session Resource Modify

8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE MODIFY REQUEST message to the NG-RAN node.

<Skip unrelated part>

For each PDU session, if the Redundant QoS Flow Indicator IE is included and set to “false” for all QoS flows, the NG-RAN node shall, if supported, stop the redundant transmission and release the redundant tunnel for the concerned PDU session as specified in TS 23.501 [9].

For each PDU session, if the MBS Session Association IE is set to “Add”, the NG-RAN node shall, if supported, store the multicast context information if not yet existing and bind the PDU session with this MBS context. For each included MBS QoS flow it shall, if supported, set up the resources if not yet setup while not setting up the unicast resources for the indicated associated unicast QoS flow.

For each PDU session, if the MBS Session Association IE is set to “Update”, the NG-RAN node shall, if supported, update the multicast QoS parameters other than ARP.

For each PDU session, if the MBS Session Association IE is set to “Release”, the NG-RAN node shall, if supported, remove the binding of the PDU session with this indicated MBS session. It shall also remove the MBS context and release the corresponding resources if there is no remaining binding.

<Skip unrelated part>

If the NG-RAN node supports MBS, it shall add the MBS Support Indicator IE into the PDU Session Resource Setup Response Transfer IE.

<Skip unrelated part>

8.4	UE Mobility Management Procedures

<Skip unrelated part>

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

<Skip unrelated part>

If the NG-RAN node supports MBS, it shall add the MBS Support Indicator IE into the Handover Request Acknowledge Transfer IE.

<Skip unrelated part>

8.4.4	Path Switch Request

8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

<Skip unrelated part>

If the NG-RAN node supports MBS, it shall add the MBS Support Indicator IE into the Path Switch Request Transfer IE.

<Skip unrelated part>

## 8.X	Multicast Session Management Procedures

### 8.X.1	Multicast Session Update

#### 8.X.1.1	General

The purpose of the Multicast session update procedure is to active, or deactivate, or update APR of QoS flow parameters for multicast session.

The procedure uses MBS service associated signaling.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2-1: Multicast session update: successful operation

The AMF initiates the procedure by sending an MULTICAST SESSION UPDATE message to the NG-RAN node.

The NG-RAN node responds with an MULTICAST SESSION UPDATE RESPONSE message to acknowledge the request and provide additional information for the update.

For each multicast session, if the Multicast Session Update IE is set to “Update”, the NG-RAN node shall update Multicast context for the indicated multicast session ID.

For each multicast session, if the Multicast Session Update IE is set to “Activate”, the NG-RAN node shall activate the multicast session for the indicated multicast session ID.

For each multicast session, if the Multicast Session Update IE is set to “Deactivate”, the NG-RAN node shall deactivate the multicast session for the indicated multicast session ID.

#### 8.X.1.3	Unsuccessful Operation

Figure 8.7.2.3-1: Multicast session update: unsuccessful operation

If the NG-RAN cannot accept the update, it shall respond with an MULTICAST SESSION UPDATE FAILURE message and appropriate cause value.

#### 8.X.1.4	Abnormal Conditions

Void.

### 8.X.2	Multicast Distribution Establishment

#### 8.X.2.1	General

The purpose of the Multicast distribution Establishment procedure is to establish one or more shared N3 tunnels for multicast user date distribution from MB-UPF to NG-RAN.

The procedure uses MBS service associated signaling.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Multicast distribution establishment: successful operation

The NG-RAN node initiates the procedure by sending an MULTICAST DISTRIBUTION ESTABLISHMENT REQUEST message to the AMF. The AMF responds with an MULTICAST DISTRIBUTION ESTABLISHMENT RESPONSE message to acknowledge the request and provide additional information.

If the Shared N3 Tunnel Establish Item IE is contained in the MULTICAST DISTRIBUTION ESTABLISHMENT REQUEST message, the AMF shall configure the shared N3 tunnel parameters to the NG-RAN node.

#### 8.X.2.3	Unsuccessful Operation

Figure 8.X.2.3-1: Multicast distribution establishment: unsuccessful operation

If the AMF cannot accept the update, it shall respond with an MULTICAST DISTRIBUTION ESTABLISHMENT FAILURE message and appropriate cause value.

#### 8.X.2.4	Abnormal Conditions

Void.

### 8.X.3	Multicast Distribution Release

#### 8.X.3.1	General

The purpose of the Multicast distribution release procedure is to release one or more shared N3 tunnels for multicast user date distribution from MB-UPF to NG-RAN.

The procedure uses MBS service associated signaling.

#### 8.X.3.2	Successful Operation

Figure 8.X.2.2-1: Multicast distribution release: successful operation

The NG-RAN node initiates the procedure by sending an MULTICAST DISTRIBUTION RELEASE REQUEST message to the AMF. The AMF responds with an MULTICAST DISTRIBUTION RELEASE RESPONSE message to acknowledge the request and provide additional information for the release procedure.

If the Shared N3 Tunnel Release Item IE is contained in the MULTICAST DISTRIBUTION RELEASE REQUEST message, the AMF shall release the shared N3 tunnel accordingly.

#### 8.X.3.3	Unsuccessful Operation

Figure 8.X.2.3-1: Multicast distribution update: unsuccessful operation

If the AMF cannot accept the update, it shall respond with an MULTICAST DISTRIBUTION RELEASE FAILURE message and appropriate cause value.

#### 8.X.3.4	Abnormal Conditions

Void.

## 8.Z	Broadcast Session Management Procedures

### 8.Z.1	Broadcast Session Resource Setup

8.Z.1.1	General

The purpose of the Broadcast Session Resource Setup procedure is to request the NG-RAN node to determine whether an upcoming broadcast Session for a given broadcast service can be accommodated by the NG-RAN node within a broadcast service area.

The procedure uses MBS service associated signaling.

8.Z.1.2	Successful Operation

Figure 8.Z.1.2-1. Broadcast session resource setup procedure. Successful operation.

The AMF initiates the procedure by sending an BROADCAST SESSION RESOURCE SETUP REQUEST message to the NG-RAN node. The NG-RAN node responds with an BROADCAST SESSION START RESPONSE message to acknowledge the request.

If the NG-U UP TNL Information IE is included in the BROADCAST SESSION RESOURCE SETUP REQUEST message, the SMF shall consider that the included information as the downlink termination point for the broadcast session.

8.Z.1.3	Unsuccessful Operation

Figure 8.Z.1.3-1. Broadcast session resource setup procedure. Unsuccessful operation.

If the NG-RAN node is not capable of correctly processing the request (e.g. the Broadcast resources could not be established at all in any cell), the NG-RAN node shall send BROADCAST SESSION START FAILURE message.

8.Z.1.4	Abnormal Conditions

Void.

### 8.Z.2	Broadcast Session Resource Release

8.Z.2.1	General

The purpose of the Broadcast Session Resource Release procedure is to inform the NG-RAN node about the release of a Broadcast Session for a given Broadcast Service, and that the NG-RAN node releases the allocated resources and that the associated logical shared N3 connection is also released. The Broadcast Session Stop procedure is triggered by the AMF.

The procedure uses MBS service associated signaling.

8.Z.2.2	Successful Operation

Figure 8.Z.2.2-1. Broadcast Session Resource Release procedure. Successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION RESOURCE RELEASE COMMAND message to the NG-RAN node. Upon receipt of the BROADCAST SESSION RESOURCE RELEASE COMMAND message, the NG-RAN node shall send the BROADCAST SESSION RESOURCE RELEASE RESPONSE message after the NG-RAN node releases the affected resources and removes the Broadcast session context.

8.Z.2.3	Unsuccessful Operation

Void.

8.Z..4	Abnormal Conditions

Void.

### 8.Z.3	Broadcast Session Update

8.Z.3.1	General

The purpose of the Broadcast Session Update procedure is to inform the NG-RAN node about changing characteristics of the Broadcast session.

The procedure uses MBS service associated signaling.

8.Z.3.2	Successful Operation

Figure 8.Z.3.2-1. MBS Session Start procedure. Successful operation.

The AMF initiates the procedure by sending an BROADCAST SESSION UPDATE REQUEST message to the NG-RAN node. The NG-RAN node responds with an BROADCAST SESSION UPDATE RESPONSE message to acknowledge the request.

8.Z.3.3	Unsuccessful Operation

Figure 8.Z.3.3-1. Broadcast Session Update procedure. Unsuccessful operation.

If the NG-RAN node is not capable of correctly processing the request, the NG-RAN node shall send BROADCAST SESSION UPDATE FAILURE message.

8.Z.3.4	Abnormal Conditions

Void.

# 9	Elements for NGAP Communication

## 9.0	General

<Skip unrelated part>

## 9.2	Message Functional Definition and Content

### 9.2.1	PDU Session Management Messages

<Skip unrelated part>

### 9.2.XX	Multicast session Management Messages

#### 9.2.XX.1	MULTICAST SESSION UPDATE REQUEST

This message is sent by the AMF and is used to request the NG-RAN node to update APR of QoS flow parameters for one or several Multicast session resources.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Multicast Session ID | M |  | FFS |  | - |  |
| Multicast Session Update Request Transfer | M |  | OCTET STRING | Containing the Multicast Session Update Request Transfer IE specified in subclause 9.3.X.Y1. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMultiastSessions | Maximum no. of Multicast sessions allowed towards one UE. Value is FFS. |

#### 9.2.XX.2	MULTICAST SESSION UPDATE RESPONSE

This message is sent by the NG-RAN node as a response to the request to update resources on Uu and NG-U for one Multicast session resources.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Multicast Session ID | M |  | FFS |  | - |  |
| Multicast Session Update Response Transfer | M |  | OCTET STRING | Containing the Multicast Session Update Response Transfer IE specified in subclause 9.3.X.Y2. | - |  |

#### 9.2.XX.3	MULTICAST SESSION UPDATE FAILURE

This message is sent by the NG-RAN node as a failure to the request to update resources on Uu and NG-U for one Multicast session resources.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Multicast Session ID | M |  | FFS |  | - |  |
| Multicast Session Update Unsuccessful Transfer | M |  | OCTET STRING | Containing the Multicast Session Update Unsuccessful Transfer IE specified in subclause 9.3.X.Y3. | - |  |

9.2.XX.4	MULTICAST DISTRIBUTION ESTABLISHMENT REQUEST

This message is sent by the NG-RAN and is used to request to establish the shared N3 tunnel between NG-RAN and MB-UPF

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Multicast Session ID | M |  | FFS |  | - |  |
| Multicast Distribution Establishment Request Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Establishement Request Transfer IE specified in subclause 9.3.X.Y4 |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMultiastSessions | Maximum no. of Multicast sessions allowed towards one UE. Value is FFS. |

#### 9.2.XX.5	MULTICAST DISTRIBUTION ESTABLISHMENT RESPONSE

This message is sent by the NG-RAN node as a response to the request to establish shared N3 tunnel for one or several Multicast session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Multicast Session ID | M |  | FFS |  | - |  |
| Multicast Distribution Establishment Response Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Establishment Response Transfer IE specified in subclause 9.3.X.Y5. | - |  |

#### 9.2.XX.6	MULTICAST DISTRIBUTION ESTABLISHMENT FAILURE

This message is sent by the NG-RAN node as a failure to the request to establish shared N3 tunnel for one or several Multicast session resources.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Multicast Distribution Establishement Unsuccessful Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Establishment Unsuccessful Transfer IE specified in subclause 9.3.X.Y6. | - |  |

#### 9.2.XX.7	MULTICAST DISTRIBUTION RELEASE REQUEST

This message is sent by the NG-RAN and is used to request to release the shared N3 tunnel between NG-RAN and MB-UPF

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Multicast Session ID | M |  | FFS |  | - |  |
| Multicast Distribution Release Request Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Request Transfer IE specified in subclause 9.3.X.Y7 |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMultiastSessions | Maximum no. of Multicast sessions allowed towards one UE. Value is FFS. |

#### 9.2.XX.8	MULTICAST DISTRIBUTION RELEASE RESPONSE

This message is sent by the NG-RAN node as a response to the request to release shared N3 tunnel for one or several Multicast session resources.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Multicast Session ID | M |  | FFS |  | - |  |
| Multicast Distribution Release Response Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Response Transfer IE specified in subclause 9.3.X.Y8. | - |  |

#### 9.2.XX.9	MULTICAST DISTRIBUTION RELEASE FAILURE

This message is sent by the NG-RAN node as a failure to the request to release shared N3 tunnel for one or several Multicast session resources.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Multicast Distribution Release Unsuccessful Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Unsuccessful Transfer IE specified in subclause 9.3.X.Y9. | - |  |

#### 9.2.XX.7	MBS PAGING

This message is sent by the AMF and is used to page all UEs joining Multicast session in one or several tracking areas.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Multicast Session ID | M |  | FFS |  | YES | ignore |
| Paging DRX | O |  | 9.3.1.90 |  | YES | ignore |
| TAI List for Paging |  | 1 |  |  | YES | ignore |
| >TAI List for Paging Item |  | 1..<maxnoofTAIforPaging> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Paging Priority | O |  | 9.3.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Paging Origin | O |  | 9.3.3.22 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.3.1.69 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is 16. |

### 9.2.YY	Broadcast session Management Messages

#### 9.2.YY.1	BROADCAST SESSION RESOURCE SETUP REQUEST

This message is sent by the AMF and is used to request the NG-RAN node to allocate resources on Uu and NG-U for one Broadcast session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Broadcast Session ID | M |  | FFS |  | YES | reject |
| Service Area | M |  | FFS |  |  |  |
| Broadcast Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the Broadcast Session Resource Setup Request Transfer IE specified in subclause 9.3.Y.1. | YES | reject |

#### 9.2.YY.2	BROADCAST SESSION RESOURCE SETUP RESPONSE

This message is sent by the NG-RAN node as a response to the request to allocate resources on Uu and NG-U for one Broacast session resources.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Broadcast Session ID |  |  | FFS |  |  |  |
| Broadcast Session Resource Setup Response Transfer | M |  | OCTET STRING | Containing the Broadcast Session Resource Setup Response Transfer IE specified in subclause 9.3.Y.2. | YES | reject |

#### 9.2.YY.3	BROADCAST SESSION RESOURCE SETUP FAILURE

This message is sent by the NG-RAN node to indicate that the setup of the broadcast session context was unsuccessful.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Broadcast Session ID |  |  | FFS |  |  |  |
| Broadcast Session Resource Setup Failure Transfer | M |  | OCTET STRING | Containing the Broadcast Session Resource Setup Failure Transfer IE specified in subclause 9.3.Y.3. | YES | reject |

#### 9.2.YY.4	BROADCAST SESSION RESOURCE RELEASE COMMAND

This message is sent by the AMF and is used to request the NG-RAN node to release resources on Uu and NG-U for one Broadcast session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Broadcast Session ID | M |  | FFS |  | YES | reject |
| Broadcast Session Resource Release Command Transfer | M |  | OCTET STRING | Containing the Broadcast Session Resource Release Command Transfer IE specified in subclause 9.3.Y.4. | YES | reject |

#### 9.2.YY.5	BROADCAST SESSION RESOURCE RELEASE RESPONSE

This message is sent by the NG-RAN node as a response to the request to allocate resources on Uu and NG-U for one Broacast session resources.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Broadcast Session ID |  |  | FFS |  |  |  |
| Broadcast Session Release Response Transfer | M |  | OCTET STRING | Containing the Broadcast Session Release Response Transfer IE specified in subclause 9.3.Y.5. | YES | reject |

#### 9.2.YY.6	BROADCAST SESSION UPDATE REQUEST

This message is sent by the AMF and is used to request the NG-RAN node to update resources on Uu and NG-U for one Broadcast session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Broadcast Session ID | M |  | FFS |  | YES | reject |
| Service Area | O |  | FFS |  |  |  |
| Broadcast Session Update Request Transfer | M |  | OCTET STRING | Containing the Broadcast Session Update Request Transfer IE specified in subclause 9.3.Y.6. | YES | reject |

#### 9.2.YY.7	BROADCAST SESSION UPDATE RESPONSE

This message is sent by the NG-RAN node as a response to the request to update resources on Uu and NG-U for one Broadcast session resources.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Broadcast Session ID |  |  | FFS |  |  |  |
| Broadcast Session Update Response Transfer | M |  | OCTET STRING | Containing the Broadcast Session Update Response Transfer IE specified in subclause 9.3.Y.7 | YES | reject |

#### 9.2.YY.8	BROADCAST SESSION UPDATE FAILURE

This message is sent by the NG-RAN node to indicate that the update of the broadcast session context was unsuccessful.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Broadcast Session ID |  |  | FFS |  |  |  |
| Broadcast Session Update Failure Transfer | M |  | OCTET STRING | Containing the Broadcast Session Update Failure Transfer IE specified in subclause 9.3.Y.8. | YES | reject |

9.3	Information Element Definitions

<Skip unrelated part>

### 9.3.1	Radio Network Layer Related IEs

<Skip unrelated part>

#### 9.3.1.x	Multicast session management

This IE contains the Multicast session management for a list of multicast session. Each multicast session can be added, or updated, or released by SMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID |  |  | FFS |  |
| > CHOICE multicast session management |  |  |  |  |
| >>Add |  |  |  |  |
| >>>Multicast context |  |  | 9.3.1.y |  |
| >>Update |  |  |  |  |
| >>>Multicast context |  |  | 9.3.1.y |  |
| >>Release |  |  |  |  |

#### 9.3.1.y	Multicast Context

This IE contains the MBS context information for a multicast session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS QoS Flow List |  | 1 |  |  |
| >MBS QoS Flow Item |  | 1..<maxnoofMBSQoSFlows> |  |  |
| >>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  |
| >>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.12 |  |
| >>Associated unicast QoS Flow Identifier | O |  | QoS Flow Identifier 9.3.1.51 | If this IE is present, the associated unicast resource of this QoS Flow Identifier defined in QoS Flow Add or Modify Request Item IE shall not be configured. |
| Service Area |  |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSQoSFlows | Maximum no. of MBS QoS Flows in the MBS Sessions. Value is 16. |

### 9.3.4	SMF Related IEs

<Skip unrelated part>

9.3.4.2	PDU Session Resource Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL QoS Flow per TNL Information | M |  | QoS Flow per TNL Information 9.3.2.8 | NG-RAN node endpoint of the NG-U transport bearer for delivery of DL PDUs, together with associated QoS flows. | - |  |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows and corresponding to the Additional UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | - |  |
| Security Result | O |  | 9.3.1.59 |  | - |  |
| QoS Flow Failed to Setup List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information 9.3.2.8 | NG-RAN node endpoint of the NG-U transport bearer(s) for delivery of DL PDUs of the indicated Redundant QoS Flow(s) and corresponding to the Redundant UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows and corresponding to the Additional Redundant UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | YES | ignore |
| Used RSN Information | O |  | Redundant PDU Session Information 9.3.1.136 |  | YES | ignore |
| Global RAN Node ID of Secondary NG-RAN Node | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| MBS Support Indicator | O |  | ENUMERATED (TRUE) |  | YES | ignore |

#### 9.3.4.3	PDU Session Resource Modify Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  |  | 9.3.1.102 |  |  |  | YES |  | reject |  |
| UL NG-U UP TNL Modify List |  | 0..1 |  |  |  |  |  | YES |  | reject |  |
| >UL NG-U UP TNL Modify Item |  | 1..<maxnoofMultiConnectivity> |  |  |  | This IE(s) are included only for modification of an existing tunnel. |  | - |  |  |  |
| >>UL NG-U UP TNL Information | M |  |  | UP Transport Layer Information 9.3.2.2 |  | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. |  | - |  |  |  |
| >>DL NG-U UP TNL Information | M |  |  | UP Transport Layer Information 9.3.2.2 |  | Identifies the NG-U transport bearer at the NG-RAN node. |  | - |  |  |  |
| >>Redundant UL NG-U UP TNL Information | O |  |  | UP Transport Layer Information 9.3.2.2 |  | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission. |  | YES |  | ignore |  |
| >>Redundant DL NG-U UP TNL Information | O |  |  | UP Transport Layer Information 9.3.2.2 |  | Identifies the NG-U transport bearer at the NG-RAN node for the redundant transmission. |  | YES |  | ignore |  |
| Network Instance | O |  |  | 9.3.1.113 |  | This IE is ignored if the Common Network Instance IE is included. |  | YES |  | reject |  |
| QoS Flow Add or Modify Request List |  | 0..1 |  |  |  |  |  | YES |  | reject |  |
| >QoS Flow Add or Modify Request Item |  | 1..<maxnoofQoSFlows> |  |  |  |  |  | - |  |  |  |
| >>QoS Flow Identifier | M |  |  | 9.3.1.51 |  |  |  | - |  |  |  |
| >>QoS Flow Level QoS Parameters | O |  |  | 9.3.1.12 |  |  |  | - |  |  |  |
| >>E-RAB ID | O |  |  | 9.3.2.3 |  |  |  | - |  |  |  |
| >>TSC Traffic Characteristics | O |  |  | 9.3.1.130 |  | This IE may be present in case of GBR QoS flows and is ignored otherwise. |  | YES |  | ignore |  |
| >>Redundant QoS Flow Indicator | O |  |  | 9.3.1.134 |  | This IE indicates whether this QoS flow is requested for the redundant transmission. |  | YES |  | ignore |  |
| QoS Flow to Release List | O |  |  | QoS Flow List with Cause 9.3.1.13 |  |  |  | YES |  | reject |  |
| Additional UL NG-U UP TNL Information | O |  |  | UP Transport Layer Information List 9.3.2.12 |  | UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of UL PDUs for split PDU session. |  | YES |  | reject |  |
| Common Network Instance | O |  |  | 9.3.1.120 |  |  |  | YES |  | ignore |  |
| Additional Redundant UL NG-U UP TNL Information | O |  |  | UP Transport Layer Information List 9.3.2.12 |  | UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of redundant UL PDUs for split PDU session. |  | YES |  | ignore |  |
| Redundant Common Network Instance | O |  |  | Common Network Instance 9.3.1.120 |  |  |  | YES |  | ignore |  |
| Redundant UL NG-U UP TNL Information | O |  |  | UP Transport Layer Information 9.3.2.2 |  | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission of the Redundant QoS Flow(s). |  | YES |  | ignore |  |
| Multicast session management | O |  |  | 9.3.1.x |  |  |  | YES |  | ignore |  |
| MBS Support Indicator | O |  | ENUMERATED (TRUE) |  |  |  | YES |  | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMultiConnectivity | Maximum no. of connectivity allowed for a UE. Value is 4. The current version of the specification supports up to 2 connectivity. |

<Skip unrelated part>

9.3.4.8	Path Switch Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | M |  |  | UP Transport Layer Information 9.3.2.2 |  | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. |  | - |  |  |  |
| DL NG-U TNL Information Reused | O |  |  | ENUMERATED (true, …) |  | Indicates that DL NG-U TNL Information has been reused. |  | - |  |  |  |
| User Plane Security Information | O |  |  | 9.3.1.60 |  |  |  | - |  |  |  |
| QoS Flow Accepted List |  | 1 |  |  |  | QoS flows associated with the DL NG-U UP TNL Information IE. |  | - |  |  |  |
| >QoS Flow Accepted Item |  | 1..<maxnoofQoSFlows> |  |  |  |  |  | - |  |  |  |
| >>QoS Flow Identifier | M |  |  | 9.3.1.51 |  |  |  | - |  |  |  |
| >>Current QoS Parameters Set Index | O |  |  | Alternative QoS Parameters Set Index 9.3.1.152 |  | Index to the currently fulfilled alternative QoS parameters set. |  | YES |  | ignore |  |
| Additional DL QoS Flow per TNL Information | O |  |  | QoS Flow per TNL Information List 9.3.2.1 |  | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows. |  | YES |  | ignore |  |
| Redundant DL NG-U UP TNL Information | O |  |  | UP Transport Layer Information 9.3.2.2 |  | NG-RAN node endpoint of the NG-U transport bearer, for delivery of redundant DL PDUs. |  | YES |  | ignore |  |
| Redundant DL NG-U TNL Information Reused | O |  |  | ENUMERATED (true, …) |  | Indicates that Redundant DL NG-U TNL Information has been reused. |  | YES |  | ignore |  |
| Additional Redundant DL QoS Flow per TNL Information | O |  |  | QoS Flow per TNL Information List 9.3.2.1 |  | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of Redundant DL PDUs for split PDU session, together with associated QoS flows. |  | YES |  | ignore |  |
| Used RSN Information | O |  |  | Redundant PDU Session Information 9.3.1.136 |  |  |  | YES |  | ignore |  |
| Global RAN Node ID of Secondary NG-RAN Node | O |  |  | Global RAN Node ID 9.3.1.5 |  |  |  | YES |  | ignore |  |
| MBS Support Indicator | O |  | ENUMERATED (TRUE) |  |  |  | YES |  | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

<Skip unrelated part>

9.3.4.11	Handover Request Acknowledge Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | To deliver forwarded DL PDUs. | - |  |
| Security Result | O |  | 9.3.1.59 |  | - |  |
| QoS Flow Setup Response List | M |  | QoS Flow List with Data Forwarding 9.3.2.13 | QoS flows associated with the DL NG-U UP TNL Information IE. | - |  |
| QoS Flow Failed to Setup List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Data Forwarding Response DRB List | O |  | 9.3.1.77 |  | - |  |
| Additional DL UP TNL Information for HO List |  | 0..1 |  |  | YES | ignore |
| >Additional DL UP TNL Information for HO Item |  | 1..<maxnoofMultiConnectivityMinusOne> |  | Additional DL UP TNL Information for split PDU session, in the same order as the UPF endpoint of the additional NG-U transport bearer(s) received in the Handover Request Transfer IE of the Handover Request message. | - |  |
| >>Additional DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the additional NG-U transport bearer for delivery of DL PDUs. | - |  |
| >>Additional QoS Flow Setup Response List | M |  | QoS Flow List with Data Forwarding 9.3.2.13 | QoS flows associated with the Additional DL NG-U UP TNL Information IE. | - |  |
| >>Additional DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint to deliver forwarded DL PDUs. | - |  |
| >>Additional Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the additional NG-U transport bearer for delivery of redundant DL PDUs. | YES | ignore |
| UL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | To deliver forwarded UL PDUs | YES | reject |
| Additional UL Forwarding UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | NG-RAN node endpoint to deliver forwarded UL PDUs for split PDU session. | YES | reject |
| Data Forwarding Response E-RAB List | O |  | 9.3.1.121 |  | YES | ignore |
| Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs for the redundant transmission. | YES | ignore |
| Used RSN Information | O |  | Redundant PDU Session Information 9.3.1.136 |  | YES | ignore |
| Global RAN Node ID of Secondary NG-RAN Node | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| MBS Support Indicator | O |  | ENUMERATED (TRUE) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMultiConnectivityMinusOne | Maximum no. of connectivity allowed for a UE minus one. Value is 3. The current version of the specification supports 1. |

<Skip unrelated part>

9.3.X	MB-SMF Related IEs

9.3.X.Y1	Multicast Session Update Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multicast Session List | M |  |  |  |
| > Multicast Session Item |  | 1..<maxnoofMBSSessions> |  |  |
| >>MBS Session ID | M |  | FFS |  |
| >>MBS QoS flow ID | M |  | 9.3.1.51 |  |
| >>MBS QoS Flow Parameters | M |  | FFS |  |
| >>mapped QoS Flow Information | M |  | FFS |  |
| >> CHOICE multicast session update | M |  |  |  |
| >>>activate | M |  | ENUMERATED (true, …) | This IE is used to activate the related multicast session at RAN node. |
| >>>deactivate | M |  | ENUMERATED (true, …) | This IE is used to deactivate the related multicast session at RAN node. |
| >>>update |  |  |  |  |
| >>>Allocation and Retention Priority | M |  | 9.3.1.19 |  |

9.3.X.Y2	Multicast Session Update Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Multicast Session List | M |  | FFS |  | YES | reject |

9.3.X.Y3	Multicast Session Update Unsuccessful Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Multicast Session Update Unsuccessful Item | M |  | FFS |  | YES | reject |

9.3.X.Y4	Multicast Distribution Establishment Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Shared N3 Tunnel Establish Request Item |  |  | FFS |  |  |  |

9.3.X.Y5	Multicast Distribution Establishment Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Shared N3 Tunnel Establish Response Item | M |  | FFS |  |  |  |

9.3.X.Y6	Multicast Distribution Establishment Unsuccessful Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Shared N3 Tunnel Establish Unsuccessful Item | M |  | FFS |  | YES | reject |

9.3.X.Y7	Multicast Distribution Release Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Shared N3 Tunnel Release Request Item |  |  | FFS |  |  |  |

9.3.X.Y8	Multicast Distribution Release Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Shared N3 Tunnel Release Update Response Item | M |  | FFS |  |  |  |

9.3.X.Y9	Multicast Distribution Release Unsuccessful Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Shared N3 Tunnel Release Unsuccessful Item | M |  | FFS |  | YES | reject |

9.3.Y.1 	Broadcast Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID |  |  | FFS |  |  |  |
| MBS QoS Flow ID |  |  | 9.3.1.51 |  |  |  |
| MBS QoS flow QoS parameters |  |  | FFS |  |  |  |
| Mapped QoS Flow ID Information |  |  | FFS |  |  |  |
| NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 |  | Yes | reject |
| QoS Flow Setup Request List |  | 1 | FFS |  |  |  |

9.3.Y.2 	Broadcast Session Resource Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Broadcast IP Address |  |  | FFS |  |  |  |

9.3.Y.3 	Broadcast Session Resource Setup Failure Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| FFS |  |  |  |  |  |  |

9.3.Y.4 	Broadcast Session Resource Release Command Transfer

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| FFS |  |  |  |  |  |  |

9.3.Y.5 	Broadcast Session Resource Release Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| FFS |  |  |  |  |  |  |

9.3.Y.6 	Broadcast Session Update Request Transfer

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| FFS |  |  |  |  |  |  |

9.3.Y.7 	Broadcast Session Update Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| FFS |  |  |  |  |  |  |

9.3.Y.8 	Broadcast Session Update Failure Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| FFS |  |  |  |  |  |  |

----------------------------------The END-------------------------------------