3GPP TSG-RAN WG3 Meeting #114-e	R3-215664

E-meeting, 01-11 Nov. 2021

Title: 	Further discussions on remaining open issues about general principles and framework

Source: 	Huawei

Agenda item:	18.2

Document Type:	Other

1. Introduction

The general principles and functional framework for data collection was discussed during RAN3#113-e. In R3-214220 [1], the summary of the offline discussion was presented. This paper tried to have further discussions on the remaining open issues.

2. Background

Whether to Keep the model performance feedback arrow from model inference to model training using a dash line or together with some clarification text needs to be decided in the next meeting.

Open issues:

FFS2 whether and how to signal the metrics and validity time together with or as part of the inference output.

FFS3 if the study assumes single vendor environment, e.g., if model payload is proprietary and if the model deployment/update procedure is proprietary.

To be continued...

3. Discussion

For the model performance feedback, the following issues should be considered:

- Model performance feedback is used for transferring the model performance from the model inference to the model training. For example, it is used by the model training to learn new knowledge from new training data. However, there is no good understanding about the model performance by the model inference itself due to the lack of the labelled data. Therefore, this is typically done only after the action is taken can the model be evaluated. In this case, the model performance feedback can be transferred from data collection to model training to support online training or reinforcement learning.

Observation 1: Model inference could not provide the model performance feedback to the model training without the help of action.

- The model performance feedback is not known at the time of inference and has to be calculated till the labelled data are received. Some companies argued that the labelled data could be collected from the data collection to the model inference. However, since the model inference is typically located in the RAN that has limited capacity and does not have the data storage function, the acquiring and storing labelled data may increase the burden for the model inference to calculate the model performance feedback.

Observation 2: Since the model inference is typically located in the RAN that has limited capacity and does not have the data storage function, it is difficult for the model inference to acquire and store the labelled data for calculation of model performance feedback.

- Data collection is responsible for collecting all types of data, and it can be located in any entities providing the data related to the AI model training. Therefore, even the model inference could calculate the model performance feedback, the data collection does already support the transferring the model performance feedback from model inference to model training. There’s no need to duplicate model performance feedback from model inference to model training.

Observation 3: Data collection could be responsible for collecting data which could be used for training and/ or inference.

Therefore, we propose to remove the model performance feedback arrow. We also propose to describe the data collection function in further details.

Remove the “model performance feedback” arrow.

For the model deployment/update, it is conceptually correct from the functional framework perspective. However, it is argued what information will “Model deployment/update” transfer, whether this information will need to be standardized and if not, what are the assumptions on this information.

Before we jump into this discussion, we should mention that the group agreed the following high level principles:

- The detailed AI/ML algorithms and models for use cases are out of RAN3 scope.

The reason behind that is the AI/ML algorithms and models are implementation specific and as such the corresponding details of the AI/ML algorithms and models are various and software and hardware dependent, which is difficult to be standardized. For example, the AI/ML algorithms are highly related to the model technologies (e.g., gradients in the learning setting) and model execution environment, which are not expected to be specified in 3GPP.

Therefore, the compromise proposed in last meeting about the single vendor environment is a suitable way. That is, add a note to clarify that the details of model deployment/update is vendor proprietary.

Add a NOTE to clarify that the details of model deployment/update is vendor proprietary.

For the level of accuracy of the output prediction, it is helpful for the receiver to understand the prediction performance of the algorithm that generated it, in order to understand how to trust the prediction. However, we should mention that the accuracy of the prediction is exactly the confidence, but not the accuracy of the AI/ML algorithm and model.

- Accuracy: it defines the skill of the learning algorithm to predict accurately. It defines the percentage of correct predictions made from all predictions. It is used to describe the trained AI/ML model at the model training function. For example, if the algorithm makes 970 good predictions out of 1000 examples, it means the algorithm accuracy is 97%. Typically, during the model training phase, a portion of the sample data is saved for validation purpose.

- Confidence: AI/ML is probabilistic and the probability of each single prediction can be indicated by the confidence score. In other words, the confidence scores reflect the chances that the AI/ML is correct. Typically, the confidence score can be calculated for single input as well giving the meaning as how much the algorithm is confident for that class. If a class has high probability then it has high confidence.

As we could see that for either accuracy or confidence, it is to evaluate the efficiency or performance of the model training which is also algorithm dependent, and it seems to be used for the model training or inference to evaluate itself.

Observation 4: Technically, accuracy is to describe the precision of a model training, while confidence is to indicate the probability of inference output approaching the reality; but they seem to be used for the model training or inference to evaluate its performance by itself.

For the validity time for which the output information is valid, it may not be always needed for some use cases.

- The assumption by RAN3 so far is that the received information are valid before a new version of such information is received. For example, in the resource status reporting procedure, the NG-RAN node will use the latest received cell load information exchanged over the Xn interface with the neighbour NG-RAN node to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cell or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to achieve network energy saving.

- The output information can be reported to the subscribed node with a given periodicity, which means the output information is valid during the time period (validity time). For example, in the resource status reporting initiation procedure, the NG-RAN node can request the target node to report the load information with the reporting periodicity, e.g., 500ms, 1000ms, 2000ms.

Therefore, there is no need to include the validity time together with the output information every time. Instead, it can be considered on a case by case manner when the use case and solution are discussed.

No need to include the validity time together with the output information every time.

There is still one more open issue about data storage at RAN due to model training, As summarized in the SoD [1], the different understanding on this point is mainly about whether RAN is allowed to store data. The main argumentation for allowing data storage is that data collection and model training would require storing data, and some companies also think that it might be related with different use cases. The reset of the paper will try to discuss the relations among data collection, data storage and model training, and share our observations.

- Data storage

Technically, the data storage is a basic part of a data server where a software program/platform is used to provide database services like storing, processing and securing data, and data server contains the installation of the database system, from which database services are provided and consumed by other software programs or components.

With this understanding, apparently it is not RAN’s duty to provide database services, which is built on top of data storage function.

Observation 5: It is not RAN’s duty to provide database services, which is built on top of data storage function.

- Data collection

Some companies argued that data collection might require storing data, it is true, but storing data and data storage are technically different with each other. Actually measurement reporting is a typical data collection process, through which the receiving end, i.e. base station, will temporarily store the received data for later usage, e.g. HO decision or forward to OAM. Here the most important point is temporary storing, i.e. with data storage space, device can keep and retain data even the device is power down, while in current data collection behavior for measurement report or MDT report, reported data is not required to be retained. In other words, data collection doesn’t require data storage function.

Observation 6: Data collection doesn’t require data storage function.

- Model training

During last meeting, the concept of offline training and online training was mentioned. Technically, the more precise in AI/ML should be offline or online learning which includes training and inferring. Simply speaking, in online learning the training is done with the data coming in a in a sequential order and model (e.g. the weight) is kept updating along with the new input data; while for offline learning the training is done with the whole data set being learned and the model is stable.

From the above descriptions, we could see that difference is obvious, online training doesn’t require to store the data (or temporary storing a batch of data in sequence for batch based training), the data just comes and are input to the model in sequence and consumed, which is quite similar like the existing process that measurement report comes and is input to HO function for decision making; while for offline training, the whole data set has to be available in advance upon for request which means the large amount of data storage. And, we also see that the existing framework, e.g. measurement report and HO decision making, already supports online training, no fundamental changes are needed to support online training with current framework.

Observation 7: Offline training requires the large amount of data storage while online training doesn’t.

Observation 7bis: The process of data input for model training and inferring in online training is quite similar as the existing function of measurement report as input for HO decision, i.e. the existing framework already supports online training.

Taking all the observations above into account, we could conclude that offline training should not be performed at RAN side, since the RAN just acts as a transmission pipeline which doesn’t provide any data storage or data base services, and this role should not be changed by the introduction of AI/ML operation. Thus we would like to have the following proposal:

The introduction of AI/ML operation at RAN side should not force RAN to provide data base service function.

- As the location of offline training, it was the RAN3 common understanding the OAM should be the suitable place which was indicated in the LS to SA5 [2].

4. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: Model inference could not provide the model performance feedback to the model training without the help of action.

Observation 2: Since the model inference is typically located in the RAN that has limited capacity and does not have the data storage function, it is difficult for the model inference to acquire and store the label data for calculation of model performance feedback.

Observation 3: Data collection could be responsible for collecting data which could be used for training and/ or inference.

Observation 4: Technically, accuracy is to describe the precision of a model training, while confidence is to indicate the probability of inference output approaching the reality; but they seem to be used for the model training or inference to evaluate its performance by itself.

Observation 5: It is not RAN’s duty to provide database services, which is built on top of data storage function.

Observation 6: Data collection doesn’t require data storage function.

Observation 7: Offline training requires the large amount of data storage while online training doesn’t.

Observation 7bis: The process of data input for model training and inferring in online training is quite similar as the existing function of measurement report as input for HO decision, i.e. the existing framework already supports online training.

Proposal 1:	Remove the “model performance feedback” arrow.

Proposal 2:	Add a NOTE to clarify that the details of model deployment/update is vendor proprietary.

Proposal 3:	No need to include the validity time together with the output information every time.

Proposal 4:	The introduction of AI/ML operation at RAN side should not force RAN to provide data base service function.

5. Reference

- R3-214220, Summary of offline discussion on AI RAN general framework.

Annex – TP

START OF CHANGE

4	General Framework

Editor Note: high level principles for RAN intelligence enabled by AI, the functional framework (e.g. the AI functionality and the input/output of the component for AI enabled optimization)

4.1	High-level Principles

The following high level principles should be applied for AI-enabled RAN intelligence:

- The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope.

- The study focuses on AI/ML functionality and corresponding types of inputs/outputs.

- The input/output and the location of the Model Training and Model Inference function should be studied case by case.

- The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope.

- The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope.

- Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

- The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm.

- The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g. via subscription), or nodes that are subject to actions based on the output from model inference.

- An AI/ML model used in a Model Inference function has to be initially trained, validated and tested before deployment.

- NG-RAN is prioritized; EN-DC is included in the scope. FFS on whether MR-DC should be down-prioritized.

- A general framework and workflow for AI/ML optimization should be defined and captured in the TR. The generalized workflow should not prevent to “think beyond” the workflow if the use case requires so.

- User data privacy and anonymisation should be respected during AI/ML operation.

Note: The introduction of AI/ML operation at RAN side should not force RAN to provide data base service function.

4.2	Functional Framework

Editor’s Note: Data Preparation aspects may be further refined

Figure 4.2-1: Functional Framework for RAN Intelligence

This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1. For the functions and data/information flows shown in the Figure 4.2-1, whether there is any standardization impact and what is the standardization impact are discussed in clause 5.

- Data Collection is a function that provides input data to Model training and Model inference functions. AI/ML algorithm specific data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) is not carried out in the Data Collection function.  
Examples of input data may include measurements from UEs or different network entities, feedback from Actor, output from an AI/ML model.

  - Training Data: Data needed as input for the AI/ML Model Training function.

  - Inference Data: Data needed as input for the AI/ML Model Inference function.

- Model Training is a function that performs the ML model training, validation, and testing. The Model training function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

  - Model Deployment/Update: Deploy or update an AI/ML model to Model Inference function.

  - Model training may include online training which could be performed inside RAN, and offline training which should be outside RAN, e.g. at OAM.

NOTE: The payload of Model Deployment/Update is vendor proprietary.

- Model Inference is a function that provides AI/ML model inference output (e.g. predictions or decisions). The Model inference function is also responsible for data preparation (e.g. data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

  - Output: The inference output of the AI/ML model produced by a Model Inference function.

- Actor is a function that receives the output from the Model inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

  - Feedback: Information that may be needed to derive training or inference data or performance feedback or model performance feedback.