3GPP TSG-RAN WG3 #114-e	                                               R3-215682

Online, Nov 1st – Nov 11th, 2021

Agenda item:	19.2.1

Source:	CMCC

Title:	(TP for TS 38.455) Positioning Accuracy Improvements

Document for:	Discussion

Introduction

In RAN3#113e meeting, the following proposals of positioning accuracy improvements are shown below:

- Define a new Assistance Information for UL-AoA IE with the Expected AoA/ZoA values and their Uncertainties using a CHOICE structure. Stage 3 details are FFS

- Agree to introduce the UL-AoA assistance information IN NRPPA MEASUREMENT REQUEST message, within the TRP Measurement Request List IE

- Agree to introduce the UL-AoA assistance information IN NRPPA MEASUREMENT UPDATE message, within a new TRP Measurement Update List IE (FFS)

- Agree to introduce a Zenith Angle of Arrival IE to the TRP Measurement Result IE. Adding description for linear array FFS

- No indication from gNB to LMF is needed for requesting the Expected UL AoA

- An F1AP TP is needed. Maintain the BL CR up to date with respect to NRPPa progress.

In this contribution, we will continue to work on stage3 details of positioning accuracy and provide the TP for positioning accuracy improvements in TS 38.455.

Discussion

From the discussion in last meeting, there are still some remaining issues waiting to be discussed. The first point is the presence of uncertainty range in the UL-AoA assistance information. In our view, the uncertainty range should be indicated as mandatory. The MEASUREMENT REQUEST message is sent by the LMF to request the NG-RAN node to configure a positioning measurement. LMF should have the ability to estimate a coarse location before requesting the RAN node to perform measurement. Thus, LMF should report expected Azimuth and Zenith AoA Uncertainty Range to RAN.

Proposal 1: The presence of uncertainty range should be marked as mandatory.

The other modification worthy of attention is whether to only request the Zenith AoA measurement included in TRP Measurement Type IE through MEASUREMENT REQUEST message. If only requesting for Zenith AoA, RAN node has the probability of not having the ability for Zenith AoA measurement. Thus, we only recommend to provide both Azimuth AoA and Zenith AoA measurement to RAN and remove ZoA in TRP Measurement Type IE.

Proposal 2: Do not recommend to request the Zenith Angle of Arrival only and remove ZoA in TRP Measurement Type IE.

Discussion

Proposal 1: The presence of uncertainty range should be marked as mandatory.

Proposal 2: Do not recommend to request the Zenith Angle of Arrival only and remove ZoA in TRP Measurement Type IE.

Text proposal for TS 38.455

START OF CHANGES

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >> AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.x4 |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min is not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Response Time | O |  | 9.2.x6 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

NEXT CHANGE

### 9.2.x4	UL-AoA assistance information

This information element contains the expected uplink Angle of Arrival and uncertainty range.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE AngleMeasurement | M |  |  |  |
| >Expected UL Angle of Arrival |  |  |  |  |
| >>Expected Azimuth AoA |  | 1 |  | Defined as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) |
| >>>Expected Azimuth AoA Value | M |  | INTEGER(0..3599) | φAOA component of Expected Azimuth AoA |
| >>>Expected Azimuth AoA Uncertainty Range | M |  | INTEGER(0..3599) | ΔφAOA component of Expected Azimuth AoA |
| >>Expected Zenith AoA |  | 0..1 |  | Defined as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2) |
| >>>Expected Zenith AoA Value | M |  | INTEGER(0..1799) | θZOA component of Expected Zenith AoA |
| >>>Expected Zenith AoA Uncertainty Range | M |  | INTEGER(0..1799) | ΔθZOA component of Expected Zenith AoA |
| >Expected UL Angle of Arrival Zenith Only |  |  |  | Defined as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2) |
| >>Expected Zenith AoA Value | M |  | INTEGER(0..1799) | θZOA component of Expected Zenith AoA |
| >>Expected Zenith AoA Uncertainty Range | M |  | INTEGER(0..1799) | ΔθZOA component of Expected Zenith AoA |
| LCS to GCS Translation |  | 0..1 |  | If absent, the azimuth and zenith are provided in GCS. |
| >Alpha | M |  | INTEGER (0..3599) |  |
| >Beta | M |  | INTEGER (0..3599) |  |
| >Gamma | M |  | INTEGER (0..3599) |  |

NEXT CHANGE

Expected-Azimuth-AoA ::= SEQUENCE {

expected-Azimuth-AoA-value			Expected-Value-AoA,

expected-Azimuth-AoA-uncertainty	Uncertainty-range-AoA	,

...

}

Expected-Zenith-AoA ::= SEQUENCE {

expected-Zenith-AoA-value			Expected-Value-ZoA,

expected-Zenith-AoA-uncertainty		Uncertainty-range-ZoA	,

...

}

NEXT CHANGE

TRPMeasurementQuantities-Item ::= ENUMERATED {

gNB-RxTxTimeDiff,

uL-SRS-RSRP,

uL-AoA,

uL-RTOA,

}

End OF CHANGES