3GPP TSG-RAN WG3 #114e                                                                               R3-215695

Online, 1st – 11th November 2021

Title: 	(TP to TS 38.413)UE history information in MR-DC

Source: 	CMCC, CATT

Agenda item:	10.2.1.4

Document Type:	other

# 1. Introduction

This contribution is the TP for TS 38.413 BLCR with addition of UE history information for SN.

# 3. Annex – TP to TS 38.413 BLCR

Start of the first change

9.3.1.95	UE History Information

This IE contains information about cells that a UE has been served by in active state prior to the target cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Last Visited Cell Item |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list. |
| >Last Visited Cell Information | M |  | 9.3.1.96 |  |
| > Last Visited PSCell List |  | 0..1 |  |  |
| >> Last Visited PSCell Item |  | 1..< maxnoOfPSCellsinUEHistoryInfo > |  | Most recent information is added to the top of this list |
| >>> Last Visited PSCell Information | M |  | 9.2.1.X |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum no. of cells in the UE history information. Value is 16. |
| maxnoOfPSCellsinUEHistoryInfo | Maximum length of the list. Value is 16. |

9.3.1.X	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Last Visited PSCell Information | M |  |  |  | - |  |
| >E-UTRAN Cell |  |  |  |  | - |  |
| >>Last Visited E-UTRAN PSCell Information | M |  | OCTET STRING | Defined in TS 36.413 [16]. | - |  |
| >NG-RAN Cell |  |  |  |  |  |  |
| >>Last Visited NG-RAN PSCell Information | M |  | 9.3.1.Y |  |  |  |

9.3.1.Y	Last Visited NG-RAN PSCell Information

The Last Visited NG-RAN PSCell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell Global Cell ID | O |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the PSCell in seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the PSCell in 1/10 seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 40950. | YES | ignore |

End of the last change