3GPP TSG-RAN WG3 #114-e	R3-215733

1 - 11Nov 2021

Online

Title: 	TP for 38.413 Report Amount for M4, M5, M6, M7 measurement

Source: 	ZTE

Agenda item:	8.3.4

Document for:	Approval

1	Introduction and discussion

As discussed in corresponding discussion contribution:

Observation : For M4,M5,M7, current value range of report amount should be extended.

Proposal 1: To introduce report amount for M4,M5,M6,M7.

The value range of M4 is ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity).

The value range of M5,M6,M7 is ENUMERATED (1, 2, 4, 8, 16, 32, 64, 128, 256,512,1024,2048, infinity).

This contribution provided the TP for TS38.413.

2	TP to TS 38.413

*** Start changed ***

9.3.1.172	M4 Configuration

This IE defines the parameters for M4 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M4 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M4 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |
| M4 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64,infinity …) | Number of reports. |

*** next change ***

9.3.1.173	M5 Configuration

This IE defines the parameters for M5 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M5 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M5 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |
| M5 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64,128, 256,512,1024,2048, infinity …) | Number of reports. |

*** next change ***

9.3.1.174	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, …) |  |
| M6 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64,128, 256,512,1024,2048, infinity …) | Number of reports. |

*** next change ***

9.3.1.175	M7 Configuration

This IE defines the parameters for M7 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | INTEGER (1..60, …) | Unit: minutes |
| M7 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |
| M7 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64,128, 256,512,1024,2048, infinity …) | Number of reports. |

*** End of changed ***