3GPP TSG-RAN WG3 Meeting #114-e	R3-215783

1 – 11 November 2021

Online

Agenda Item:	30.4

Source:	Huawei, Telecom Italia

Title:	(TP for BL CR TS 38.401) Use of node ID in eNB

Document for:	Other

# 1	Introduction

During RAN3#113-e, the only remaining issue which is controversial is about whether introduce a new ID or reuse the node ID. In this contribution, we try to further discuss the issue.

# 2	Discussion

As mentioned in our previous contributions, the relationship between the eNB-CP and the eNB-UP are peer to peer. That is, the eNB-CP or eNB-UP knows well which node is communicating to. Therefore, the gNB-CU-UP ID could be used to uniquely identify the gNB-CU-UP for the gNB-CU CP-UP separation, or to uniquely identify the ng-eNB-CU-UP for the ng-eNB-CU CP-UP separation, or to uniquely identify the eNB-UP for the eNB CP-UP separation.

Note that, nothing is broken with the reuse of exisiting gNB-CU-UP ID, so the following is proposed.

Proposal 1: Reuse the gNB-CU-UP ID and agree the TP in annex.

# 3	Conclusion

In this contribution the remaining problem of WI has been discussed, and the following proposals have been made:

Proposal 1: Reuse the gNB-CU-UP ID and agree the TP in annex.

# Annex: TP for TS 38.401

6.2.4	gNB-CU-UP ID

The gNB-CU-UP ID is configured at the gNB-CU-CP and used to uniquely identify the gNB-CU-UP at least within a gNB-CU-CP. The gNB-CU-UP provides its gNB-CU-UP ID to the gNB-CU-CP during the E1 Setup procedure. The gNB-CP-UP ID is used only within E1AP procedures.

NOTE 1: This identity is also used to uniquely identify the ng-eNB-CU-UP at least within an ng-eNB-CU-CP in case CP/UP separation is implemented in ng-eNB, unless explicitly specified otherwise.

NOTE 2: This identity is also used to uniquely identify the eNB-UP at least within an eNB-CP in case CP/UP separation is implemented in eNB, unless explicitly specified otherwise.