3GPP TSG-RAN3 Meeting #114-e	R3-215796

Electronic meeting, 1- 11 November 2021

3GPP TSG-WG SA2 Meeting #147E e-meeting 		S2-2107860

Elbonia, October 18 – 22, 2021

Title:	LS Response on full registration request message to be rerouted via RAN

Response to:	S2-2107061 / S3-213035

Release:	Rel-17

Work Item:	FS_AMFREAL_SEC

Source:	SA2

To:	SA3, RAN3

Cc:	CT1,  CT4

Contact Person:

#### Name:	Fenqin Zhu

E-mail Address:	zhufenqin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS23.502 CR3150

1. Overall Description:

SA2 thanks SA3 for their LS on full registration request message to be rerouted via RAN. SA2 has the following answer to the question mentioned in the LS.

Question 1: Is the full Registration Request message the complete Registration Request message that contains both clear text IEs and non-clear text IEs, as defined in clause 4.4.6. in TS 24.501?

[Answer 1]:  When the initial AMF reroute the registration request message to the target AMF, it is expected that the complete Registration Request message received by the initial AMF are transferred to the target AMF.

The complete registration request message is:

- For A UE have valid security context: The registration request message UE sent in step 1 of clause 4.2.2.2.2 in TS23.502 includes the clear and non-clear text IEs. The complete Registration Request message is in the non-clear text IE in NAS container received from the UE.

- UE have no valid security context. After activating a 5G NAS security context resulting from a security mode control procedure, the UE includes the complete registration request message in clear text in the NAS container within the Security Mode Complete message, i.e. step 9b in clause 4.2.2.2.2 of TS23.502.

When the Registration Request message need be transferred from initial AMF to target AMF as described in step 7(A) and 7(B) of clause 4.2.2.2.3 of TS 23.502, the latest complete registration request message received by the initial AMF is transferred to the target AMF. For the 7(A), the complete registration request message is transferred as clear text together with the UE security context as specified in 33.501. SA2 has endorsed the attached CR to clarify the rerouted Registration Request message.

Question 2: What is the Initial UE message in step 7b?

[Answer 2]:  The Initial UE message in step 7b is the NGAP message received in the reroute NAS message sent by the initial AMF to the RAN. Note that the Initial UE message in this Reroute NAS Message is NOT necessarily the one received from the RAN in step 1, as it has to include the complete Registration message and this is not available to the AMF in Step 1) if the UE has no valid NAS security context.

Question 3: Does the Registration Request message the initial AMF sends out refer to

- the message step 1 of 4.2.2.2.3 Registration with AMF re-allocation, or

- the complete registration request received by the initial AMF in NAS Security mode complete message when the message in step 1 is not protected as UE had no security context; and the complete registration request message in the NAS container in the message in step 1 which is protected as UE had security context?

[Answer 3]:  When the initial AMF reroute the registration request message to the target AMF, the registration request message transferred to the target AMF is the complete Registration Request message (as described in Answer 1).

Question 4: Is it feasible from SA2 perspective if the initial AMF sends the message in Step 1 of 4.2.2.2.3 for reroute?

[Answer 4]:  There are two cases for the registration request message sent by UE in step 1:

- UE has not NAS security context. In this case the message in step 1 cannot include the non-clear text IEs, which need be ciphered as defined in TS 24.501, e.g. Requested S-NSSAI.

- UE has NAS security context. In this case the message sent by UE in step 1 includes ciphered IE.

Then the Registration Request can also be received in security mode complete message as described in Answer 1 above.

The target AMF is expected to use the received Registration Request message for the continuation of the Registration Request procedure.

For the 7(B) case (i.e. rerouting via RAN node), SA2 considers that SA3 shall take the lead and decide how the Registration Request message shall be rerouted as part of the study (e.g. it’s not necessary to consider only the Registration Request in step 1 for rerouting as described in Answer 2).

2. Actions:

To SA3 group:

ACTION: 	SA2 kindly requests SA3 to take the responses provided above into account.

To RAN3 group:

ACTION: 	SA2 kindly requests RAN3 to take the responses provided above into account and provide any comments to the reply and also to the technically endorsed CR

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2 #148e	15 - 19 Nov 2021	Elbonia

3GPP SA2 #149e	14 - 25 Feb 2022	Elbonia