3GPP TSG-RAN WG3 #114e	R3-215952

# Online, 1st – 11th November 2021

Agenda Item:	14.3

Source:	Lenovo, Motorola Mobility (Moderator)

Title: 	         Summary of offline discussion on CPAC

Document for:	Discussion and Approval

1	Introduction

CB: # MRDC3_CPAC

- Check RAN2 progress

- Support preparation of single T-SN in SN initiated inter-SN CPC first to progress, and then discuss how to prepare multiple T-SNs as second priority? Check RAN2 progress and focus on open issues from last meeting

- CPAC replace and cancel procedure?

- Capture agreements as stage2/stage3 CRs and check details, split work, if needed

- List open issues for next meeting in the summary

(Lenovo - moderator)

Summary of offline disc R3-215864

The offline discussion will comprise 2 phases

- Phase 1: Try to identify easy agreements and controversial issues for Phase 2 discussion

  - Deadline: November 3rd, Wednesday, 4am UTC

- Phase 2: Further discuss, e.g., CPAC replace/cancel signalling design, and try to come up with TP if agreeable

  - Intermediate deadline: End of Friday 5th Nov, companies providing comments to email discussion questions

  - Final deadline: Nov 9th, Tuesday, 4am UTC, upload agreeable TPs and companies providing comments to agreeable TPs

2	For the Chairman’s Notes (Phase 2)

Proposal A: During CPA and MN/SN initiated inter-SN CPC, MN cannot decide the PSCells to be cancelled and indicate to the target SN.

Proposal B: During CPA and MN/SN initiated inter-SN CPC, when MN reduces the maximum number of PSCells can be prepared to a value less than the number of PSCells have been prepared, target SN shall cancel some prepared PSCells (e.g., in the SN Modification Request Acknowledge message).

Proposal C: During SN initiated inter-SN CPC, when source SN reduces the maximum number of PSCells can be prepared to a value less than the number of PSCells have been prepared, target SN shall cancel some prepared PSCells (e.g., in the SN Modification Request Acknowledge message).

FFS1: During SN initiated inter-SN CPC, if source SN can decide the PSCells to be cancelled and indicate to the target SN via MN.

Signalling between MN and target SN

Proposal D: In CPA and MN/SN initiated inter-SN CPC, MN can:

- Update/modify previous CPAC configurations provided in CPAC addition using MN initiated SN modification procedure

- Cancel all prepared PSCells at target SN and release the target SN using MN initiated SN release procedure

Proposal E: In CPA and MN/SN initiated inter-SN CPC, target SN can:

- Update/modify previous CPAC configurations provided in CPAC addition using SN initiated SN modification procedure

- Add prepared PSCells within the limit given by the MN or source SN using SN initiated SN modification procedure

- Cancel some of the prepared PSCells using SN initiated SN modification procedure.

- Cancel all prepared PSCells using SN initiated SN release procedure

Signalling between MN and source SN

Proposal F: in SN initiated inter-SN CPC, source SN can:

- Update/modify previous CPC configurations provided in CPC preparation using SN change required procedure

- Cancel all prepared PSCells at target SN and release the target SN using SN change required procedure

Proposal G: New IEs are introduced in SN change required message indicating CPC configuration modification and target SN release.

Proposal H: in MN/SN initiated inter-SN CPC, MN can inform source SN about the triggered target SN release or some prepared PSCells cancellation at a target SN using:

- A new class2 XnAP procedure

- A new class2 X2AP procedure

TPs to be agreed

R3-216038 was R3-215109, TP for CPAC BL CR TS37.340, agreed

R3-216147, TP for CPAC BL CR TS38.423, agreed

R3-216148, TP for CPAC BL CR TS36.423, agreed

R3-216167, TP for CPAC BL CR TS36.420, agreed

R3-216207, CPAC BL CR TS38.420, Endorsed as BL CR

3	Discussion (Phase 2)

According to the feedback received in phase 1 and to avoid confusion, in phase 2 discussion, moderator aims to formulate “replace” and “cancel” as clear as possible.

3.1 Cancellation of some prepared PSCells without releasing SN

Regarding whether MN or source SN can trigger cancellation of some prepared PSCells at candidate SN while keep the target SN, companies have different understanding. In [14] [18], companies believe MN can explicitly indicate to candidate SN some prepared PSCells are to be cancelled. MN may determine that based on its received measurement result. In [10] and commented by some companies in phase 1 discussion, MN may reduce the number of max PSCells can be prepared, and in case it becomes less than the number of PSCells have been prepared, candidate SN need to cancel some of the prepared PSCells without releasing SN.

Question A: Companies are kindly asked for in CPA and MN/SN initiated inter-SN CPC, can MN trigger cancellation of some prepared PSCells without releasing the target SN by any of the following way?

- MN explicitly indicate the PSCells to be cancelled to the target SN.

- MN reduces the max number of PSCells to prepared, which will lead to PSCells cancellation if the limit is less than the number of PSCells that have been prepared.

- None

| Company | 1)2)3) | Comments (Please also comment which Xn procedure shall be used, e.g. MN initiated SN modification, and possible enhancement) |
| --- | --- | --- |
| Nokia | 2 | As discussed before: MN may not have knowledge to select which PSCells are to be prepared – it may not be able to interpret or even read the NR measurements. This is already reflected in the Addition. But it also means it can’t select which PSCells are not needed and shall be cancelled. Of course, the MN may need to force the SN to limit the number of prepared PSCells – but the selection of the PSCells to release or to keep is up to the target SN. |
| Lenovo, Motorola Mobility | 1) 2) | 2) is clear. 1) in our understanding, MN is aware of the prepared PSCells, and MN will receive measurement result from UE, which might indicate some PSCells are no longer qualified. In such case, we think it’s ok for MN to decide and cancel some prepared PSCells. It’s similar as source RAN node can cancel some CHO prepared cells by sending Candidate Cells To Be Cancelled List IE included in the HANDOVER CANCEL message. Both 1) and 2) can be done via MN initiated SN procedure. For 1), a Candidate Cells To Be Cancelled List IE can be added, similar as the one in HANDOVER CANCEL message. |
| ZTE | 2 | Agree with Nokia, more, it is different from CHO, the MN has no idea of PSCell information. |
| Huawei | 2 | The PSCell assignment is up to the target SN, therefore the MN can only control the max number of PSCells, but not which PSCell to be prepared. |
| CATT | 2) | Same view as Nokia |
| NEC | 2 or 3 | PScell is decided by SN. For simplicity, if can just dismiss the “MN trigger cancellation of some prepared PSCells”, then 3). |
| Samsung | 2 | Agree with Nokia. |
| E/// | 2 | 2 seems a simpler implicit way without introducing any further enhancement. |

Moderator’s Observation:

Majority companies do not want to support option 1). All companies agree with option 2)

Proposal A: During CPA and MN/SN initiated inter-SN CPC, MN cannot indicate the PSCells to be cancelled to the target SN.

Proposal B: During CPA and MN/SN initiated inter-SN CPC, when MN reduces the maximum number of PSCells can be prepared to a value less than the number of PSCells have been prepared, target SN shall cancel some prepared PSCells (e.g., in the SN Modification Request Acknowledge message).

Question B: Companies are kindly asked for in SN initiated inter-SN CPC, can source SN trigger cancellation of some prepared PSCells without releasing the target SN by any of the following way?

- SN explicitly indicate the PSCells to be cancelled to the target SN.

- SN reduces the max number of PSCells to prepared, which will lead to PSCells cancellation if the limit is less than the number of PSCells that have been prepared.

- None

| Company | 1)2)3) | Comments (Please also comment which Xn procedure shall be used, e.g. SN change required, and possible enhancement) |
| --- | --- | --- |
| Nokia | 2 | Here, one may argue that the source SN can interpret the NR measurements. This may be true, but despite this, in case of a classic SN change, RAN3 agreed in Rel.15 that the selection of the target PSCell is done in the target SN. If this principle is to be changed, RAN3 must identify and agree the reasons for that: what is different in case of CPC that should enable the source SN to do something that was not allowed before? Option 2 (reducing the max number) is in line with the past agreements and can reuse CPA signalling. Hence the selection. |
| Lenovo, Motorola Mobility | 1)2) | Same comment as in Q-A but for source SN |
| ZTE | 1 and 2 | For this case, the S-SN can indicate PSCells. |
| Huawei | 2 | The PSCell assignment is up to the target SN, therefore the source SN can only control the max number of PSCells, but not which PSCell to be prepared. |
| CATT | - 2) |  |
| NEC | 2 or 3 | PScell is decided by SN. For simplicity, if can just dismiss the “MN trigger cancellation of some prepared PSCells”, then 3). |
| Samsung | 1 and 2 |  |
| E/// | 2 | Same as QA |

Moderator’s observation:

All companies support 2), while some companies support 1).

Proposal C: During SN initiated inter-SN CPC, when source SN reduces the maximum number of PSCells can be prepared to a value less than the number of PSCells have been prepared, target SN shall cancel some prepared PSCells (e.g., in the SN Modification Request Acknowledge message).

FFS1: During SN initiated inter-SN CPC, if source SN can explicitly indicate the PSCells to be cancelled to the target SN.

3.2 Signalling between MN and target SN

Question C: Do companies agree in CPA and MN/SN initiated inter-SN CPC, MN can:

- Update/modify previous CPAC related configurations using MN initiated SN modification procedure

- Cancel all prepared PSCells at target SN and release the target SN using MN initiated SN release procedure

| Company | 1)2) | Comments (please also comment the possible enhancement, e.g. new IE/cause value) |
| --- | --- | --- |
| Nokia | 1: likely yes 2: yes | About 1: it is a bit criptic what is behind the “CPAC related configurations”. We understand it as the information provided in the CPAC Addition. In this case, it is “yes”. |
| Lenovo, Motorola Mobility | 1)2) | We don’t foresee any enhancement in particular. |
| ZTE | Both yes |  |
| Huawei | Agree with Lenovo |  |
| CATT | Both yes |  |
| NEC | 2) | MN initiated release procedure, just for simplicity reason. |
| Samsung | Yes for both |  |
| E/// | Both yes | Partial and full cancellation should be supported. |

Moderator’s observation:

Clear majority to support both. CPAC related configurations provided in CPAC addition could be a good clarification.

Proposal D: In CPA and MN/SN initiated inter-SN CPC, MN can:

- Update/modify previous CPAC configurations provided in CPAC addition using MN initiated SN modification procedure

- Cancel all prepared PSCells at target SN and release the target SN using MN initiated SN release procedure

From target SN perspective, RAN3 agreed following in the previous meeting

For CPA, the T-SN can trigger replace and cancel of prepared PSCells in the T-SN. For MN initiated inter-SN CPC, the T-SN can trigger replace and cancel of prepared PSCells in the T-SN, while the S-SN cannot trigger replace and cancel. For SN initiated inter-SN CPC, the T-SN can trigger replace and cancel of prepared PSCells in the T-SN.

Question D: Do companies agree in CPA and MN/SN initiated inter-SN CPC, target SN can:

- Update/modify previous CPAC related configurations using SN initiated SN modification procedure

- Add prepared PSCells within the limit given by the MN or source SN using SN initiated SN modification procedure

- Cancel some of the prepared PSCells using SN initiated SN modification procedure.

- Cancel all prepared PSCells using SN initiated SN release procedure

| Company | 1)2)3)4) | Comments (please also comment the possible enhancement, e.g. new IE/cause value) |
| --- | --- | --- |
| Nokia | 1: likely yes 2,3,4: yes | About 1: Similar like above, this depends on what is the “CPAC related configurations”. We understand it is the scope of the Addition Request Acknowledge. In this case the answer is “yes”, but it then overlaps with 2 and 3 (which is all right – there, the answer is “yes”, too). |
| Lenovo, Motorola Mobility | 1)2)3)4) | The modification of previous configuration, configuration for new added PSCells, and cancellation of prepared PSCells, will be all carried in the single jumbo RRC container. There might be impact to Xn message if RAN3 confirms that target SN will inform MN about prepared PSCells in Xn message too. We can check later. Besides this, we don’t fore see any necessary enhancement. |
| ZTE | All yes |  |
| Huawei | Agree with Lenovo | T-SN is the boss of prepared PSCells |
| CATT | Yes to all |  |
| NEC | 4) | Using SN initiated SN release procedure, just for simplicity reason. |
| Samsung | Yes for all | Agree with Lenovo. |
| E/// | All allowed |  |

Moderator’s observation:

Clear majority to support all.

Proposal E: In CPA and MN/SN initiated inter-SN CPC, target SN can:

- Update/modify previous CPAC configurations provided in CPAC addition using SN initiated SN modification procedure

- Add prepared PSCells within the limit given by the MN or source SN using SN initiated SN modification procedure

- Cancel some of the prepared PSCells using SN initiated SN modification procedure.

- Cancel all prepared PSCells using SN initiated SN release procedure

3.3. Signalling between MN and source SN

Even though the full flow diagram for SN initiated inter-SN CPC depends on RAN2 conclusion, especially the signalling exchange between MN and source SN after getting the feedback from target SN, we can still discuss how does source SN trigger CPAC replace/cancel, i.e., the first step.

Question E: Do companies agree in SN initiated inter-SN CPC, source SN can:

- Update/modify previous CPC related configurations using SN change required procedure

- Cancel all prepared PSCells at target SN and release the target SN using SN change required procedure

| Company | 1)2) | Comments (please also comment the possible enhancement, e.g. new IE/cause value) |
| --- | --- | --- |
| Nokia | 1,2: yes | However, this would require new IEs in the SN Change, like “SN Change cancel/modify”, to let the MN know it is a modification of CPC, not a new CPC. |
| Lenovo, Motorola Mobility | 1)2) | For target SN release, there should be a new IE indicating releasing which target SN. Whether it can be a list of target SNs depending on the discussion whether one SN change procedure can support multiple SN preparation. |
| ZTE | Both yes |  |
| Huawei | Both yes, but | For 2) or it is also possible to use a new CPC cancel procedure, which can be a procedure applicable for bi-direction. |
| CATT | Both yes |  |
| NEC | 2) |  |
| Samsung | Both yes |  |
| E/// | Both yes | Pending on RAN2 discussion |

Moderator’s observation:

Clear majority to support both.

Proposal F: in SN initiated inter-SN CPC, source SN can:

- Update/modify previous CPC configurations provided in CPC preparation using SN change required procedure

- Cancel all prepared PSCells at target SN and release the target SN using SN change required procedure

Proposal G: New IEs are introduced in SN change required message indicating CPC configuration modification and target SN release.

Another issue is how does MN inform source SN about the triggered CPC cancel, partially or fully. For Xn interface a new class 2 procedure can be introduced. For X2 interface, since we reverted our previous agreement of using a class 2 procedure to inform source SN about the “CPC triggered” “CPC executed”, we may need to “re-introduce” the procedure for the purpose of CPC cancel.

Question F: Do companies agree in MN/SN initiated inter-SN CPC, MN can inform source SN about the triggered target SN release or some prepared PSCells cancellation at a target SN using:

- A new class2 XnAP procedure

- A new class2 X2AP procedure

| Company | 1)2) | Comments |
| --- | --- | --- |
| Nokia | 1,2: yes | This can be a WA for the time being. |
| Lenovo, Motorola Mobility | 1)2) |  |
| ZTE | Both yes |  |
| Huawei | Agree with Lenovo | And maybe this class 2 procedure can also be used by the source SN to trigger CPC cancel in case of SN initiated CPC. |
| catt | Both yes |  |
| NEC |  | FFS or WA. |
| Samsung | Yes for both |  |
| E/// | Yes to both |  |

Moderator’s observation:

Clear majority to support both.

Proposal H: in MN/SN initiated inter-SN CPC, MN can inform source SN about the triggered target SN release or some prepared PSCells cancellation at a target SN using:

- A new class2 XnAP procedure

- A new class2 X2AP procedure

4	For the Chairman’s Notes (Phase 1)

RAN3 issues which has RAN2 dependency:

RAN3 waits for RAN2 conclusion before discussing:

- May modify SN Addition Requestion Ack message to include a new inter-node RRC message containing full list of CG-Config(s) if introduced by RAN2

- May revisit previous RAN3 agreement “Introduce “List of Prepared PSCell IDs” in SN Addition Request ACK” if RAN2 agrees to provide prepared PSCells ID within the new inter-node RRC message but outside CG-Config

- For SN initiated CPC, if RAN2 agrees to support solution 2, RAN3 discusses the signaling design for MN to receive updated configuration from source SN before sending the RRC message to UE. May revisit previous RAN3 agreement “Introduce “List of Prepared PSCell IDs” in SN Change Confirm.”

CPAC terminology usage:

Proposal 1: RAN3 confirms the following use of different terms in principle:

  - “CPAC replace” means updating/modifying previously provided CPAC configuration before CPAC execution.

  - “Add prepared PSCells” means prepare extra PSCell(s) after CPAC is configured and before CPAC execution.

  - “CPAC cancel” at least means releasing previously prepared SN and relevant configuration. FFS if it covers the case of releasing some previously prepared PSCell(s) but keeping the SN.

CPA related:

Proposal 2: MN can trigger CPA replace and CPA cancel after CPA is configured.

Proposal 3: Target SN can add prepared PSCells within the limit given by the MN after CPA is configured.

MN initiated CPC related:

Proposal 4: MN can trigger CPC replace and CPC cancel after MN initiated inter-SN CPC is configured.

Proposal 5: Target SN can add prepared PSCells within the limit given by the MN after MN initiated inter-SN CPC is configured.

Proposal 6: In MN initiated inter-SN CPC, MN will inform source SN about the CPC cancel once triggered.

SN initiated CPC related:

Proposal 7: As a baseline, in SN initiated inter-SN CPC one SN change procedure can be used to prepare PSCells in one single target SN. FFS if one SN change procedure can be used for multiple SNs and may revisit upon RAN2 discussion.

Proposal 8: MN can trigger CPC replace and CPC cancel after SN initiated inter-SN CPC is configured.

Proposal 9: Source SN can trigger CPC replace and CPC cancel after SN initiated inter-SN CPC is configured.

Proposal 10: Target SN can add prepared PSCells within the limit given by the source SN after SN initiated inter-SN CPC is configured.

Proposal 11: In SN initiated inter-SN CPC, MN will inform source SN about the CPC cancel once triggered.

Others:

Proposal 12: Agree R3-214747, TP for CPAC BLCR to TS36.423, (i.e., X2AP class 2 Data Address Indication procedure is used for MN to inform the source SN about “CPC triggered” for MN initiated inter-SN CPC)

Proposal 13: X2AP class 2 Data Address Indication procedure is used for MN to inform the source SN about

- “CPC triggered” and ”CPC executed” for MN initiated inter-SN CPC

- “CPC executed” for SN initiated inter-SN CPC

Phase 2: to discuss CPAC replace/cancel signalling details, e.g., which/what message and IE.

5	Discussion (Phase 1)

3.1	RAN3 impacts considering RAN2 progress

In this section, moderator tries to analyse the possible RAN3 impacts considering RAN2 progress in their last meeting and discuss appropriate RAN3’s action accordingly.

In LS R3-214697, RAN2 informs RAN3 about the following agreements on RRC container design from candidate target SN to MN and execution condition handling at MN during SN initiated inter-SN CPC.

R3-214697: Inter-node RRC container design: RAN3 Question: - About the inter-node RRC container design - In case multiple PSCells are prepared in one CPAC procedure, RAN3 would like to ask RAN2 to feedback on the inter-node RRC container design: will one RRC container for one PSCell be used, or one RRC container for multiple PSCells? RAN2 feedback: RAN2 has concluded that the configuration of multiple PSCell candidates can be included within a single RRC inter-node message during CPAC procedures from the candidate target SN to the MN. Execution conditions at SN initiated inter-SN CPC RAN3 Question: - About the SN initiated inter-SN CPC, RAN3 would like to ask RAN2 to feedback on the following two alternatives: - Alternative 1: MN performs the association between the execution condition received from the source SN and the RRC configuration of the candidate PSCell received from the candidate SN. - Alternative 2: MN forwards the execution condition received from the source SN to the candidate SN. The candidate SN sends the execution condition and the RRC configuration of the candidate PSCell to the MN. RAN2 feedback: RAN2 has concluded that the MN associates the execution condition configuration to an RRCReconfiguration message provided by the target SN. The MN does not need to comprehend the execution condition set by the source SN. FFS if T-SN is informed of the execution conditions.

As agreed by RAN2, one candidate target SN will send one single RRC container to MN including the configuration of multiple PSCell candidates. In the meanwhile, more detailed RRC signalling design is still under discussion in RAN2 [1], e.g., if a new inter-node RRC message is needed and if the accepted target PSCell IDs will be provided outside CG-Config but within the same RRC inter-node message from target SN to MN.

In the email discussion report [1] which will be discussed in the ongoing RAN2#116e meeting, the following proposals are suggested reflecting majority companies’ view.

R2-2109871 Report for Inter-node message design: Proposal 1: Introduce a new inter-node RRC message that includes the full list of CG-Config(s). Proposal 2: Specify the accepted target PSCell identity (frequency and PCI) outside the corresponding CG-Config in the new inter-node message. Proposal 3: Send an LS to RAN3 to inform about the new inter-node RRC message that includes a full list of CG-Config(s), and the corresponding impact to RAN3 specification.

With respect to RAN3 impact, in moderator’s understanding, three aspects related to Xn message design could be worth checking.

Issue 1: New inter-node RRC message added to SN Addition Request Acknowledge

First, if a new inter-node RRC message (e.g., CG-CandidateList) is introduced to include the full list of CG-Config(s), the new message will be added as another option for S-NG-RAN node to M-NG-RAN node Container, for instance as proposed in [2]:

9.1.2.2 S-NODE ADDITION REQUEST ACKNOWLEDGE This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation. Direction: S-NG-RAN node M-NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. M-NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the M-NG-RAN node; YES; reject. S-NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the S-NG-RAN node; YES; reject. [...]. S-NG-RAN node to M-NG-RAN node Container; M; OCTET STRING; Includes the CG-Config message or the CG-CandidateList defined in subclause 11.2.2 of TS 38.331 [10].; YES; reject. [...].

Issue 2: The need of “List of Prepared PSCell IDs” in SN Addition Request Acknowledge

Secondly, in the previous RAN3 meeting, RAN3 agreed to introduce “List of Prepared PSCell IDs” in SN Addition Request ACK message let MN be aware of the prepared PSCells IDs by reading Xn message without opening the RRC container.

Introduce “List of Prepared PSCell IDs” in SN Addition Request ACK.

At the same time, RAN2 agreed, as in the LS, it is MN that associates the execution condition configuration to an RRCReconfiguration message provided by the target SN, which implies the MN may anyway open the RRC container to associate the execution condition and SN RRC reconfiguration. Besides, to reduce the complexity and avoid reading the CG-Config, [2] also proposes to specify the accepted target PSCell identity (frequency and PCI) outside the corresponding CG-Config in the new inter-node message (e.g., CG-CandidateList). As such, MN does not need to open the CG-Config to understand the prepared PSCells. The RRC message may look like following as discussed in [2]

CG-CandidateList-IEs ::= SEQUENCE { cg-CandidateList-r17 SEQUENCE (SIZE (1..FFS)) OF CG-CandidateInfo-r17 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } CG-CandidateInfo-r17 ::= SEQUENCE { ssbFrequency ARFCN-ValueNR, physCellId PhysCellId, candidateCG-Config OCTET STRING (CONTAINING CG-Config) }

Considering RAN2’s agreement and the inter-node RRC message design proposals, from moderator’s perspective, if RAN2 agrees that the prepared PSCell IDs will be contained in the inter-node RRC message but outside CG-Config, is it still necessary to include the “List of Prepared PSCells IDs” in the part of Xn message as agreed by RAN3 before. Both designs are aiming to reduce the complexity at MN in similar manner, which seems a bit replicated.

Issue 3: Signaling exchange between MN and Source SN before configuring UE

For SN initiated inter-SN CPC, RAN2 agreed a working assumption in their last meeting to support solution 2, i.e., MN will receive updated configuration from source SN before sending the RRC message to UE [3]. An exemplary procedure is provided in Figure 1.

RAN2#115e Agreement Working assumption: We go for solution 2. Should make sure multiple re-negotiation procedures (i.e. two nested procedures or anything that requires negotiation cannot be used) is not allowed. Inform RAN3 and take their feedback into account.

Figure 1: Exemplary procedure for solution 2 (modified from [4])

In the exemplary procedure for solution 2, MN uses message 4 to inform source SN about the accepted candidate PSCells, then source SN uses message 5 to update the source SN configuration (e.g., execution condition), and MN will inform source SN about SN RRC complete using message 8. Depending on RAN2 progress, if solution 2 is agreed, what exact Xn messages are message 4, 5, 8 is worth RAN3 discussion. There seems to be two possible options:

- Option 1:

  - Message 4: SN change confirm

  - Message 5: SN modification required

  - Message 8: SN modification confirm

- Option 2:

  - Message 4: SN modification request

  - Message 5: SN modification response

  - Message 8: SN change confirm

In addition, if RAN3 agrees with Option 2 above, the following RAN3 agreement shall be revisited. Since if Option 2 is adopted, the list of prepared PSCells IDs is conveyed in Message 4 SN modification request instead of SN Change Confirm.

RAN3#113e Agreement: - Introduce “List of Prepared PSCell IDs” in SN Change Confirm.

In the following question, to ensure companies to be on the same page, companies are asked to confirm the possible RAN3 impact which has RAN2 dependency as analyzed above. RAN3 can wait for RAN2 conclusion before further discussion, i.e., RAN3 will not discuss the relevant issues in this meeting.

Question 1: To align companies understanding, companies are kindly asked to confirm if the following RAN3 aspects depend on RAN2 progress and thus shall wait for RAN2 conclusion first.

- May modify SN Addition Request Ack message to include a new inter-node RRC message containing full list of CG-Config(s) if introduced by RAN2

- May revisit previous RAN3 agreement “Introduce “List of Prepared PSCell IDs” in SN Addition Request ACK” if RAN2 agrees to provide prepared PSCells ID within the new inter-node RRC message but outside CG-Config

- For SN initiated CPC, if RAN2 agrees to support solution 2, RAN3 discusses the signaling design for MN to receive updated configuration from source SN before sending the RRC message to UE. May revisit previous RAN3 agreement “Introduce “List of Prepared PSCell IDs” in SN Change Confirm.”

| Company | 1)2)3) | Comments |
| --- | --- | --- |
| ZTE | Agree with 1,2 | For 3, so far it is WA other than agreement in RAN2, so RAN3 can wait for RAN2 progress until it is agreement and stable. |
| Nokia | Agree with 1,2,3, but details are FFS | As written, the general principles of all 3 points are good and need to be addressed. But details (e.g. if a new container is needed, or existing reused) are to be discussed further. |
| Lenovo, Motorola Mobility | Agree 1)2)3) | RAN3 does not need to discuss the relevant issues as in 1) 2) 3), since they are dependent on RAN2 conclusion. |
| Google | OK for 1,2,3 but details are FFS | The previous RAN3 and RAN2 discussions seem to interlace with each other so that RAN2 made some decision without considering the agreed RAN3 BLCRs. It would be better to capture stable conclusions from both sides then. |
| Huawei | Ok for 1,2,3 |  |
| E/// | In principle agree with 1, 2, 3 | We don’t preclude any solution in RAN3 before RAN2 reaches the conclusion. For example, in 1) whether a new inter-node message is required or simply a new reference is added to the existing RRC message. |
| CATT | Agree with 1,2,3 | We don’t need discuss them before RAN2 lockdown it |
| NEC |  | 1) we understand that the received RAN2 answer in LS (R3-214679) showing the RAN2 agreed to introduce a single RRC container including multiple PSCells candidates, therefore thought at least this can be reflected in baseline CR (because it is only a baseline), but if RAN2 is still discussing, then wait for RAN2 further conclusion is probably OK. 2) it is important to avoid MN complexity to dig into the RRC container, the list of prepared PScell IDs that is visible in Xn/X2 level is preferable. 3) can wait RAN2 to conclude. |
| LGE | Agree with 1,2,3 | Details can be checked further when RAN2 achieves further progress |
| Qualcomm | Agree with 1, 2 | Wait for RAN2 decision on 3. |
| Samsung | Agree with 1,2,3 | We can wait for RAN2’s further discussion |

Moderator’s observation:

It seems common understanding among clear majority the following issues may depend on RAN2 discussion thus shall wait for RAN2 conclusion first.

  - May modify SN Addition Request Ack message to include a new inter-node RRC message containing full list of CG-Config(s) if introduced by RAN2

  - May revisit previous RAN3 agreement “Introduce “List of Prepared PSCell IDs” in SN Addition Request ACK” if RAN2 agrees to provide prepared PSCells ID within the new inter-node RRC message but outside CG-Config

  - For SN initiated CPC, if RAN2 agrees to support solution 2, RAN3 discusses the signaling design for MN to receive updated configuration from source SN before sending the RRC message to UE. May revisit previous RAN3 agreement “Introduce “List of Prepared PSCell IDs” in SN Change Confirm.”

Since this question is more to align companies understanding, no real proposal is needed.

3.2 Issues related to CPA

NOTE: Moderator observes some mixed use of “CPAC replace” and “Add prepared PSCells” which is sometimes confusing. Thus, it could be beneficial to clarify the difference between “CPAC replace” and “Add prepared PSCells”. In this email discussion, inheriting the spirit from last meeting discussion [13], moderator assumes:

  - “CPAC replace” means updating/modifying previously provided CPAC configuration before CPAC execution.

  - “Add prepared PSCells” means prepare extra PSCell(s) after CPAC is configured and before CPAC execution.

  - “CPAC cancel” means cancelling previously prepared PSCell(s) and relevant configuration.

In moderator’s understanding, in order to change a prepared PSCell to another, one needs to trigger “Add prepared PSCells” and “CPAC cancel” at the same time. “CPAC replace” serves different purpose than changing a prepared PSCell to another.

It’s suggested to take above definition for the sake of discussion for the time being, and RAN3 can discuss later if “Add prepared PSCells” and “CPAC cancel” can be considered as part of “CAPC replace” and the relevant signaling design.

Phase 1 discussion focuses on CPAC principles while the exact signaling design is left to phase 2 depending on the progress.

In the last meeting, RAN3 agreed that for CPA, target SN can trigger replace and cancel of prepared PSCells in the T-SN.

For CPA, the T-SN can trigger replace and cancel of prepared PSCells in the T-SN.

In the submitted contributions [5][6][7][8][9], companies believe MN can also trigger replace and cancel of prepared PSCells in the target SN. Besides, [6] also believes that after CPA configuration target SN can trigger adding prepared PSCells within the limit given by MN. However, [12] thinks allowing target SN to add extra prepared PSCells during CPA replace is unnecessary and only adds complexity.

[10][14] believe MN can also trigger the procedure to add prepared PSCell(s), however, in moderator’s understanding MN can only modify the list of suggested candidate PSCell(s) as part of CPAC replace, while technically MN cannot add prepared PSCell(s). Maybe some clarification from companies is needed.

Question 2: Companies are kindly asked if MN can trigger the following after CPA is configured?

- CPA replace (i.e., modifying previously provided CPA related configurations)

- CPA cancel

- Add prepared PSCells (Please clarify how it is done)

| Company | 1)2)3) | Comments |
| --- | --- | --- |
| ZTE | Yes for 1) and 2) | For 1) the MN can modify the “Maximum Number of PSCells To Prepare” IE via MN initiated SN modification procedure. For 2), if MN decides to cancel all of CPA, then the legacy MN initiated SN release procedure can be reused without enhancement. |
| Nokia | 1 (partially) and 2 (partially) | The red line is that the MN can’t decide on particular PSCells – this is up to the SN (like in the addition, where it decides which PSCells are to be prepared). With this in mind we understand that 1 and 2 are acceptable: 1) The MN may modify the preparation condition, e.g. provide new measurements from the UE or new max number of cells to prepare. 2) The MN may force the SN to release some preparations by limiting the max number of PSCells to prepare. It may also release completely CPA to the target SN. |
| Lenovo, Motorola Mobility | Yes 1)2) | Whether all configurations, which has been configured by MN before, can be modified in CPA replace can be discussed in phase 2. CAP cancel should be allowed, and how to use SN modification/release message can be further discussed in phase 2. |
| Google | Yes for 1, 2 | Also agree with Nokia’s comments |
| Huawei | Yes for 1,2 No for 3 |  |
| E/// | Yes for 1, 2 |  |
| CATT | Yes 1,2 |  |
| LGE | Yes for 1, 2 | On 3, the intention was not to decide and add extra PSCell by MN. According to the newly received measurement report, MN can suggest/change other potential Scells, which for sure will be decided finally by SN on whether to be the extra new PSCell. That is, moderator’s understanding is correct, MN can only modify the list of suggested candidate PSCell(s) as part of CPAC replace. |
| Qualcomm | Yes for 1, 2 |  |
| Samsung | Yes for 1, 2 |  |

Moderator’s observation:

To clarify the definition of CPAC replace and address concern from some companies, it’s suggested RAN3 to agree on the meaning of each term (i.e., CPAC replace is only about updating the previously provided configuration, which will automatically mean MN will not decide on particular PSCell during CPAC replace).

Proposal 1: RAN3 confirms the following use of different terms:

  - “CPAC replace” means updating/modifying previously provided CPAC configuration before CPAC execution.

  - “Add prepared PSCells” means prepare extra PSCell(s) after CPAC is configured and before CPAC execution.

  - “CPAC cancel” means cancelling previously prepared PSCell(s) and relevant configuration.

All companies agree MN can trigger CPA replace and cancel.

Proposal 2: MN can trigger CPA replace and CPA cancel after CPA is configured.

Question 3: Companies are kindly asked if target SN can add prepared PSCells within the limit given by the MN after CPA is configured?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia | Yes |  |
| Lenovo, Motorola Mobility | Yes |  |
| Google | Yes |  |
| Huawei | Yes |  |
| E/// | Yes |  |
| CATT | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |

Moderator’s observation:

All companies agree.

Proposal 3: Target SN can add prepared PSCells within the limit given by the MN after CPA is configured.

3.3 Issues related to MN initiated inter-SN CPC

In the last meeting, RAN3 agreed that for MN initiated inter-SN CPC, target SN can trigger replace and cancel of prepared PSCells in the T-SN.

For MN initiated inter-SN CPC, the T-SN can trigger replace and cancel of prepared PSCells in the T-SN, while the S-SN cannot trigger replace and cancel.

Similar as the discussion for CPA replace, some companies [7][8][10] think MN can also trigger CPC replace and cancel in case of MN initiated inter-SN CPC.

Question 4: Companies are kindly asked if MN can trigger the following after MN initiated inter-SN CPC is configured?

- CPC replace

- CPC cancel

- Add prepared PSCells (please clarify how it is done)

| Company | 1)2)3) | Comments |
| --- | --- | --- |
| ZTE | Yes for 1) and 2) | Same as question 2 |
| Nokia | Same as Q2 | Same as Q2. In general, the MN to target SN communication in case of CPC shall follow the same principles as CPA. |
| Lenovo, Motorola Mobility | Yes 1)2) |  |
| Google | Same as Q2 | Same as Q2 |
| Huawei | Same as Q2 |  |
| E/// | Yes for 1, 2 |  |
| CATT | Yes 1,2 |  |
| LGE | Yes for 1, 2 | On 3, the intention was not to decide and add extra PSCell by MN. According to the newly received measurement report, MN can suggest/change other potential Scells, which for sure will be decided finally by SN on whether to be the extra new PSCell. That is, moderator’s understanding is correct, MN can only modify the list of suggested candidate PSCell(s) as part of CPAC replace. |
| Qualcomm | Yes 1, 2 |  |
| Samsung | Yes for 1, 2 | Same as Q2 |

Moderator’s observation:

All companies agree with 1) and 2)

Proposal 4: MN can trigger CPC replace and CPC cancel after MN initiated inter-SN CPC is configured.

Question 5: Companies are kindly asked if target SN can add prepared PSCells within the limit given by the MN after MN initiated inter-SN CPC is configured?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia | Same as Q2 | Same as Q2. In general, the MN to target SN communication in case of CPC shall follow the same principles as CPA. |
| Lenovo, Motorola Mobility | Yes |  |
| Google | Yes |  |
| Huawei | Yes |  |
| E/// | Yes |  |
| CATT | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |

Moderator’s observation:

All companies agree.

Proposal 5: Target SN can add prepared PSCells within the limit given by the MN after MN initiated inter-SN CPC is configured.

One issue raised by companies is, in case the CPC is cancelled either by target SN or MN (if supported), shall MN inform source SN about it.  [6][10][12] believe it could be beneficial since source SN will at least cancel the ongoing early data forwarding.

Question 6: Companies are kindly asked that in MN initiated inter-SN CPC does MN need to inform SN about the CPC cancel triggered by target SN or MN (if supported)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | The SN shall be notified, then early data forwarding if configured can be stopped by this indicator |
| Nokia | Yes | This can be left FFS, but seems needed, e.g. to stop data forwarding. |
| Lenovo, Motorola Mobility | Yes |  |
| Google | Yes | In case of early data forwarding, this seems needed |
| Huawei | Yes | The source SN needs to be informed about CPC cancel, to stop data forwarding. Not know if the question is also related to inform the source SN about whether the CPC cancel is triggered by target or MN, do not see the need for the source SN to distinguish it. |
| E/// | Yes | For next step |
| CATT | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |

Moderator’s observation:

All companies agree.

Proposal 6: In MN initiated inter-SN CPC, MN will inform source SN about the CPC cancel once triggered.

3.4 Issues related to SN initiated inter-SN CPC

In the previous meeting, RAN3 has made the following working assumption.

WA: In case of SN initiated inter-SN CPC, prepare multiple PSCells in one target SN by one SN Change procedure is the baseline.

However, it is still under debate if that is the best solution since one SN change procedure preparing single target SN would imply parallel preparations for multiple target SNs is allowed. [2] thinks allowing parallel preparation procedures for different target SNs will delay the whole procedure by waiting for the acknowledge messages from all the target SNs. In addition, the interaction of open parallel procedures will cause complexity to the state machine and error handling of network node. [15] thinks one SN change procedure can only prepare one target SN. [11] suggests to first support preparation of single T-SN in SN initiated inter-SN CPC first to progress, and then discuss how to prepare multiple T-SNs as second priority.

Question 7: Companies are kindly asked in SN initiated inter-SN CPC one SN change procedure can be used to prepare PSCells in one single target SN or multiple target SNs?

- One single target SN

- Multiple target SNs

- RAN3 considers 1) for the time being

| Company | 1)2)3) | Comments |
| --- | --- | --- |
| ZTE | 2) | Based on the WA from RAN2, the option 2) is better. In the previous RAN3 meetings, we worried about the complicated early data forwarding, but if RAN2’s WA changed to agreement, the data forwarding issue is not existed. In detail, as seen in figure1, the early data forwarding is handled after step 5. So that, multiple target SNs configured in one SN change required message is more efficient than many SN change required message. |
| Nokia | 3 | So far, option 1 seems easier from RAN3 perspective, so we would prefer to keep it. We can come back to this question once RAN2 confirms the WA. |
| Lenovo, Motorola Mobility | 3 | Similar view as Nokia, if RAN2 supports solution 2, option 2 seems better. |
| Google | 3 |  |
| Huawei | 3 | As proposed in [11]. |
| E/// | 2) | Share similar view as ZTE. 3) as a compromise may lead to duplicated functionalities for support of SN-initiated CPC. |
| CATT | 3 |  |
| NEC | 1) or 3) | For the Multiple Target SN preparation in parallel, since the complexity especially for the error handling is foreseen, e.g. waiting but no response from one T-SN within time expiry, therefore we think we have a simple solution i.e. one shot aiming one target, no multiple targets. |
| LGE | 3 |  |
| Qualcomm | 2） | 2) is more flexible. 1) can be a special case of 2). |
| Samsung | 3 |  |

Moderator’s observation:

1) 1

2) 3

3) 8

Some companies also think the complexity of supporting 1) may dependent on if RAN2 agrees on solution 2. Moderator would propose 3) but think it could also be fine to wait for RAN2 conclusion first.

Proposal 7: As a baseline, in SN initiated inter-SN CPC one SN change procedure can be used to prepare PSCells in one single target SN. FFS if one SN change procedure can be used for multiple SNs and may revisit upon RAN2 discussion.

In the previous meeting, RAN3 agreed the following

For SN initiated inter-SN CPC, the T-SN can trigger replace and cancel of prepared PSCells in the T-SN.

Similar as the case for CPA and MN initiated inter-SN CPC, the following questions are asked regarding CPC replace/cancel for SN initiated inter-SN CPC.

Question 8: Companies are kindly asked if MN can trigger the following after SN initiated inter-SN CPC is configured?

- CPC replace

- CPC cancel

- Add prepared PSCells (please clarify how it is done)

| Company | 1)2)3) | Comments |
| --- | --- | --- |
| ZTE | Maybe 1) and 2) based on RAN2 WA. | We suggest to discuss SN initiated CPC as low priority because RAN2’s WA is not stable. For 1) the MN can modify the “Maximum Number of PSCells To Prepare” IE via MN initiated SN modification procedure. For 2), if MN decides to cancel all of CPA, then the legacy MN initiated SN release procedure can be reused without enhancement. |
| Nokia | Same as Q2 | Same as Q2. In general, the MN to target SN communication in case of CPC shall follow the same principles as CPA. Please note, even in case of SN-initiated CPC, the MN may have own reasons to cancel DC operation towards the target SN. Then, for (1) and (2), the MN must be allowed to transfer requests from the source SN. |
| Lenovo, Motorola Mobility | Yes 1)2) | Don’t think this is relevant to RAN2 solution 2 discussion, but we are also fine to postpone if majority companies believe so. |
| Google | Same as Q2 | Same as Q2 |
| Huawei | Yes 1, 2 No 3 | Same as Q2. |
| E/// | Yes for 1, 2 |  |
| CATT | Yes 1,2 |  |
| LGE | Yes 1, 2 | On 3, the intention was not to decide and add extra PSCell by MN. According to the newly received measurement report, MN can suggest/change other potential Scells, which for sure will be decided finally by SN on whether to be the extra new PSCell. |
| Qualcomm | Yes for 1, 2 |  |
| Samsung | Yes for 1, 2 | Same as Q2. |

Moderator’s observation:

All companies agree with 1) and 2)

Proposal 8: MN can trigger CPC replace and CPC cancel after SN initiated inter-SN CPC is configured.

Question 9: Companies are kindly asked if source SN can trigger the following after SN initiated inter-SN CPC is configured?

- CPC replace

- CPC cancel

- Add prepared PSCells (please clarify how it is done)

| Company | 1)2)3) | Comments |
| --- | --- | --- |
| ZTE | Maybe 1) and 2) based on RAN2 WA. | We suggest to discuss SN initiated CPC as low priority because RAN2’s WA is not stable. |
| Nokia | 1 (partially) and 2 (partially) | The red line is that the source SN can’t decide on particular PSCells – this is up to the target SN (like in the classic SN change, where it decides which PSCells are to be prepared). With this in mind we understand that 1 and 2 are acceptable: 1) The source SN may modify the preparation condition, e.g. provide new measurements from the UE or new max number of cells to prepare. 2) The source SN may force the target SN to release some preparations by limiting the max number of PSCells to prepare. It may also ask the MN to release completely CPC to the target SN. |
| Lenovo, Motorola Mobility | Yes 1)2) | Don’t think this is relevant to RAN2 solution 2 discussion, but we are also fine to postpone if majority companies believe so. |
| Google | 1, 2 (maybe) | Agree with Nokia |
| Huawei | Yes 1) 2) No 3) | Agree with Nokia’s analyses. |
| E/// | Yes for 1, 2 | For next step |
| CATT | Yes 1, 2 |  |
| LGE | Yes for 1, 2 | On 3, the intention was not to decide and add extra PSCell by Source SN. According to the newly received measurement report, Source SN can suggest/change other potential Scells, which for sure will be decided finally by SN on whether to be the extra new PSCell. In Q7, we have “WA: In case of SN initiated inter-SN CPC, prepare multiple PSCells in one target SN by one SN Change procedure is the baseline.” Source SN may change the suggested target SNs with time going, thus the potential scells in the target may be changed. Final decision can be done by the target SN. |
| Qualcomm | Yes for 1, 2 |  |
| Samsung | Yes for 1, 2 |  |

Moderator’s observation:

All companies agree with 1) and 2)

Proposal 9: Source SN can trigger CPC replace and CPC cancel after SN initiated inter-SN CPC is configured.

Question 10: Companies are kindly asked if target SN can add prepared PSCells within the limit given by the source SN after SN initiated inter-SN CPC is configured?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes based on RAN2 WA. | We suggest to discuss SN initiated CPC as low priority because RAN2’s WA is not stable. |
| Nokia | Yes | We’re fine to have it as FFS |
| Lenovo, Motorola Mobility | Yes | Don’t think this is relevant to RAN2 solution 2 discussion, but we are also fine to postpone if majority companies believe so. |
| Google | Yes |  |
| Huawei | Yes |  |
| E/// | Yes |  |
| CATT | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |

Moderator’s observation:

All companies agree.

Proposal 10: Target SN can add prepared PSCells within the limit given by the source SN after SN initiated inter-SN CPC is configured.

Question 11: Companies are kindly asked that in SN initiated inter-SN CPC does MN need to inform SN about the CPC cancel triggered by target SN or MN (if supported)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia | Yes |  |
| Lenovo, Motorola Mobility | Yes |  |
| Google | Yes |  |
| Huawei | Yes | Same to Q6. The source SN needs to be informed about CPC cancel, to stop data forwarding. Not know if the question is also related to inform the source SN about whether the CPC cancel is triggered by target or MN, do not see the need for the source SN to distinguish it. |
| E/// | Yes |  |
| CATT | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |

Moderator’s observation:

All companies agree.

Proposal 11: In SN initiated inter-SN CPC, MN will inform source SN about the CPC cancel once triggered.

3.5 Other issues

Another issue raised by couple of companies [16] is that, even though RAN3 made the following agreement to introduce new X2AP message to let MN inform source SN about “CPC triggered”/”CPC executed”, in another RAN3 discussion (see R3-212741) about CHO in EN-DC it has been agreed that in the case of MN to eNB/gNB Change, for CHO with EN-DC, the existing X2AP Data Forwarding Address Indication procedure is used to initiate/stop early data forwarding. Thus, [16] suggests to revert previous agreement and follow the principle from CHO in EN-DC, i.e., using the existing X2AP class 2 Data Address Indication procedure from MN to inform the source SN about “CPC triggered”.

In case of MN initiated inter-SN CPC, introduce new X2AP class 2 procedure from MN to inform the source SN about “CPC triggered”. In case of SN initiated inter-SN CPC, using a class 2 procedure in both X2AP and XnAP to indicate “CPC executed”. For X2, a new class2 procedure is introduced.

Although [16] focuses on “CPC triggered”, moderator believes the same issue applies to “CPC executed” indication over X2AP.

Question 12: Companies are kindly asked if X2AP class 2 Data Address Indication procedure can be used for MN to inform the source SN about

- “CPC triggered” and ”CPC executed” for MN initiated inter-SN CPC

- “CPC executed” for SN initiated inter-SN CPC

| Company | 1)2) | Comments |
| --- | --- | --- |
| ZTE | Yes for 1) and 2) | We agree with both |
| Nokia | Yes for both |  |
| Lenovo, Motorola Mobility | Yes 1)2) |  |
| Google | Yes for both |  |
| Huawei | No or neutral. | Prefer to keep previous agreement, as the data forwarding address related IEs are mandatory IEs, which are not needed in these CPC scenarios. |
| E/// | Yes | Assume the same IE is used for the above 2. |
| CATT | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes for both |  |
| Samsung | Yes for both |  |

Moderator’s observation:

There is clear majority to use X2AP class 2 Data Address Indication for CPC triggered and CPC executed information transfer.

Proposal 12: X2AP class 2 Data Address Indication procedure is used for MN to inform the source SN about

- “CPC triggered” and ”CPC executed” for MN initiated inter-SN CPC

- “CPC executed” for SN initiated inter-SN CPC

In the last meeting, RAN3 discussed the early data forwarding in CPAC and has the following agreements and FFS:

For PDCP SDU Forwarding and discarding:

Reusing the IEs within the First DL COUNT branch in the EARLY STATUS TRANSFER message.

Reusing the existing IEs in the DL Discarding branch in the EARLY STATUS TRANSFER message.

Rxtending the EARLY STATUS TRANSFER message to the following cases: from the source SN to the MN, and from the MN to the candidate SNs.

For PDCP PDU Forwarding

- FFS: the node hosting PDCP entity does not need to send the first DL count to the corresponding node.

- FFS: that the node receiving the forwarded DL PDCP SDUs can forward the DL PDCP PDUs to other nodes in early data forwarding.

For PDCP PDU Forwarding, it is FFS how to inform the discarding of DL PDCP PDU SNs:

Option 1: user plane solution, i.e. reuse the DL USER DATA frame

Option 2: control plane solution, i.e. the early status transfer message

Question 13: Companies are kindly asked whether to use User plane solution or control plane solution to inform the discarding of DL PDCP PDU SNs, for PDCP PDU forwarding.

| Company | 1)2) | Comments |
| --- | --- | --- |
| Huawei | 2) | Slightly prefer2), 1) is also acceptable. |
| CATT | 2) |  |

Moderator’s observation:

Since only one companies discussed this issue in their contribution paper, moderator suppose companies want more time to think about it thus didn’t include in the phase 1 discussion. If companies want, we can quickly check in Phase 2, otherwise we can discuss next meeting based on contributions.

6	Conclusion

Reference

[1] R2-2109871	Report for Inter-node message design	Ericsson

[2] R3-214767	(TP for CPAC BL CR to TS 38.423) Inter-node message and Support of SN-initiated CPC	Ericsson

[3] R2-2108831 RAN2-115-e LTE 71 GHz DCCA Multi-SIM and RAN slicing (Tero)_2021-08-27-EOM2

[4] R2-2103109 Summary of [Post113-e][234][eDCCA] CPAC procedures (CATT)

[5] R3-214748	(TP for CPAC BL CR to TS 37.340) Remaining issues for MN initiated CPC ZTE

[6] R3-214809	(TP to CPAC BL CR to 38.423, LTE_NR_DC_enh2-Core) Max number of PSCells, cancellation and modification of a CPAC	Nokia, Nokia Shanghai Bell

[7] R3-214920	CPAC replace procedures	Qualcomm Incorporated

[8] R3-215328	Left issues on CPAC 	Lenovo, Motorola Mobility

[9] R3-215527	Open issues on CPAC replace and cancel 	LG Electronics

[10] R3-214749	(TP for CPAC BLCR to TS36.423/TS38.423) CPA and CPC replace and cancel	ZTE

[11] R3-215109	(TP to 37.340 CPAC BL CR) Support of inter-SN CPC	Huawei

[12] R3-215113	Further Considerations on Conditional PSCell ChangeAddition	CATT

[13] R3-214304 Summary of Offline Discussion on MRDC3_CPAC Huawei

[14] R3-215527	Open issues on CPAC replace and cancel	LG Electronics

[15] R3-214943	(TP for CPAC BL CR to TS 36.423 ) On CPAC (Conditional PSCell Addition and PSCell Change)	NEC

[16] R3-214747	(TP for CPAC BLCR to TS36.423) Reusing X2AP Data Forwarding Address Indication procedure for Early data forwarding	ZTE, Qualcomm Incorporated, Lenovo, Ericsson

[17] R3-215110 (TPs to 38.423 36.423 38.473 38.425 CPAC BL CRs) PDCP PDU forwarding and discard		Huawei

[18] R3-214768	(TP for CPAC BL CR to TS 38.423) CPC/CPA Cancellation	Ericsson

Annex