3GPP TSG-RAN WG3 #114-e																		   R3-215972

1-11 Nov 2021

Online

Agenda Item:	8.1

Source:	ZTE

Title:	(TP for BL CR for 38.413) Multicast group paging

Document for:	Approval

- Introduction

Based on the result of following CB, we provide the corresponding TP for TS38.413.

CB: # 8_MBSPaging

- RAN3 confirm RAN2 understanding? Stage3 TP?

- Reply LS to RAN2

(ZTE - moderator)

[NWM] Summary of offline disc R3-215811

- Text Proposal for TS36.413

----------Start of the First Change--------------

### 8.5.X	Multicast Group Paging

#### 8.5.X.1	General

The purpose of the Multicast Group Paging procedure is to enable the AMF to notify CM-IDLE UEs which have joined an MBS Session about its activation.

The procedure uses non-UE associated signalling.

#### 8.5.X.2	Successful Operation

Figure 8.5.X.2-1: Multicast Group Paging

The AMF initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the NG-RAN node.

At the reception of the MULTICAST GROUP PAGING message, the NG-RAN node shall perform multicast group paging of the MBS Session identified by the MBS Session ID IE utilising information provided by the AMF.

If the Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 38.304 [12].

Editor’s Note:	Detailled parameterisation of the message is FFS.

#### 8.5.1.3	Abnormal Conditions

Void.

*****************Next changes*******************

#### 9.2.4.X	MULTICAST GROUP PAGING

This message is sent by the AMF to trigger multicast group paging for a multicast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.x |  | YES | ignore |
| MBS Service Area | M |  | (FFS) | Editor’s Note: The type definition of this IE is FFS. | YES | ignore |
| Multicast Paging Area | O |  | 9.3.1.y | Editor’s Note: The presence and the usage of this IE is type definition of this IE is FFS |  |  |
| Paging Priority (FFS) | O |  | 9.3.1.78 | Editor’s Note: The optional inclusion of this IE. | YES | ignore |
| UE Paging List |  | 0..1 |  |  | YES | ignore |
| >UE Paging Item |  | 1..<maxnoofUEsforPaging> |  |  | - |  |
| >> UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| >>Paging DRX | O |  | 9.3.1.90 | Editor’s Note: It is FFS that this IE is per UE or per MBS session. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEsforPaging | Maximum no. of UEs for paging. Value is FFS. |

9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

#### 9.3.1.y	Multicast Paging Area

This IE is a list of TAIs corresponding to the multicast paging area.

Editor’s Note: The details of the IE is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAI List for Paging |  | 1..<maxnoofTAIforPaging> |  |  |
| >TAI | M |  | 9.3.3.11 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is FFS. |

----------End of the Change--------------