3GPP TSG-RAN3 Meeting #114-e	R3-215980

E-meeting, 1 -11 Nov 2020

Title: 	(TP for eIIOT BLCR for TS 38.473) Survival time remaining issues

Source: 	Huawei

Agenda item:	21.3

Document Type:	Discussion

- Introduction

This TP for TS 38.473 is to capture the agreements of CB # NRIIOT2_NewQoS.

2. TP for 38.473 on top of R3-213175

Change Begins

9.3.1.142	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [21]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.143 | Periodicity as specified in TS 23.501 [21]. | - |  |
| Burst Arrival Time | O |  | 9.3.1.144 | Burst Arrival Time as specified in TS 23.501 [21]. | - |  |
| Survival Time | O |  | 9.3.1.aaa | Survival Time as specified in TS 23.501 [21]. | YES | ignore |

<Unchanged Text Omitted>

9.3.1.aaa	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Expressed in units of 1 us. |

<Unchanged Text Omitted>