3GPP TSG-RAN WG3 Meeting #114-e	R3-216150

E-meeting, 1-11 Nov 2021

Title: 	Stage 2 TP to 38.300 BL CR on QoE

Source: 	ZTE, Huawei, Nokia, Ericsson, Qualcomm, Samsung

Agenda item:	15.2.1.1

Document Type:	Discussion and Approval

Introduction

This document provides text proposal to TS38.300 BL CR for stage 2 descriptions of NR QoE, based on current agreements in RAN3.

References

Text Proposal to TS38.300

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

<unchanged omitted>

LDPC	Low Density Parity Check

MCE	Measurement Collection Entity

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MT	Mobile Termination

<unchanged omitted>

TA	Timing Advance

TCE	Trace Collection Entity

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

X	 QoE Measurement Collection

  - X.1	Overview

The QoE Measurement Collection function enables collection of application layer measurements from the UE.

The measurements are supported for the following service types:

-	Streaming services.

-	MTSI services.

-	VR services.

Both signalling-based and management-based QoE measurement collection are supported.

X.2	 Configuration Aspects

X.2.1	General

X.2.2 RAN overload QoE handling

QoE Measurement Collection pause/resume procedure is used to pause/resume the reporting for all QoE reports or to pause/resume QoE reporting per QoE configuration in a UE in case of RAN overload.

X.2.3 QoE Measurement handling in RRC_IDLE and RRC_INACTIVE

In this version of the specification, the UE autonomously releases the QoE measurement configuration when it enters the RRC_IDLE mode.

Editor’s Note: The structure of TS 38.300 BLCR needs further discussion, especially this section.

  - X.3	 QoE Measurement Collection Activation and Reporting

The feature is activated in the NG-RAN, either by direct configuration from the OAM system (management-based activation), or by signalling from the OAM via the CN (signalling-based activation), using UE-associated siganlling. One or more QoE measurement collection jobs can be activated at a UE per service type. The UE reports the QoE measurement results to the NG-RAN node in uplink RRC message(s), and the NG-RAN node transfers the received QoE measurement report(s) to the MCE.

For signalling-based QoE, the OAM initiates the QoE measurement activation for a specific UE via the Core Network, and the NG-RAN node receives the QoE measurement configuration by means of UE-associated signalling. The QoE measurement configuration for signalling-based activation includes application layer measurement configuration list and the corresponding information for QoE measurement collection. Each application layer measurement configuration is encapsulated in a transparent container. The NG-RAN node forwards the corresponding QoE measurement configuration(s) to the UE in a downlink RRC message.

For management-based activation, the OAM sends the QoE measurement configurations to the NG-RAN node. The QoE measurement configuration for management-based activation includes application layer measurement configuration list and the corresponding information for QoE measurement collection. Each application layer measurement configuration is encapsulated in a transparent container. The NG-RAN node selects the UE(s) that meet the required QoE measurement capability, Area Scope and Slice Scope.

Editor’s Note: The detailed content of QoE configuration needs to be revisited based on the output of each subtopic in RAN2 and RAN3.

The UE reports QoE measurement results to the NG-RAN node in an uplink RRC message. The NG-RAN node transmits the QoE report and the corresponding QoE Reference ID to the MCE.

  - X.4	 QoE Measurement Collection Deactivation

QoE Measurement Collection deactivation permanently stops all or some of QoE measurement collection jobs towards a UE, resulting in the release of the corresponding QoE measurement configuration(s) in the UE. The deactivation of QoE measurement collection is supported by using UE-associated siganlling. A list of QoE Reference is used to deactivate the corresponding QoE measurement collection job(s).

  - X.5	 QoE Measurement Continuity for Mobility

The QoE Measurement Collection continuity for intra-system intra-RAT mobility is supported, with the area scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the area. For RRC_CONNECTED state mobility, the source NG-RAN node may transmit the QoE Measurement configuration(s) of a specific UE to the target NG-RAN node via XnAP or NGAP. For RRC_INACTIVE state mobility, signaling-based (FFS) QoE measurement configuration(s) of a specific UE can be restored from the node hosting the UE context when it resumes to RRC_CONNECTED state. Multiple sets of QoE measurement configurations should be supported during mobility.

Editor’s Note: Further descriptions pending the work progress in RAN2.

  - X.6	 RAN Visible QoE Measurement

RAN visible QoE measurement is configured by the NG-RAN node, where a subset of QoE metrics are reported from the UE. A set of QoE values could also be used for RAN visible QoE measurement. Both QoE metrics and QoE values could be utilized by the NG-RAN node for optimization. RAN visible QoE measurements are supported for the DASH streaming and VR services.

Editor’s Note: This section is pending work in other WGs, e.g., FFS on whether QoE value(s) can be reported from the UE or derived by the NG-RAN node, and the specific definition of RAN visible QoE values (if agreed, pending SA4 decision).

  - X.7	 Per-slice QoE Measurement

When a service is provided within a configured slice, the QoE Measurement for this service type could also be configured together with the corresponding slice scope, so that the user experience of this service could also be evaluated on a per-slice basis. The UE includes the slice ID inside the QoE report container when reporting QoE measurement result (pending SA4 decision).

  - X.8	  Alignment of MDT and QoE Measurements

Radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The immediate MDT can be configured before the QoE measurement is configured or the two could be configured simultaneously. The TCE/MCE is responsible for the correlation of the immediate MDT results and the QoE measurement results collected at the same UE. The NG-RANincludes time stamp information and related ID information for the MCE/TCE to enable the correlation of corresponding measurement results of MDT and QoE.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>