3GPP TSG-RAN WG3 Meeting #114-e	R3-216220

E-meeting, 1 – 11 Nov 2021

Agenda item:	10.2.1.4

Source:	Nokia, Nokia Shanghai Bell, ZTE, Lenovo, Motorola Mobility, China Unicom

Title:	[NR_ENDC_SON_MDT_enh, TP SON BL CR to TS 38.413] Addition of SCG UHI information

Document for:	Endorsement

# 1	Introduction

At the RAN3 #114 meeting, RAN3 agreed to add PSCell information to the existing UE History Information. NG-RAN cells are defined in NGAP, hence a TP to this specification is needed.

# 2	Text proposal for TS 38.413

The text proposal below is based in the most recently endorsed NGAP BL CR [R3-214607]. Please note: ASN.1 to be added once the TP is endorsed.

First change, ommited text not changed

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |  |  |
| Cell Type | M |  | 9.3.1.98 |  |  |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |  |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |  |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. |  |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.2.3.X | The PSCell related information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is FFS. |

Next change, ommited text not changed

#### 9.2.3.X	Last Visited Secondary PSCell Information (FFS)

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited PSCell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited NG-RAN PSCell Information | M |  | FFS | FFS |
| >E-UTRAN Cell |  |  |  |  |
| >>Last Visited E-UTRAN PSCell Information | M |  | FFS | FFS |

Remaining text not changed

[ASN.1 to be added, if endorsed]

Remaining text not changed