3GPP TSG-RAN WG3 Meeting #114-e	R3-216275

Electronic Meeting, November 1st – 11th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0192 | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RA-SDT BLCR to TS 38.401 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2021-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To capture agreements for Rel-17 SDT WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3-114 (Electronic) - R3-216071 (Huawei) : Introduced the basic structures of RACH based SDT procedure with FFSes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 SDT is missing in TS 38.401. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.X	Overall procedure for Small Data Transmission during RRC Inactive

#### 8.X.1	RACH based SDT

The procedure for RACH based small data transmission in RRC Inactive is shown in Figure 8.X.1-1.

Figure 8.X.1-1: RACH based Small Data Transmission in RRC Inactive state.

1.	The UE in RRC Inactive sends RRCResumeRequest message together with UL small data.

2.	The gNB-DU buffers the UL small data.

3.	The step 3 is as defined in step 4 in clause 8.6.2, with including an indication of SDT access.

4-5.	The steps 4-5 are as defined in steps 6-7 in clause 8.9.6.2.

Editor's Note: FFS whether UL or DL F1-U TEIDs in steps 4-5 is limited to the SDT DRBs.

6-7.	The steps 6-7 are as defined in steps 9-10 in clause 8.9.6.2.

Editor’s Note: Other details of this whole procedure is FFS.

Editor’s Note: FFS on how to support UL SDT SRB over F1 interface.